3GPP TSG-RAN WG3 Meeting #125	R3-244706

Maastricht, Netherlands, 19 – 23 August 2024

Agenda Item:	21.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion for CB: # XR2_NRDC

Document for:	Discussion

# Introduction

CB: # XR2_NRDC

- Discuss open issues above and other issues is any

- Provide CRs to capture agreements

(moderator - Nok)

Summary of offline disc R3-244706

# For the Chair’s Notes

Agree the following TPs:

# Discussion

## PDU Set based handling

Notes from online session:

Turn WA to agreement?

WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

Whether to support per-node PDU Set based Handling Indicator over NG?

Q1: Please share your view on 1) whether turn above WA to agreement. 2) whether support per-node PDU Set based Handling Indicator over NG.

| Company | Comment |
| --- | --- |
| Nokia | We agree to turn the WA to agreement, and the related XnAP TP. For per-node indicator over NG, no strong view. We can accept the majority view. |

Summary

Potential proposals:

## DL PSI based Discard coordination

Notes from online session:

For DL, MN/SN notifies SN/MN whether the DL PSI based discard is configured or not via XnAP signaling.

RAN3 to introduce new notification over F1AP and F1-U for DL PSI Discard.

Moderator suggest to use following TP as starting point.

- XnAP TP based on R3-244531 (CMCC will upload the draft TP)

- F1AP TP based on R3-244519 (Huawei will upload the draft TP)

- F1-U TP based on R3-244520 (Huawei will upload the draft TP)

Q2: Please share your view on the draft TPs in the folder.

Summary

Potential proposals:

## End of Data Burst Indication to the peer gNB

Contribution (R3-244109)  proposed source gNB should forward the End of Data Burst Indication to target gNB during the data forwarding for handover. (Nokia will upload the draft TS38.300 TP based on R3-244109)

Contribution (R3-244531) proposed to clarify that the End of Data Burst Indication is provided to the corresponding node when the Data Burst is going to transmit through the corresponding node

- Moderator consider this is a small update, and suggest it can be merged in the same TS 37.340 TP for Clause 3.4.

Q3: Please share your view on the draft TS 38.300 CR in the folder

Summary

Potential proposals:

## Burst Arrival Time reporting

It is unclear on whether the SN can receive the BAT from the MN.

(ZTE will upload the draft TS37.340 TP based on R3-244471)

Q3: Please share your view on whether SN can receive the BAT from the MN. If yes, please share your view on the draft TS 37.340 TP (based on R3-244471) in the folder.

- Please note the TS 37.340 TP also includes the proposal from R3-244531 related to End of Data Burst indication.

| Company | Comment |
| --- | --- |
| Nokia | Yes. TP for 37.340 is needed. |

Summary

Potential proposals:

## L4S

Contribution (R3-244301)  proposed following:

-	The node hosting PDCP takes into account the UL Congestion Information and/or DL Congestion information reported by the corresponding node to derive a single value of the UL Congestion Information and/or DL Congestion information and to to further send it to the UPF

(Ericsson will upload the draft TS38.425 CR based on R3-244301)

Q4: Please share your view on the draft TS 38.425 CR (based on R3-244301) in the folder.

Summary

Potential proposals:

## ECN marking for SN-initiated modification procedure

Contribution (R3-244268) describes:

The SN may trigger SN-initiated procedures to modify the configuration for XR services, and the ECN marking status may thus change (e.g. due to change of SN-DU). Therefore IEs added into the “S-NODE MODIFICATION REQUEST ACKNOWLEDGE” message for ECN marking should be copied into the “S-NODE MODIFICATION REQUIRED” message.

(CATT will upload the draft TS38.423 TP based on R3-244268)

Q5: Please share your view on following proposal:

IEs added into the “S-NODE MODIFICATION REQUEST ACKNOWLEDGE” message for ECN marking should be copied into the “S-NODE MODIFICATION REQUIRED” message.

In case you agree with the proposal, please share your view on the draft XnAP TP based on R3-244268 in the folder.

| Company | Comment |
| --- | --- |
| Nokia | Proposal is ok. |

Summary

Potential proposals:

## UE assistance information

Contribution (R3-244546) describes:

The ul-TrafficInfo IE is included in the UEAssistanceInformation message. The ul-TrafficInfo includes more information for XR QoS flow, such as jitter range, burst arrival time, traffic periodicity and so on. These information can help SN to configure a proper DRX because the gNB decides the parameters of DRX configuration considering both UL and DL traffic information. But currently, UEAssistanceInformation is not included in S-NODE ADDITION REQUEST message and/or S-NODE MODIFICATION REQUEST message.

Contribution (R3-244546) proposes: adding the UE assistance information in S-NODE ADDITION REQUEST and/or S-NODE MODIFICATION REQUEST SN to help SN for DRX configuration.

Q5: Please share your view on following:

adding the UE assistance information in S-NODE ADDITION REQUEST and/or S-NODE MODIFICATION REQUEST SN to help SN for DRX configuration.

| Company | Comment |
| --- | --- |
| Nokia | Not sure why it is needed. MN can initiate the Xn RRC Transfer procedure to transfer the UE assistance information. |

Summary

Potential proposals:

## Any other issues

Please add any other issues that you want to discuss.

| Company | Comment |
| --- | --- |

Summary

Potential proposals:

# 2nd Round (TBD)

Please add your comments in the draft TP.

| Company | Comment |
| --- | --- |

# References

- R3-243358, (TP to BL CR for TS 38.423) Discussion on the support of XR in NR-NR DC (Nokia, Nokia Shanghai Bell)

- R3-243407, (TP to BLCR for 37.340) Burst Arrival Time handling in NR-DC (Lenovo, Samsung, ZTE, Qualcomm Incorporated, Xiaomi, Huawei, CATT)

- R3-243345, (TP for NR_XR_Ph3 BL CR) Enhancement for NR-DC support of XR (Huawei)

- R3-243173, R19 XR DC Signaling Enhancements  (Qualcomm Incorporated)

- R3-243239, (TP to TS 38.423 on Support of XR in DC) ECN Marking/Congestion Information Reporting on SN Terminated bearer (NEC)

- R3-243240, (TP to TS 38.423 on Support of XR in DC) PSI-Based SDU Discarding (NEC)

- R3-243408, Remaining issues on PSI discard, ECN marking and End of Data Burst (Lenovo)

- R3-243483, (TP to BLCR for 38.423) Remaining issues for support of XR in DC (Ericsson)

- R3-243485, Support for L4S in DC (Ericsson, Deutsche Telekom, Charter, BT)

- R3-243486, Support for L4S in NR-NR DC (Ericsson, Deutsche Telekom, Charter, BT)

- R3-243487, (TP to TS38.423) Support for L4S in DC (Ericsson, Deutsche Telekom, Charter, BT)

- R3-243598, Discussion on support XR in DC (ZTE)

- R3-243599, (TP to TS 37.340 and 38.423) Support XR in DC (ZTE)

- R3-243640, Discussion on XR in DC (CATT)

- R3-243652, Discussion on support XR in DC (Samsung)

- R3-243653, (TP to TS 38.423) Support XR in DC (Samsung)

- R3-243728, (TP to TS 38.423 and 37.340) Support of XR in DC (CMCC)

- R3-243729, Discussion on support XR in DC (CMCC)