3GPP TSG-RAN WG3 Meeting #127bis	R3-251990

Wuhan, China, 7th – 11th April, 2025

Title: 	(TP for AI/ML BLCR to TS 38.401) Capturing agreements in Stage 2 RAN specifications

Source: 	Huawei, Deutsche Telekom, FiberCop, CATT

Agenda item:	11.2

Document Type:	Discussion and decision

# 1. Introduction

RAN3 is discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

This contribution provides a TP for the AI/ML BLCR to TS 38.401 capturing RAN3 agreements up to RAN3#126 (November 2024) which are relevant for Stage 2 description of Rel-19 AI/ML for NG-RAN.

# 2. Discussion

## 2.1 RAN3 agreements relevant for the AI/ML BLCR to TS 38.401

***** RAN3#125bis ***** [4] AI/ML-based Coverage and Capacity Optimization WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Introducing separate time information for future coverage state and predicted CCO issue. Rel-18 leftovers: Split architecture support for Rel-18 use cases Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. ***** RAN3#126 ***** [5] AI/ML-based Coverage and Capacity Optimization Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1. Introducing new IE for Future Coverage Modification Notification over F1. Introducing new IE for predicted CCO assistance information over F1.

It is proposed to agree the TP to AI/ML BLCR to TS 38.401 in Annex 1.

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.401 in Annex 1.

# 3. Conclusion

We make the following proposal:

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.401 in Annex 1.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

# 5. Annex 1 – TP for AI/ML BLCR to TS 38.401

3GPP TSG-RAN WG3 Meeting #127bis	R3-25xxxx

Wuhan, China, 7th – 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR |  | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for the AI/ML BLCR to TS 38.401) Support of enhancements on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom, FiberCop, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Stage 2 specification is enhanced to cover RAN3 agreements relevant for the Stage 2 description of enhancements on AI/ML for NG-RAN related to AI/ML-based CCO and split architecture support for Rel-18 use cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section “Data Collection and Reporting” is introduced to reflect the following RAN3 agreements: ***** RAN3#125bis ***** AI/ML-based Coverage and Capacity Optimization WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Introducing separate time information for future coverage state and predicted CCO issue. Rel-18 leftovers: Split architecture support for Rel-18 use cases Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. ***** RAN3#126 ***** AI/ML-based Coverage and Capacity Optimization Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1. Introducing new IE for Future Coverage Modification Notification over F1. Introducing new IE for predicted CCO assistance information over F1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements on AI/ML for NG-RAN related to AI/ML-based CCO and split architecture support for Rel-18 use cases are not captured at Stage 2 level. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 7.11	Support of AI/ML for NG-RAN

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

### 7.11.x	Data Collection and Reporting

The following information can be configured to be reported by the gNB-DU:

-	Energy Cost (EC).

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

For, e.g., proactive resolution of CCO issues, the predicted CCO issue with the associated timing information are transferred to the gNB-DU via the gNB-CU Configuration Update procedure, while the future coverage state with the associated timing information are transferred to the gNB-CU via the gNB-DU Configuration Update procedure.

<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>