3GPP TSG RAN WG3 Meeting #127-bis	R3-252023

Wuhan, China, 07-11 April, 2025

Agenda Item:	18.2

Source:	Ericsson

Title:	Support of Low power radio WUS

Document for:	Discussions

# 1	Introduction

This paper discusses further progress to consider in RAN3 to support low power radio WUS considering below open points:

- the terminology of “LP-WUS” - the LP-WUS node capability. - Including Extended UE Identity Index、 - The maximum value of CN subgroup ID、 - Emergency call back.

# 2	Discussion on RAN3 impacts

## 2.1	LP-WUS terminology

RAN2 answered SA2 in [1] that LP-WUS terminology is clear in RAN without any misunderstanding. Thus, the “LP-WUS” terminology can be used. Therefore, there is no need to update RAN3 specifications with new terminology.

Proposal 1: There is no need to update RAN3 specifications with new terminology different than LP-WUS.

## 2.2	LP-WUS node capability.

On the issue of UE radio capability for paging information, RAN2 had the following agreement:

- Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for Release 15 gNB and Release 16 gNB and for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the coming RAN2 meetings based on company contributions.

For LP-WUS, an inter-operability issue with paging might happen if initial registration was done in a legacy gNB which does not support or understand LP-WUS, and later paging happens at a gNB supporting LP-WUS that would not realize the UE supports LP-WUS because the legacy gNB did not transfer the LP-WUS paging capability to CN during initial registration. The UE would become unreachable in that cell. It is proposed to wait for RAN2 how will they handle the case of non-homogeneous release among gNBs. Possibly there could be no solution to this except to consider homogenous deployment on the same release for the paging feature to work correctly.

Proposal 2: RAN3 to wait for RAN2 how will they handle the case of non-homogeneous releases among gNBs. Possibly there could be no solution to this except to consider homogenous deployment on the same release for the paging feature to work correctly.

## 2.3	Including Extended UE Identity Index

RAN1 agreed that the maximum number of subgroups for LP-WUS is 31.

R1-2500794 Summary #4 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31

Additionally whether new IE extended value of 5G-S-TMSI with more bits, or extending the hashed 5G-S-TMSI can be used for UE-ID based LP-WUS subgrouping needs to wait for RAN2.

Proposal 3: remove the FFS on maximum number of subgroup IDs for LP-WUS.

## 2.4	Emergency callback

Similarly to PEI, when the UE has an emergency PDU session LP-WUS should not be used. Based on on-going discussion between RAN2 and SA2 on LS on Emergency Services and eDRX [2], according to most companies in SA2, if the UE has emergency PDU session, if the UE is kept in RRC_CONNECTED, then paging is not triggered and LP-WUS (together with paging optimization features that may cause delay to callback) are not used. Hence there is no issue.

Proposal 4: RAN3 to align with conclusion of LS on emergency call back and paging where the UE is kept in RRC_CONNECTED, and LP-WUS is not used, i.e., there is no issue.

# 3	Conclusion

Proposal 1: There is no need to update RAN3 specifications with new terminology different than LP-WUS.

Proposal 2: RAN3 to wait for RAN2 how will they handle the case of non-homogeneous releases among gNBs. Possibly there could be no solution to this except to consider homogenous deployment on the same release for the paging feature to work correctly.

Proposal 3: remove the FFS on maximum number of subgroup IDs for LP-WUS.

Proposal 4: RAN3 to align with conclusion of LS on emergency call back and paging where the UE is kept in RRC_CONNECTED, and LP-WUS is not used, i.e., there is no issue.

# 4	References

- R3-251515, Reply LS on LP-WUS subgrouping, RAN2

- R3-251521, Reply LS on emergency call back and paging