3GPP TSG RAN WG3 Meeting #127bis	R3-25xxxxx

Wuhan, China, 7th – 11th April 2025

Agenda Item:	10.2

Source:	Samsung (moderator)

Title:	SoD of MRO for others

Document for:	Approval

# Introduction

This is a summary of offline discussions on MRO for CHO with candidate SCG(s) and S-CPAC.

# For the Chairman’s Notes

MRO for CHO with Candidate SCG(s)

In case of too late CHO execution, reuse Handover Report message from the last serving MN to the relevant candidate MN.

FFS on the additional information included in handover report.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

TPs:

TP for TS38.423 R3-25xxxx (Samsung)

TP for TS37.340 R3-25xxxx rev of R3-250084 (Nokia)

TP for TS38.300 R3-25xxxx rev of R3-250724 (ZTE)

Open issues for failure cases (the blue part could be discussed online):

1. Additional information reported from the UE in RLF Report

a)	The fulfilled execution conditions before the RLF is encountered

b)	Time elapsed between the fulfilled execution condition and RLF

c)	the SCG failure information

d) The number of times each candidate cell met the execution conditions

e) Identifiers of non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred

f) The number of times each non-associated cell pair met the execution conditions

2. Additional information reported from the UE in SCGFailureInformation

- The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition)

- Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered

3. LS to RAN2 if any agreement for 1 and 2

4. Support MRO for MCG+SCG failure cases ?

Open issues for near failure cases:

- Whether the near failure monitoring for CHO with Candidate SCGs is in Rel. 19 or intention is rather to move it Rel.20?

- Issues to support near failure case:

- Correlation of SHR and SPR is needed?

  - Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR

- Addition information to be reported in SHR and SPR for supporting the near failure case in case of CHO with candidate SCG(s)?

  - Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type

  - The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR

  - The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR

- SPR forwarding?

- Scenarios e.g. CHO with candidate SCG(s) and CHO only are configured, CHO only is executed successful, this case can be considered as an near failure case?

MRO for S-CPAC

The forwarding mechanism in BLCR for TS37.340 need to be refined.

TP for TS37.340: rev of R3-250610 (Samsung)

Open issues for failure cases (the blue part could be discussed online):

- UE reported information in SCGFailureInforamtion:

  - The setting of the information on previousPSCellId and timeSCGFailure needs to be updated to support the failures due to the following S-CPAC

  - The UE report the indication information on the S-CPAC being triggered by an MN or an SN to the network

  - An indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC

  - initiating PSCell ID

- Xn impact

Include subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) in the SCG Failure Information Report message

Open issues for Successful use cases:

- SCG UHI should be updated to the new serving SN/target SN during S-CPAC procedure?

- From MN to the candidate/target SN

  - Add UHI to SN MODIFICATION REQUEST

- Inform the source SN about the outcome of mobility events for SN initiated S-CPAC by sending the UHI to the initiating SN using the SN MODIFICATION REQUEST

- Enabling of the optimal usage of S-CPAC as part of SON based optimization?

- Request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC?

# Discussion

## MRO for CHO with Candidate SCG(s)

### Agreements

At RAN3#123bis meeting, the following initial agreements were achieved:

MRO for CHO with candidate SCG failure and near failure cases

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

R19 SON/MDT solution discussion is based on R18 work.

Further discussion on the use cases and solutions…

Agreements at RAN3#124 meeting:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

Agreements at RAN3#125 meeting:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes:

- The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

- FFS whether to optimize the association of the PCell and PSCell.

Agreement at RAN2#127 meeting:

- UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

Agreements at RAN3#125bis meeting:

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

The fulfilled events before the RLF is encountered?

Agreements at RAN2#127bis meeting:

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

Agreements at RAN3#126 meeting:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

- The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

- The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation

Agreements at RAN2#128 meeting:

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

Agreements at RAN3#127 meeting:

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

Agreements at RAN2#129 meeting:

Agreements - Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any. - Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC. - Enhance RLF report for CHO with candidate SCGs to include at least the following information: - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered; - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered; - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered. - Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any. - We should avoid specifying the procedure in a way that the UE sends redundant information

### Stage 2 and stage3 mpact

#### Failure case

Additional information included in handover report:

- Clarify that Target cell CGI is suitable cell id in case of too late CHO with candidate SCG(s): 1748 (Samsung), 1775 (CATT)

whether Handover Report message is needed for case 7a

- Yes: 1593 (Nokia),

- No: 1775 (CATT), 2191(ZTE), 1748(Samsung)

When to send the SCG Failure Indication message? Immediately or not? Whether need to aggregate other SCGFailureInformation?

- Immediately, no need to aggregate: 1748 (Samsung), 1588 (QC), 1925 (HW), 1933 (LV)

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Check whether respective MN is only a candidate?

- Only a candidate

  - 1748

- Can also be the source

  - 1593

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- UE ID: 1748(Samsung), 1925 (HW), 1588 (QC)

- CPC failure type: 1748(Samsung), 1775 (CATT)

- Suitable PSCell ID: 1775 (CATT),

- Source PSCell ID: 2246 (CU)

- Failed PSCell ID: 2246 (CU)

Additional information reported from the UE in RLF Report

a)	The fulfilled execution conditions before the RLF is encountered

b)	Time elapsed between the fulfilled execution condition and RLF

c)	the SCG failure information

d)     Identifiers of candidate PCell(s) or PSCell(s) that met execution conditions before RLF, while their paired cells did not;

e)      The number of times each candidate cell met the execution conditions.

f)       Identifiers of non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred;

h)      The number of times each non-associated cell pair met the execution conditions.

Additional information reported from the UE in SCGFailureInformation

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

MCG failure + SCG failure

- Different views are given in the contributions. Can observe the progress of other agreed cases firstly.

TPs

TP for TS38.423

TP for TS38.300

TP for TS37.340

LS to RAN2 including RAN3 agreement on the information reported from the UE?

#### Near failure case

Whether the near failure monitoring for CHO with Candidate SCGs is in Rel. 19 or intention is rather to move it Rel.20?

Issues to support near failure case:

- Correlation of SHR and SPR is needed?

  - Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR [1925]

- Addition information to be reported in SHR and SPR for supporting the near failure case in case of CHO with candidate SCG(s)?

  - The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR [1933]

  - The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR [1933]

- SPR forwarding?

- Scenarios e.g. CHO with candidate SCG(s) and CHO only are configured, CHO only is executed successful, this case can be considered as an near failure case?

## MRO for S-CPAC

### Agreements

At RAN3#123bis meeting, the following initial agreements were achieved:

Work on the scenarios of failure in S-CPAC. The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

R19 SON/MDT solution discussion is based on R18 work.

Further discussion on the use cases and solutions…

Agreements at RAN3#124 meeting:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

Agreements at RAN3#125 meeting:

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution.

Editor’s note: FFS whether the differentiation between initial and following CPC is needed.

### Failure case

Stage 2

The refined TP for the forwarding mechanism has been agreed in RAN3#127 meeting. The Editor’s note is still there for people to double check whether the description is right.

The text description in stage 2 is right?

Any addition or change is needed?

1595: different taste of the description

1775: additional text

Remove the Editor’s note on MRO for S-CPAC in TS38.423?

1748: remove the editor’s note

Stage 3:

UE reported information in SCGFailureInforamtion:

  - The setting of the information on previousPSCellId and timeSCGFailure needs to be updated to support the failures due to the following S-CPAC [1748]

  - The UE report the indication information on the S-CPAC being triggered by an MN or an SN to the network [1748].

  - An indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC [1933]

  - initiating PSCell ID [1775]

Xn impact:

- Include subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) in the SCG Failure Information Report message [1933] [2191]

### Successful use cases

SCG UHI should be updated to the new serving SN/target SN during S-CPAC procedure?

- From MN to the new serving MN/target SN

  - Include SCG UHI in SN Reconfiguration Complete message from MN to the target SN

  - Add UHI to SN MODIFICATION REQUEST

- inform the initiating SN about the outcome of mobility events for SN initiated S-CPAC  using the SN RELEASE REQUEST or SN RELEASE CONFIRM (1925)

TPs for TS38.423 on SCG UHI in 1748,  .

TP for 37.340 and 38.423 on SCG UHI: 1925

Enabling of the optimal usage of S-CPAC as part of SON based optimization?

- request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC?