3GPP TSG RAN WG3 Meeting #127bis	R3-252302

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	10.4

Source:	Nokia (moderator)

Title:	SoD of SONMDT R18 leftovers

Document for:	Approval

# Introduction

This is a summary of offline discussions on MRO for SONMDT R18 leftovers.

# For the Chairman’s Notes

MRO for MR-DC SCG failure

UHI Enhancement for SCG Deactivation/Activation

RACH optimization for SDT

# Background

Previous agreements in RAN3

MRO for MR-DC SCG failure:

Keep the following R18 agreement in R19:

Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios.

LS to RAN2 on the feasibility to support MRO for SCG failure in R19 in R3-242195

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

MHI Enhancement for SCG Deactivation/Activation

RAN3 agree to support a network-based solution for enhancing PSCell activation/deactivation.

RACH optimization for SDT:

Work on scenario of RACH optimization for SDT first.

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

The information that stored and reported in the RA report for SDT:

c. the time from the start of RA-SDT to the reporting of RA report.

We will send an LS to RAN2 to inform about the previous agreement to report the time from the start of RA-SDT to the reporting of RA report.

# Discussion

## MRO for MR-DC SCG failure:

Based on RAN2 agreement as below, the additional info has been included in SCGFailureInformationNR message for EN-DC.

Agreements Add reporting of the following parameters for SCG failure report in EN-DC scenario: ⁻ For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell; ⁻ For timeSCGFailure: value range 0-1023; ⁻ For failureType: Reuse the legacy field. - perRA-InfoList

[Lenovo] proposes to enhance SCG FAILURE INFORMATION REPORT message in X2/Xn interface for EN-DC SCG failure to include failureType and perRA-InfoList.

- Q1: Whether add failureType and perRA-InfoList in SCG FAILURE INFORMATION REPORT message in X2/Xn interface?

Conclusion:

[Huawei] thinks LTE MN only understands SSB measurement results and cannot decode finer granularity measurement results, while NR SN is able to decode measResultSCG and use it to analyze suitable PSCell.

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

measResultSCG

Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]

[Lenovo] prefers not transfer the suitable PSCell ID from SN to MN in X2 or Xn interface since MN can select the suitable PSCell based on SSB measurement results instead of CSI-RS measurement results

- Q2: For MN initialed PSCell change, LTE MN or NR SN to select the next suitable PSCell? If it is NR SN, whether provide the suitable PSCell from SN to MN?

Conclusion:

## UHI Enhancement for SCG Deactivation/Activation

For network based solution, [Samsung, Jio Platforms] and [Huawei] proposes to use relative time to present SCG activation time.

- Q3: Whether to use absolute time or relative time to express SCG activation time?

Conclusion:

Regarding to the standard impact [Samsung, Jio Platforms] think both MN and SN knows whether SCG is deactivated or not. Based on the time duration information, the MN could understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, and only stage-2 is needed considering the info is not shared between the nodes.

[Huawei] wants to add SCG activation time in the Last Visited PSCell Information.

- Q4: Whether to add SCG activation time in in Last Visited PSCell Information, or only stage-2 is needed?

Conclusion:

[Samsung, Jio Platforms] [Huawei] think there is no need to introduce additional information.

- Q5: Whether to include UE traffic characteristics or quantity of deactivations of the SCG during the stay in the PSCell in UHI?

Conclusion:

## RACH optimization for SDT

Note that RAN2 already have the following agreement:

For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log.

[Samsung, Jio Platforms] [Lenovo] in contrast to RAN2 agreements to provide reporting related to the time of SDT evaluation propose to report RSRP at the point of time when RA-SDT fails and remaining UL data volume to be transmitted upon detecting failed RA-SDT or successful SDT in the RA report.

- Q6: Whether to report RSRP when RA-SDT fails and remaining UL data volume to be transmitted upon detecting failed RA-SDT or successful SDT in the RA report?

Conclusion:

# Reference

| 1 | R3-251750 | (TP for SON BLCR for TS37.340) Rel-18 SON and MDT leftovers (Samsung, JIO Platforms) | other |
| --- | --- | --- | --- |
| 2 | R3-251929 | (TP for SON BLCR for 38.413, 36.423) R18 leftovers (Huawei) | other |
| 3 | R3-251935 | Discussion on MRO for R18 leftovers (Lenovo) | discussion |