3GPP TSG-RAN3 Meeting #127bis	R3-251585

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Architecture, Protocols and Signaling to support Topology 1 of A-IoT

Document for:	Discussion/Decision

# Background

In this paper, we continue to provide our views on aspects related to Architecture, Protocols and signaling to support Topology 1 of AIoT Topology.

# Discussion

## Architectural assumptions

The following was agreed in RAN3#127:

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

AIOTF supports specific functionalities such as termination of AIoT NAS protocol with AIoT device, triggering the gNB to perform AIoT service operations (such as inventory/command) towards the AIoT Device(s), Reader selection, authorizing AF’s AIoT service operation request, retrieving AIoT device profile data from ADM etc.

Further, AIOTF doesn’t perform several operations supported by an AMF. Hence the gNB should consider the AIOTF as a separate Core NF and not as an A-IoT enabled AMF.

AIOTF supports specific functionalities (such as termination of AIoT NAS protocol with AIoT device, triggering the gNB to perform AIoT service operations such as inventory/command towards the AIoT Device(s), Reader selection, authorizing AF’s AIoT service operation request, retrieving AIoT device profile data from ADM etc.) and doesn’t perform several operations supported by an AMF

The gNB should consider AIOTF as a separate Core NF and not as an “A-IoT enabled AMF”

| Scenario | Proposal |
| --- | --- |
|  | A gNB can connect to one or more AIOTFs directly. The figure to the left shows an example where 2 AIOTFs connected directly. (e.g., for receiving Inventory Requests from 2 different AIOTFs and/or for forwarding A-IoT NAS PDUs for two different A-IoT devices from these 2 different AIOTFs) |
|  | A gNB can connect to one or more AIOTFs indirectly (via the AMF). The figure to the left shows an example where 2 AIOTFs are connected indirectly. |
|  | A gNB can connect to an AIOTF indirectly via one or more AMFs |
|  | A gNB supporting both Uu and A-IoT radio can simultaneously connect to an AMF (e.g., for communication with a UE) and with an AIOTF (e.g., for communication with an AIoT device) |
|  | A gNB connects to two different AIOTFs – one directly and one indirectly via an AMF |
|  | A gNB can connect to the same AIOTF via direct path and indirect path (via AMF) |

RAN3 agrees to the following architectural assumptions:

  - A gNB can connect to more than one AIOTFs directly

  - A gNB can connect to more than one AIOTFs indirectly (via different AMFs)

  - A gNB can connect to an AIOTF indirectly via more than one AMF

  - A gNB supporting both Uu and A-IoT radio can simultaneously connect to an AMF (e.g., for communication with a UE) and with an AIOTF (e.g., for communication with an AIoT device)

  - A gNB can connect to more than one AIOTFs – one directly and one indirectly via an AMF

  - A gNB can connect to the same AIOTF via direct path and indirect path (via AMF)

## Interface management procedures

AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

FFS how to handle A-IoT only support case.

FFS how to handle the not applicable mandatory IEs

In case of indirect communication, FFS whether existing NG interface management procedures are used between the gNB and the AMF

In case of direct communication, FFS whether existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF

In RAN3#127, RAN3 agreed to not define a new terminology called “A-IoT RAN node” (and reuse the terminology of gNB) and to support the scenario where a gNB only supports communication with A-IoT devices over A-IoT radio.

Besides the legacy deployment scenario (i.e., gNB supporting only Uu), RAN3 needs to discuss how to support different deployment scenarios as shown in the figure below:

- gNB supporting only A-IoT radio with direct connectivity to AIOTF (case 1)

- gNB supporting only A-IoT radio with indirect connectivity to AIOTF (case 2)

- gNB supporting both Uu and AIoT with direct connectivity to AIOTF (case 3)

- gNB supporting both Uu and AIoT with indirect connectivity to AIOTF (case 4)

Therefore, the following is proposed:

RAN3 to support signaling to support different deployment scenarios of gNB:

  - gNB supporting only A-IoT radio (with direct and indirect connectivity to AIOTF)

  - gNB supporting both Uu and A-IoT radio (with direct and indirect connectivity to AIOTF)

For the above deployment scenarios, RAN3 further needs to discuss how to handle the A-IoT non applicable mandatory IEs.

Currently, NG Setup Request contains some mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) which are not applicable to A-IoT e.g., there is no concept of TA List (if TA is not reused as A-IoT area) or slices or Paging DRX for A-IoT.

NG Setup Response contains some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity) which are also not all relevant to A-IoT e.g., there is no GUAMI at A-IoT device or there is no requirement to support AMF load balancing for which Relative AMF Capacity was defined.

NG Setup Request contains mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) and NG Setup Response also includes some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

To handle this, we see two options for this:

| Deployment | Option 1 | Option 2 |
| --- | --- | --- |
| Direct | Use A-IoT NG Setup Request/Response for interface management between gNB and AIOTF | Use NG Setup Request/Response for interface management between gNB and AIOTF with an A-IoT support flag |
| Indirect | Use NG Setup Request/Response for interface management between gNB and AMF with an A-IoT support flag | Use NG Setup Request/Response for interface management between gNB and AMF with an A-IoT support flag |
| Evaluation | Pros: Cleaner design i.e. there is no need for gNB supporting direct connectivity to AIOTF to include random/junk values in NG Setup Request and AIOTF to ignore the A-IoT non applicable mandatory IEs (and vice-versa) Cons: Two separate set of NG interface management procedures depending on whether gNB supports direct or indirect connectivity to AIOTF | Cons: gNB supporting A-IoT (irrespective of direct or indirect connectivity to AIOTF) would need to include random/junk values in NG Setup Request and AMF/AIOTF would need to ignore the A-IoT non applicable mandatory IEs (and vice-versa) Pros: Existing NG interface management procedures is reused irrespective of whether gNB supports direct or indirect connectivity to AIOTF (i.e., no need of new AIoT NG interface management procedure) |

RAN3 to discuss the following 2 options for interface management with AIOTF

  - Option 1: Use different interface management procedures depending on direct vs. indirect connectivity to AIOTF (A-IoT NG Setup Request/Response for direct vs. NG Setup Request/Response for indirect)

  - Option 2: Use the same NG Setup Request/Response for both direct and indirect

Reusing NG Setup Request/Response in case of direct communication with AIOTF would mean that gNB/AIOTF need to include random/junk values in the A-IoT non-applicable mandatory IEs and AIOTF/gNB would need to ignore these values upon reception

For a cleaner design, we prefer Option 1 in Proposal 4 and therefore the following is proposed:

For the case of direct communication with AIOTF, define a new “AIoT NG Setup Request/Response” message for gNB to setup N2 association with AIOTF and to avoid using the AIoT non-applicable mandatory IEs.

AIoT NG Setup Request can include several IEs such as gNB ID, gNB’s serving area, Supported BS Reader(s), BS Reader’s serving area, depending on the discussion in section 2.5. In response, AIOTF signals an AIoT NG Setup Response and may contain relevant information e.g., AIOTF ID, PLMN ID (details pending further discussion).

For the case of indirect communication with AIOTF (via AMF), reuse NG Setup Request/Response message with an “AIoT support flag” for gNB to setup N2 association with AMF and to handle the AIoT non-applicable mandatory IEs

If this “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request. Further, gNB can include random/junk values in those A-IoT non-applicable mandatory IEs.

Also in response, the AMF includes an “A-IoT support” flag with value “A-IoT only” in NGAP Setup Response and includes random/junk values in those A-IoT non applicable mandatory IEs. gNB ignore the A-IoT non applicable mandatory values upon receiving this NGAP Setup Response.

If this “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT + Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT + Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

## Area information for A-IoT

SA2 had sent a LS to RAN3 asking whether A-IoT RAN node can further provide supported A-IOT RAN node serving area and BS reader serving area and requested RAN3 to define these parameters:

SA2 assumes that OAM can configure the AIOTF with needed information for this selection.

SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information to the core network. e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area

SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF.

If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters

RAN3 agreed the following in RAN3#127:

AIoTF obtain the AIoT RAN information (supported AIoT area, served reader ID list, location of reader) via OAM configuration

Further, RAN3 identified the following open issue in RAN3#127:

For gNB selection by AIOTF, FFS whether to define a new A-IoT Area or use Tracking Area dedicated for A-IoT

There is no concept of Tracking areas at the A-IoT device. Also, RAN2 agreed to not support any Tracking Area Update like procedure for A-IoT device(s).

There was a discussion in RAN3#127 whether to reuse the concept of TAs for each gNB to indicate its supported service areas for A-IoT and in turn AIOTF selecting appropriate gNBs based on the indicated TAs.

A gNB supporting only A-IoT doesn’t need to understand the concept of tracking areas or cells and in our view, hence it is easier to specify AIoT service areas as geographical areas rather than reusing TAI and/or cells to indicate the AIoT service area.

A gNB supporting only A-IoT doesn’t need to understand the concept of tracking areas or cells as there is no concept of TA and cells over the A-IoT radio interface.

A gNB supporting both A-IoT and Uu can understand the concept of tracking areas or cells and potentially we can reuse TAI and/or cells to indicate the AIoT service area in this case. But the Tracking Area Codes might already be used for Uu communication and hence it might be difficult to reuse the existing tracking areas or reserve a dedicated TA for A-IoT.

A gNB supporting both A-IoT and Uu might be already using tracking areas for Uu communication and hence it might be difficult to reuse the existing tracking areas or reserve a dedicated TA for A-IoT.

But consider the deployment of gNB only supporting AIoT, we propose the following:

Instead of reusing TAs to indicate A-IoT service areas, define a new “A-IoT service area” in terms of geographical areas (gNB’s serving area and each reader’s serving area)

## Reader selection

The following was agreed in RAN3#127:

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area

Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers

WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list

- Based on the agreements so far, the A-IoT service area information can be indicated as an area and/or listof reader(s). Hence, we propose to turn the WA and FFS into agreement as well to cover all possible scenarios.

Turn the following WA to agreement: Upon receiving only the reader list in Inventory Request, the gNB selects the readers as indicated by the reader list

Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list

## Signaling exchange between gNB and AIOTF

RAN3 agreed the following in RAN3#127:

AIoTF obtain the AIoT RAN information (supported AIoT area, served reader ID list, location of reader) via OAM configuration

Besides the OAM based solution, we think that the AIOTF should be able to discover the above information dynamically via signalling with gNB. Even though we are currently considering T1, and the readers would be predominantly static and thereby the reader serving area fixed, we think we should allow for deployments where gNB readers are mobile e.g., readers mounted on a moving vehicle or change of reader’s serving area due to switch on/off or power adaptation.

A-IoT service area and/or readers supported by a gNB may change “dynamically” in some scenarios e.g, if readers are mounted on a moving vehicle and/or due to Reader Tx/Rx power adaptation or Reader switch on/off

We therefore propose the following:

AIOTF obtains the following information via signaling with gNB

  - Readers supported by the gNB

  - A-IoT service area (gNB's serving area, Reader’s serving area)

  - Reader location

## “AIoT Reader Control” (A-RC) between gNB and AIOTF

In RAN3#127, RAN3 discussed two alternatives for the protocol stack and interfaces for Topology 1

Alt 1: Define a new protocol "AIoT Reader Control" (A-RC) between gNB and AIOTF. A-RC is carried over NGAP for both direct and indirect options

Alt 2: No end-to-end "A-IoT Reader Control" (A-RC) protocol is defined between gNB and AIOTF. A-IoT related information is carried over NGAP between gNB and AIOTF (or AMF) in case of direct (or indirect) options respectively

Also based on majority view, RAN3 took a working assumption on Alt 2 last meeting:

WA: Include AIoT information in NGAP

We evaluate the pros and cons of the 2 alternatives again below:

| Alternatives | Evaluation |
| --- | --- |
| Alt 1: Define a new protocol "AIoT Reader Control" (A-RC) between gNB and AIOTF to transport A-IoT related information A-RC is carried over NGAP in case of both direct and indirect option | Same behavior for AIOTF for direct and indirect options AIOTF uses A-RC protocol to transport A-IoT related information to gNB for both direct and indirect option Direct option with Alt 1 Indirect option with Alt 1 Need to define a new spec for A-RC protocol |
| Alt 2: No end-to-end "A-IoT Reader Control" (A-RC) protocol is defined between A-IoT RAN node and AIOTF. A-IoT related information is carried explicitly in NGAP | Different behavior for AIOTF depending on direct vs. indirect In case of direct, AIoTF needs to transport AIoT information in NGAP. In case of indirect, AIoTF needs to transport AIoT information in API to AMF; AMF then includes the AIoTF information in NGAP Direct option with Alt 2 Indirect option with Alt 2 No need to define a new spec. NGAP is reused. |

Including A-IoT information as a NGAP container (Alt 2) compared to including A-IoT information in an end-to-end A-RC protocol (Alt 1) has the following drawbacks:

  - Different behavior for AIOTF depending on direct vs. indirect

    - In case of direct, AIOTF would need to transport AIoT information in NGAP.

    - In case of indirect, AIOTF needs to transport AIoT information in API to AMF; AMF then includes the AIoT information in NGAP

  - Different “awareness” of AIOTF at gNB in case of direct vs. indirect

    - In case of direct, gNB is aware of AIOTF and establishes N2 association with AIOTF

    - In case of indirect, gNB is not aware of AIOTF and establishes N2 association with the AMF

We still think Alt 1 is the best approach and therefore the following is proposed:

Revert the WA “Include AIoT information in NGAP” and agree to support a new protocol "AIoT Reader Control" (A-RC) between gNB and AIOTF to carry A-IoT related information for both direct and indirect options.

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Architectural assumptions

Observation 1:	AIOTF supports specific functionalities (such as termination of AIoT NAS protocol with AIoT device, triggering the gNB to perform AIoT service operations such as inventory/command towards the AIoT Device(s), Reader selection, authorizing AF’s AIoT service operation request, retrieving AIoT device profile data from ADM etc.) and doesn’t perform several operations supported by an AMF

Proposal 1:	The gNB should consider AIOTF as a separate Core NF and not as an “A-IoT enabled AMF”

Proposal 2:	RAN3 agrees to the following architectural assumptions:

a. A gNB can connect to more than one AIOTFs directly

b. A gNB can connect to more than one AIOTFs indirectly (via different AMFs)

c. A gNB can connect to an AIOTF indirectly via more than one AMF

d. A gNB supporting both Uu and A-IoT radio can simultaneously connect to an AMF (e.g., for communication with a UE) and with an AIOTF (e.g., for communication with an AIoT device)

e. A gNB can connect to more than one AIOTFs – one directly and one indirectly via an AMF

f. A gNB can connect to the same AIOTF via direct path and indirect path (via AMF)

2.2 Interface management procedures

Proposal 3:	RAN3 to support signaling to support different deployment scenarios of gNB:

a. gNB supporting only A-IoT radio (with direct and indirect connectivity to AIOTF)

b. gNB supporting both Uu and A-IoT radio (with direct and indirect connectivity to AIOTF)

Observation 2:	NG Setup Request contains mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) and NG Setup Response also includes some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

Proposal 4:	RAN3 to discuss the following 2 options for interface management with AIOTF

a. Option 1: Use different interface management procedures depending on direct vs. indirect connectivity to AIOTF (A-IoT NG Setup Request/Response for direct vs. NG Setup Request/Response for indirect)

b. Option 2: Use the same NG Setup Request/Response for both direct and indirect

Observation 3:	Reusing NG Setup Request/Response in case of direct communication with AIOTF would mean that gNB/AIOTF need to include random/junk values in the A-IoT non-applicable mandatory IEs and AIOTF/gNB would need to ignore these values upon reception

Proposal 5:	For the case of direct communication with AIOTF, define a new “AIoT NG Setup Request/Response” message for gNB to setup N2 association with AIOTF and to avoid using the AIoT non-applicable mandatory IEs.

Proposal 6:	For the case of indirect communication with AIOTF (via AMF), reuse NG Setup Request/Response message with an “AIoT support flag” for gNB to setup N2 association with AMF and to handle the AIoT non-applicable mandatory IEs

Proposal 7:	If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

Proposal 8:	If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT + Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

2.3 Area information for A-IoT

Observation 4:	A gNB supporting only A-IoT doesn’t need to understand the concept of tracking areas or cells as there is no concept of TA and cells over the A-IoT radio interface.

Observation 5:	A gNB supporting both A-IoT and Uu might be already using tracking areas for Uu communication and hence it might be difficult to reuse the existing tracking areas or reserve a dedicated TA for A-IoT.

Proposal 9:	Instead of reusing TAs to indicate A-IoT service areas, define a new “A-IoT service area” in terms of geographical areas (gNB’s serving area and each reader’s serving area)

2.4	Reader selection

Proposal 10:	Turn the following WA to agreement: Upon receiving only the reader list in Inventory Request, the gNB selects the readers as indicated by the reader list

Proposal 11:	Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list

2.5	Signaling exchange between gNB and AIOTF

Observation 6:	A-IoT service area and/or readers supported by a gNB may change “dynamically” in some scenarios e.g, if readers are mounted on a moving vehicle and/or due to Reader Tx/Rx power adaptation or Reader switch on/off

Proposal 12:	AIOTF obtains the following information via signaling with gNB

a. Readers supported by the gNB

b. A-IoT service area (gNB's serving area, Reader’s serving area)

c. Reader location

2.6 “AIoT Reader Control” (A-RC) between gNB and AIOTF

Observation 7:	Including A-IoT information as a NGAP container (Alt 2) compared to including A-IoT information in an end-to-end A-RC protocol (Alt 1) has the following drawbacks:

-	Different behavior for AIOTF depending on direct vs. indirect

i.	In case of direct, AIOTF would need to transport AIoT information in NGAP.

ii.	In case of indirect, AIOTF needs to transport AIoT information in API to AMF; AMF then includes the AIoT information in NGAP

-	Different “awareness” of AIOTF at gNB in case of direct vs. indirect

i.	In case of direct, gNB is aware of AIOTF and establishes N2 association with AIOTF

ii.	In case of indirect, gNB is not aware of AIOTF and establishes N2 association with the AMF

Proposal 13:	Revert the WA “Include AIoT information in NGAP” and agree to support a new protocol "AIoT Reader Control" (A-RC) between gNB and AIOTF to carry A-IoT related information for both direct and indirect options.