3GPP TSG-RAN3 Meeting #127bis	R3-251587

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	16.3

Source: 	Qualcomm Incorporated

Title: 	Locating Ambient IoT devices

Document for:	Discussion/Decision

# Background

In this paper, we continue to provide our views for locating an AIoT device at “reader” granularity as concluded in the SI phase.

# Discussion

## How is Reader ID sent in Inventory Report?

The following was agreed in RAN3#127:

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report

AIoT RAN node sends the Inventory Report aggregated per Reader

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally

The following was an open issue in last RAN3 meeting:

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

As the AIOTF may have the knowledge of the supported readers by a gNB, either via OAM or via interface management procedures, there is no need to include the “Global gNB ID + Reader index” in the Inventory Report. And it is sufficient to just include the “Reader index” as Reader ID in the Inventory Report.

AIOTF should have knowledge about the readers supported by a gNB and their Global Reader ID(s) (i.e., Global gNB ID + Reader index) via OAM and/or via interface management procedure

There is no need for gNB to include the Global Reader ID in Inventory Report if it already knows the mapping of gNB and its supported readers

gNB just includes the “Reader index” as Reader ID to AIOTF in Inventory Report (i.e., there is no need to include “Global gNB ID + Reader index”)

## Reader ID in Command Report

Whether to also include Reader ID in Command Report was discussed in RAN3#127 because there were concerns raised on how this Reader ID can be used.

In our view, including the Reader ID in Command Report can one way to update the A-IoT device and reader association at the AIOTF e.g., the A-IoT devices might have moved to a different Reader’s coverage in the time between Inventory and Command. Also, this can be useful in case there is a device association maintained at the AIOTF for subsequent Command after the 1st Command or in case of multiple Commands targeting the same A-IoT device

A-IoT devices might move to a different Reader’s coverage in the time between Inventory and Command.

Including the Reader ID in Command Report can be useful to keep the latest  A-IoT device and reader association at the AIOTF and can be useful to know the “last serving reader” in case of back-to-back commands.

gNB may optionally send the “Reader index” to the AIOTF in Command Report

## Periodic A-IoT location reporting

The AIOTF might want to obtain A-IoT device location (i.e., the Reader ID) at a different point in time or periodically instead of coupling it with when Inventory/Command Reports are sent to AIOTF.

Define a “A-IoT Device Location Reporting periodicity” in Inventory/Command Request so that AIOTF can collect the A-IoT device location (i.e., the Reader ID) periodically (and not couple it with when Inventory/Command Reports are sent)

## Proximity determination

TR38.769 captures a couple of options as the potential design of proximity determination:

Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item.

Solution 1:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

Solution 2:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Proximity determination based on device-side measurements is not considered.

The Rel-19 WID clarified to consider the Proximity determination via Solution 1 in TR 38.769 only.

Proximity determination can uniquely identify a device within a very close range, similar to NFC use cases. As shown in the Figure below, this range can be much smaller than the reader's maximum coverage.

Lower power for R2D/D2R within this proximity range can limit the range. How to reduce the transmission power may be dependent on the range requirement, which could be provided by the AIOTF.

For example, the max range is 15m for device 1 in T1; but the proximity range could be <1m. From a reader point of view, it may need to know whether/how to control the transmitting power for R2D based on the range requirement of the proximity determination.

Fig　Low power for proximity determination

Desired range for proximity determination is typically smaller than range for A-IoT communication (i.e., Inventory/Command)

For proximity determination based on Solution 1 in TR 38.769 (to determine “nearby” devices), the reader may need to reduce its R2D transmit power to find A-IoT devices within the proximity determination range.

To enable the gNB to perform proximity determination an in the desired range, AIOTF needs to inform gNB to (1) perform proximity determination and (2) provide the proximity determination range to the gNB so that the gNB can adapt its R2D transmit power accordingly.

This indicator and range can be provided along with Inventory Request or can be provided in a dedicated “Proximity Determination Request” message.

AIOTF requests gNB to perform proximity determination via a “Proximity Determination request” flag in Inventory Request or via a dedicated message “Proximity Determination Request”.

AIOTF may further provide a range requirement for proximity determination in Inventory Request or via a dedicated message “Proximity Determination Request” (so that the gNB knows the desired range for proximity determination and adapt its R2D Tx power accordingly).

# Summary

2.1	How is Reader ID sent in Inventory Report?

Observation 1:	AIOTF should have knowledge about the readers supported by a gNB and their Global Reader ID(s) (i.e., Global gNB ID + Reader index) via OAM and/or via interface management procedure

Observation 2:	There is no need for gNB to include the Global Reader ID in Inventory Report if it already knows the mapping of gNB and its supported readers

Proposal 1:	gNB just includes the “Reader index” as Reader ID to AIOTF in Inventory Report (i.e., there is no need to include “Global gNB ID + Reader index”)

2.2	Reader ID in Command Report

Observation 3:	A-IoT devices might move to a different Reader’s coverage in the time between Inventory and Command.

Observation 4:	Including the Reader ID in Command Report can be useful to keep the latest  A-IoT device and reader association at the AIOTF and can be useful to know the “last serving reader” in case of back-to-back commands.

Proposal 2:	gNB may optionally send the “Reader index” to the AIOTF in Command Report

2.3	Periodic A-IoT location reporting

Proposal 3:	Define a “A-IoT Device Location Reporting periodicity” in Inventory/Command Request so that AIOTF can collect the A-IoT device location (i.e., the Reader ID) periodically (and not couple it with when Inventory/Command Reports are sent)

2.4	Proximity determination

Observation 5:	Desired range for proximity determination is typically smaller than range for A-IoT communication (i.e., Inventory/Command)

Observation 6:	For proximity determination based on Solution 1 in TR 38.769 (to determine “nearby” devices), the reader may need to reduce its R2D transmit power to find A-IoT devices within the proximity determination range.

Proposal 4:	AIOTF requests gNB to perform proximity determination via a “Proximity Determination request” flag in Inventory Request or via a dedicated message “Proximity Determination Request”.

Proposal 5:	AIOTF may further provide a range requirement for proximity determination in Inventory Request or via a dedicated message “Proximity Determination Request” (so that the gNB knows the desired range for proximity determination and adapt its R2D Tx power accordingly).