3GPP TSG-RAN3 Meeting #127bis	R3-251588

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for R18 mobility mechanisms

Document for:	Discussion/Decision

# Background

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms for the following objective in Rel-19 SON/MDT WI based on the open issues and agreements last meeting.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

### Distinguishing UE-based TA vs. NW provided TA

RAN2 agreed the following:

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

Correspondingly, RAN3 has the following open issue:

FFS whether to support Network based or UE based solution for TA acquisition type (UE-determined or gNB provided)

There is no consensus whether TA acquisition type is always available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

In our view, Source gNB-CU can always determine the TA acquisition type (gNB provided or UE determined) as it knows

- Whether it has configured the UE to perform UE-based TA measurement (e.g., via the ltm-UE-MeasuredTA-ID),

- whether the candidate gNB-DU has determined TA for that LTM candidate cell upon receiving DU-CU TA Information Transfer and whether it has forwarded that info to the source gNB-DU via CU-DU TA Information Transfer and

- whether the TA value was provided by source gNB-DU to UE in CSC upon receiving the DU-CU CELL SWITCH NOTIFICATION

Also, we think that there is no need for source gNB-DU to store and retrieve UE context to determine the TA acquisition type as source gNB-CU has all the information to determine the TA acquisition type.

Source gNB-CU can always determine the TA acquisition type (gNB provided or UE determined) as it knows

  - Whether it has configured the UE to perform UE-based TA measurement (e.g., via the ltm-UE-MeasuredTA-ID)

  - Whether the candidate gNB-DU has determined TA for an LTM candidate cell upon receiving DU-CU TA Information Transfer and whether it has forwarded that info to the source gNB-DU via CU-DU TA Information Transfer.

  - Whether the TA value was provided by source gNB-DU to UE in CSC upon receiving the DU-CU CELL SWITCH NOTIFICATION

- There is no need for source gNB-DU to store and retrieve UE context to determine the TA acquisition type as source gNB-CU has all the information to determine the TA acquisition type.

There is no need for UE to report the TA acquisition type (gNB indicated or UE determined) to the gNB in RLF Report.  Send an LS to RAN2 that RAN3 has agreed on NW-based solution.

### UHI enhancements for LTM

The following was agreed in last RAN3 meeting:

DU should be able to resolve LTM ping-pong based on “information” over F1

No consensus whether UHI can be forwarded from CU to DU to assist DU in resolving LTM ping-pong

CU can adjust the LTM candidate cell list based on UHI

gNB-DU is responsible for selecting the target cell among the configured LTM candidates and sending the LTM CSC MAC-CE. Therefore, in our view gNB-DU should be responsible for avoiding ping-pong LTM cell switches. To assist the gNB-DU to resolve LTM ping-pong, we think that the gNB-CU may forward the UHI to gNB-DU.

gNB-CU may forward UHI to gNB-DU, so that gNB-DU can resolve ping-pong LTM cell switches. Enhance F1AP to support this.

Upon completing LTM cell switch, gNB-DU indicates ACCESS SUCCESS message to gNB-CU. Upon receiving this, gNB-CU should update the UHI to include the cell ID, time stayed in the cell and add an indicator for the “HO Type” (i.e., LTM or L3) to distinguish L3 and LTM cell switch.

Further, a UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and L1 execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM)

We therefore propose RAN3 to discuss how to distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM. One solution could be to optimize the UHI to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

To distinguish ping-pong handovers vs. back-to-back L3 HO and LTM, add an indicator in UHI to indicate the “last HO Type” (L3 or LTM)

### Optimizing beam information indicated in LTM CSC

RAN2 sent an LS to RAN3, which is copied below:

RAN2 have discussed the issues of:

- BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and

- LTM cell switch failure due to wrong beam

On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution.

RAN2 respectfully asks RAN3 to consider defining a network-based solution for this.

Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the latter issue.

RAN2 respectfully asks RAN3 to consider defining a network-based solution for the issue of BFR shortly after successful LTM cell switch (“near failure”).

Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the case of LTM cell switch failure due to wrong beam.

From TS 38.300,

The gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available.

As mentioned above, the gNB-DU can indicate beam information (TCI state ID, UL TCI state ID) in LTM cell switch command MAC CE to the UE.

The UE can use this information during LTM execution e.g., considers the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell during RACH-less LTM cell switch.

It is possible that the gNB-DU might not have indicated the right TCI state ID and therefore the UE might have used the wrong one thereby resulting in a near failure LTM cell switch failure. We propose the following network-based solution for obtaining the knowledge of the “recovered beam” or “reconnected beam”.

In case there is a beam failure shortly after a successful RACH-less LTM cell switch and UE successfully performs BFR in another beam, target gNB-DU can indicate the "recovered beam ID" to the source gNB-DU via the gNB-CU.

In case there is a RLF after receiving an LTM CSC and the UE reconnects in the same target cell but in a beam different from the one indicated in LTM CSC, target gNB-DU can indicate the "reconnected beam ID" to the source gNB-DU via the gNB-CU

Reply to RAN2 that RAN3 have agreed on a NW based solution to optimize indicated beam information for both "near failure" and "LTM cell switch failure"

## 2.2 MRO for CHO with candidate SCG

The following was agreed in the RAN3#126:

The failure types defined for CHO in TS 38.300 and for CPAC defined in TS 37.340 are used as baseline. Clarification or amendment could be made on top of that if needed

The cases identified for MRO for CHO with candidate SCG were classified as follow:

Case 1a), 2a), 3a), 9a): Too late handover (i.e., Too late CHO execution)

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO (i.e. Too early CHO execution) or Handover (i.e. CHO execution) to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

### 2.2.1 SCG Failure when UE is configured with CHO with candidate SCG

The following text was agreed in a TP to 37.340:

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis.

If the SCG failure has been caused by a different M-NG-RAN node, the serving M-NG-RAN node transfers the SCGFailureInformation to that M-NG-RAN node.

- Relevant RAN3 agreements:

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g., for case 3b)

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, FFS whether additional information (e.g. CPC failure type) needed in the new message

- In our view, it is sufficient if the node receiving the SCGFailureInformation from UE transfers the SCGFailureInformation to a candidate MN “immediately” in a UE-associated message i.e., there is no need to aggregate SCG Failures from multiple UEs and send it in a non-UE associated message. Therefore, the following is proposed”

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the node receiving the SCGFailureInformation from UE performs initial analysis and may transfer the SCGFailureInformation to a candidate MN “immediately” in a UE-associated message i.e., there is no need to aggregate SCG Failures from multiple UEs and send it in a non-UE associated message.

### 2.2.3 Concurrent RLF + SCG Failure scenario

Agreements and open issues so far:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed

Do not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure)

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 3c, 7c, 8c are FFS

FFS whether to support concurrent failure cases (RLF + SCG failure). The definition of these cases needs to be further clarified.

Concurrent MCG and SCG failures can be deprioritized as this is not specific to CHO with candidate SCG. This was also considered in R17/R18 (e.g., MCG failure before UE sends SCGFailureInformation) but no enhancements were specified.

The other scenario where fast MCG recovery via SCG could not be initiated because SCG fails immediately after MCG fails is already deprioritized as RAN3 agreed to not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure). Therefore, the following is proposed:

There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c

# Summary

2.1	MRO for LTM

Proposal 1:	Source gNB-CU can always determine the TA acquisition type (gNB provided or UE determined) as it knows

a.	Whether it has configured the UE to perform UE-based TA measurement (e.g., via the ltm-UE-MeasuredTA-ID)

b.	Whether the candidate gNB-DU has determined TA for an LTM candidate cell upon receiving DU-CU TA Information Transfer and whether it has forwarded that info to the source gNB-DU via CU-DU TA Information Transfer.

c.	Whether the TA value was provided by source gNB-DU to UE in CSC upon receiving the DU-CU CELL SWITCH NOTIFICATION

Observation 1.	There is no need for source gNB-DU to store and retrieve UE context to determine the TA acquisition type as source gNB-CU has all the information to determine the TA acquisition type.

Proposal 2:	There is no need for UE to report the TA acquisition type (gNB indicated or UE determined) to the gNB in RLF Report.  Send an LS to RAN2 that RAN3 has agreed on NW-based solution.

Proposal 3:	gNB-CU may forward UHI to gNB-DU, so that gNB-DU can resolve ping-pong LTM cell switches. Enhance F1AP to support this.

Proposal 4:	To distinguish ping-pong handovers vs. back-to-back L3 HO and LTM, add an indicator in UHI to indicate the “last HO Type” (L3 or LTM)

Proposal 5:	In case there is a beam failure shortly after a successful RACH-less LTM cell switch and UE successfully performs BFR in another beam, target gNB-DU can indicate the "recovered beam ID" to the source gNB-DU via the gNB-CU.

Proposal 6:	In case there is a RLF after receiving an LTM CSC and the UE reconnects in the same target cell but in a beam different from the one indicated in LTM CSC, target gNB-DU can indicate the "reconnected beam ID" to the source gNB-DU via the gNB-CU

Proposal 7:	Reply to RAN2 that RAN3 have agreed on a NW based solution to optimize indicated beam information for both "near failure" and "LTM cell switch failure"

2.2 MRO for CHO with candidate SCG

Proposal 8:	In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the node receiving the SCGFailureInformation from UE performs initial analysis and may transfer the SCGFailureInformation to a candidate MN “immediately” in a UE-associated message i.e., there is no need to aggregate SCG Failures from multiple UEs and send it in a non-UE associated message.

Proposal 9:	There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c