3GPP TSG-RAN WG3 Meeting #127bis	R3-251608

Wuhan, China, 7 – 11 April 2025

Agenda Item:	11.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Network Slicing

Document for:	Discussions & Approval

- Introduction

During the last RAN3 meeting, it was agreed that the UE performance per S-NSSAI defined by SA5 can be referenced by RAN3 as feedback, and finer granularity of UE performance was not excluded.

However, some remaining issues need further discussion:

Whether to introduce a new IE or extend the existing bitmap to request slice-specific UE performance in the Data Collection Request message.

Should the Rel-18 signaling design be taken as the starting point?

How to include the Failed Report Characteristics in the Data Collection Response message.

In this contribution, we further elaborate on our understanding of the above issues and provide the corresponding TP[1][2].

- Discussion

2.1 Slice UE performance report

Regarding the UE performance, it was agreed to reuse the UE performance per S-NSSAI defined by SA5 to transfer UE performance from target node to source node upon request. Therefore, following architecture is proposed to reflect the slice-level UE performance per S-NSSAI:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE performance List |  | 0..1 |  |  |
| > Slice UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

- Capture the UE performance per S-NSSAI in the DATA COLLECTION UPDATE message.

2.2 Slice UE performance request

Another issue is how to request the UE performance per S-NSSAI (slice-level granularity). During the RAN3 discussions, two options have been proposed to realize this functionality.

Option 1: Introduce a new IE in the UE performance configuration IE to request slice-specific UE performance

This IE indicates the configuration for UE performance measurement collection. If the Slice Level UE Performance Reporting IE is set to “true” then the UE Performance measurements are collected on a per slice granularity.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond | - |  |
| Slice Level UE Performance Reporting | O |  | ENUMERATED (true, …) | Indicates that UE Performance measurements are collected on a per slice granularity. | YES | ignore |

Limitation: This option does not support simultaneous configuration of both legacy per-UE and slice-specific UE performance in the same request. It limits flexibility and may require multiple requests. Additionally, it may not align well with the general bitmap-based mechanism used elsewhere in the report characteristics IE, causing inconsistency in design.

Option 2: Extend Existing Report Characteristics Bitmap

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Slice level Average UE Throughput DL, Eleventh Bit = Slice level Average UE Throughput UL, Twelfth Bit= Slice level Average Packet Delay, Seventh Bit = Slice level Average Packet Loss DL Other bits are ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

This approach extends the Report Characteristics for Data Collection IE (BIT STRING) by adding new bit positions for slice-level UE performance metrics:

- Tenth Bit: Slice-level Average UE Throughput DL

- Eleventh Bit: Slice-level Average UE Throughput UL

- Twelfth Bit: Slice-level Average Packet Delay

- Thirteenth Bit: Slice-level Average Packet Loss DL

This option allows both legacy and slice-specific metrics to be requested simultaneously in a single message. It reuses the existing bitmap structure, ensuring consistency and signaling efficiency.

Considering the trade-offs, Option 2 is preferred. Despite the increase in bitmap size, Option 2 provides greater flexibility, enables simultaneous reporting configurations, and follows the existing extensible bitmap-based paradigm already defined in the Report Characteristics IE. It ensures that the NG-RAN node can interpret multiple measurement requests efficiently within a single message. Furthermore, from a practical deployment perspective, the need to toggle between legacy and slice-specific performance reporting is very likely. Option 2 addresses this with minimal signaling overhead and without requiring additional negotiation steps or multiple messages.

- It is proposed to proceed with Option 2, extending the existing Report Characteristics for Data Collection IE to include slice-level UE performance metrics.

  - Failed Report Characteristics

In alignment with the selected Option 2 for the request message (i.e., extending the bitmap in the Report Characteristics for Data Collection IE), the corresponding response message (Data Collection Response) also requires enhancements. Specifically, it needs to indicate situations in which the slice-level UE performance metrics cannot be initiated by NG-RAN node 2.

Since the failed results for legacy UE performance metrics are included within the existing Node Measurement Failed Report Characteristics IE, the most straightforward solution is to also include the slice-level UE performance metrics within this same IE.

| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Seventh Bit = Measured UE Trajectory Eighth Bit = Slice level Average UE Throughput DL, Ninth Bit = Slice level Average UE Throughput UL, Tenth Bit= Slice level Average Packet Delay, Seventh Bit = Slice level Average Packet Loss DL Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |

- Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

2.4 Capacity Class Value

As part of the ongoing work on AI/ML assisted network slicing, a new measurement item — Predicted Slice Available Capacity — has been introduced. This item refers to the Slice Available Capacity defined in TS 38.423 clause 9.2.2.55. As specified, Slice Available Capacity values can be weighted using the cell capacity class value, which is included in the Composite Available Capacity Group IE.

9.2.2.55	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

To accurately compute Predicted Slice Available Capacity, the NG-RAN node needs access to the cell capacity class value per cell. However, it is unclear how this value can or should be made available within the data collection procedure, particularly when prediction is involved. Several approaches are considered to ensure availability of this value as below:

- Option 1: Retrieve Cell Capacity Class Value via Resource Status Procedure.

- Option 2: Include Measured Cell Capacity Class Value in Data Collection Update message.

- Option 3: Introduce Predicted Composite Available Capacity IE in Data Collection procedure.

As we know, the Cell Capacity Class value is static (0..100), making Option 1 or Option 2 sufficient. Therefore, there is no need to introduce the Predicted Composite Available Capacity in the Data Collection procedure. Additionally, during the discussions on AI/ML-assisted load balancing, the Predicted Composite Available Capacity information was also not considered.

- There is no need to introduce the Predicted Composite Available Capacity in the Data Collection procedure if only the Cell Capacity Class value is required.

We recommend to proceed with Option 2 — introducing the measured cell capacity class value into the Data Collection Update message alongside the Predicted Slice Available Capacity. This ensures independence from the Resource Status Procedure, and consistency of AI/ML-based prediction reporting.

- Proceed with Option 2 — introducing the measured cell capacity class value into the Data Collection Update message alongside the Predicted Slice Available Capacity

Corresponding TPs to 38.300 & 38.423 to reflect the proposals above is provided in [1][2].

3	Conclusion

Following is our proposals and observations:

- Capture the UE performance per S-NSSAI in the DATA COLLECTION UPDATE message.

- It is proposed to proceed with Option 2, extending the existing Report Characteristics for Data Collection IE to include slice-level UE performance metrics.

- Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

- There is no need to introduce the Predicted Composite Available Capacity in the Data Collection procedure if only the Cell Capacity Class value is required.

- Proceed with Option 2 — introducing the measured cell capacity class value into the Data Collection Update message alongside the Predicted Slice Available Capacity

- Adopt the corresponding TPs to 38.300 & 38.423 to reflect the proposals above

4	Reference

[1] R3-251610 [TP to 38.423] AI/ML assisted Network Slicing ZTE Corporaiton

[2] R3-251609 [TP to 38.300] AI/ML assisted Network Slicing ZTE Corporation, Lenovo, Samsung