3GPP TSG-RAN WG3 Meeting #127bis	R3-251611

Wuhan, China, 7-11 Feb 2025

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Discussion on Mobility Optimization for NR-DC

Document for:	Discussions & Approval

1	Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in Rel-18 Leftovers, as captured in TR 38.743 [2], we present our analysis of the normative work for NR-DC Mobility Optimization, Support of split architecture, and Continuous MDT Collection.

In this contribution, we continue to discuss the remaining issues for mobility optimization for NR-DC and provide the corresponding TP.

2	Discussion

2.1 UE performance over NR-DC

Another aspect is how to collect the UE performance from SN side. As specified in TS 37.340 [3], from network perspective, each bearer (MCG, SCG and split bearer) can be terminated either in MN and SN, and network side protocol termination options for MCG, SCG and split bearer in NR-DC as shown in Figure 2.

Figure 2. Network side protocol termination options for MCG, SCG and split bearers in MR-DC with 5GC

The MN can obtain the UE performance (e.g., UE throughput, packet delay and packet loss) for MN-terminated MCG, SCG, and split bearer. If MN would like to obtain the UE performance for a UE, SN may need to report UE performance for the SN-terminated MCG/SCG/splt bearer.

Existing UE performance over Xn interface is UE-level UE performance, which includes Average UE throughput UL/DL, Average Packet Delay UL/DL, Average Packet Loss DL. As defined in TS 38.314 and TS 28.552, the UE throughput, packet delay and packet loss can be measured per UE per DRB.

Packet Delay:

The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE.

The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms.

Packet Loss:

NOTE 2:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

UE throughput:

The granularity for PDCP SDU Data Volume measurement defined in TS 28.552 [2] is per DRB per UE.

The granularity for Average UE throughput measurement defined in TS 28.552 [2] is per UE and per DRB per UE.

Therefore, in the case of NR-DC, it is proposed to transfer the UE performance per DRB level over Xn interface for SN-terminated resources.

- In the case of NR-DC, introduce UE performance at the per DRB level over Xn for SN-terminated resources, reported from the SN to the MN.

As a starting point, the Rel-18 UE performance at per-UE granularity can also be considered for reporting from the SN to the MN.

- Rel-18 UE performance at per-UE granularity can also be considered for reporting from the SN to the MN.

2.2 Exit condition of collection over NR-DC

During the Rel-18 phase, the exit condition for the collection of measured UE performance and measured UE trajectory was captured in TS 38.423.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Now UE performance is also considered as feedback report from SN to MN. Therefore, the exit condition in case of NR-DC can also be defined. Firstly, the scenario “UE moves to RRC_INACTIVE or RRC_IDLE state” can reused for NR-DC case. Moreover, the time since successful completion of SN Addition procedure is equal to the value of the Collection Time Duration for UE Performance IE is the specific scenario for NR-DC. Finally, when the SN is changed or released, the collection should also be terminated.

Therefore, the exit condition for NR-DC case are concluded as follows:

-	the time since successful completion of SN Addition procedure is equal to the value of the Collection Time Duration for UE Performance IE;

-	S-NG-RAN node is changed or released;

-	UE moves to RRC_INACTIVE or RRC_IDLE state

- Capture the following exit conditions of the termination of the collection of UE peformance in case of NR-DC:

- the time since successful completion of SN Addition procedure is equal to the value of the Collection Time Duration for UE Performance IE;

- S-NG-RAN node is changed or released.

The corresponding TP to 38.423 is provided.

3	Conclusion

Following is our proposals and observations:

- In the case of NR-DC, introduce UE performance at the per DRB level over Xn for SN-terminated resources, reported from the SN to the MN.

- Rel-18 UE performance at per-UE granularity can also be considered for reporting from the SN to the MN.

- Capture the following exit conditions of the termination of the collection of UE peformance in case of NR-DC:

- the time since successful completion of SN Addition procedure is equal to the value of the Collection Time Duration for UE Performance IE;

- S-NG-RAN node is changed or released

4	Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] TS 37.340