3GPP TSG-RAN WG3 Meeting #127bis	R3-251617

Wuhan, China, 7-11 April 2025

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	[TP to 38.455 & 38.401] Support of AI/ML assisted Positioning (case 3a)

Document for:	Other

- Introduction

The objectives of AI/ML for air interface WI [1] are listed as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the current discussions in RAN1 and RAN2, the topic that requires normative work in RAN3 is AI/ML-based positioning. Further analysis indicates that Case 3a and Case 3b may have impacts on RAN3.

In previous meetings, the initial RAN3 progress was as follows:

The existing LCS framework is re-used for AI/ML positioning.

Case 3a agreements:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

- NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

- RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

- Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

- WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

For case 3b:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

In RAN3#127 meeting, further agreements for case 3a are reached:

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement.

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

In this contribution, we provide our understanding on the case 3a about assisted AI/ML positioning.

- Discussion

  - Case 3a (AI/ML assisted positioning)

2.2.1 Model training

In the last meeting, there was a heated discussion regarding Case 3a (AI/ML-assisted positioning). The main point of contention was how the gNB should request training data from the LMF and whether the gNB can trigger the request message to the LMF without adhering to the principle that the LMF must always initiate the transaction ID.

As specified in the TS 38.305,

Positioning and data acquisition transactions between a LMF and NG-RAN node are modelled by using procedures of the NRPPa protocol. There are two types of NRPPa procedures:

-	UE associated procedure, i.e. transfer of information for a particular UE, including the procedures supporting the Positioning Information Transfer, E-CID Location Information Transfer and Measurement Preconfiguration Information Transfer functions;

-	Non UE associated procedure, i.e. transfer of information applicable to the NG-RAN node and associated TRP, including the procedures supporting the OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer and PRS Information Transfer functions.

Parallel transactions between the same LMF and NG-RAN node are supported; i.e. a pair of LMF and NG-RAN node may have more than one instance of an NRPPa procedure in execution at the same time.

For possible extensibility, the protocol is considered to operate between a generic "access node" (e.g. gNB, ng-eNB) and a "server" (e.g. LMF). An NRPPa transaction is only initiated by the server.

The NRPPa transaction can only be initiated by the LMF. Therefore, for AI/ML-based positioning, this NRPPa transaction must also be initiated by the LMF. Otherwise, it would violate the traditional positioning mechanism. Whether in the context of AI/ML for Air or AI/ML for NG-RAN, AI/ML is merely an enhancement tool. We should not disrupt the legacy mechanism to enhance a specific AI/ML case. Instead, AI/ML should be introduced within the framework of legacy operations.

- For AI/ML-based positioning, the NRPPa transaction must also be initiated by the LMF, following the principle of legacy positioning mechanism.

Another key issue to be discussed is which entity should perform UE selection:

- LMF selects the UE for data collection

- NG-RAN selects the UE for data collection

According to the current agreement, "For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs." To facilitate AI/ML-assisted positioning effectively, determining which entity should perform UE selection is critical, especially in scenarios involving non-served UEs. From a technical standpoint, when considering non-served UEs, the LMF emerges as a more appropriate entity for UE selection compared to the NG-RAN node (gNB). The reasoning is outlined below:

- The LMF maintains a network-wide perspective, inherently possessing the ability to access positioning-related information from multiple NG-RAN nodes. This centralized positioning context enables the LMF to make more informed decisions regarding UE selection, particularly in scenarios involving non-served UEs whose relevant positioning data may span multiple gNB coverage areas.

- If the NG-RAN node (gNB) is tasked with UE selection, especially for non-served UEs, the complexity significantly increases due to potential signaling overhead and the need for inter-node coordination.

Given these considerations, it is recommended to align UE selection responsibilities with the LMF.

- For case 3a, it is recommended that LMF perform the UE selection for data collection.

Therefore, the LMF can first trigger a Class 2 NRPPa procedure to inform NG-RAN nodes that training data collection is available and provide initial UE-related information. Subsequently, the NG-RAN nodes can respond to the training data collection request from the LMF.

Regarding the procedure for how an NG-RAN node can request training data collection, there appear to be three possible options:

- Option 1: Reuse the existing NRPPa procedure, where the request from the NG-RAN node is embedded within a legacy procedure — i.e., the existing NRPPa response message is leveraged to signal the request requirement.

- Option 2: Introduce a new data collection initiation and reporting procedure over NRPPa, allowing the NG-RAN node to request Part B information from the LMF. This would follow a similar mechanism to the data collection procedure over XnAP.

- Option 3: Utilize an NGAP-based solution to enable the NG-RAN node to request training data collection from the LMF.

Given these considerations regarding Option 1, introducing a new dedicated DATA COLLECTION REQUEST procedure (Option 2) appears to be the most appropriate solution.

- The LMF can trigger a new Class 2 NRPPa procedure, such as a NRPPa DATA COLLECTION NOTIFICATION procedure, to inform NG-RAN nodes that training data collection is available.

- The NG-RAN node can inform the LMF about its data collection requirements via an new Class 1 procedure, such as NRPPa DATA COLLECTION INITIATION procedure to request the training data and is requirement.

Once the LMF receives a training data request from the NG-RAN node, the LMF can respond by indicating whether the requested training data can be provided, based on the specifics of the request. Subsequently, the LMF proceeds with UE selection and initiates the legacy positioning procedures. After successfully collecting the appropriate Part B information, the LMF then provides this information back to the requesting NG-RAN node. The following is the procedures for case 3a data collection.

Figure 1. The request training data requirement from gNB to LMF

- Introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP.

Following the RAN1 agreement, NG-RAN nodes can correlate Part A and Part B information using time stamp. Therefore, the matching issue is resolved and does not require further discussion in RAN3.

- RAN3 is not required to further discuss the correlation between Part A and Part B.

2.2.2 Model performance evaluation

In the previous meeting, RAN3 has achieved a working assumption that RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

And following is the RAN1 agreement on this issue:

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

RAN3 now agreed that NG-RAN node performs monitoring metric calculation for its own model. Now, there are 2 different ways for model performance evaluation：

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

For model performance evaluation, RAN3 discussed this topic during the R17 and R18 stages and concluded that model performance is out of RAN3's scope. This wording to reflect this into stage2 specification is still under discussion regarding whether it needs further revision, but the principle is to avoid transmitting model performance metrics over NG-RAN interfaces (e.g., loss, accuracy, etc.).

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

RAN1 requests confirmation from RAN3 regarding the performance monitoring metric calculation.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

  - Stage 2 impact

For the AI/ML topic, it is important to specify which network entity is responsible for model training and model inference. In the case of AI/ML for NG-RAN, the deployment is already documented in the specification. Therefore, the deployment of AI/ML-assisted positioning should also be explicitly mentioned in the specification.

For AI/ML-assisted positioning (Case 3a):

- Model inference is performed at the gNB-CU.

- Model training is performed at either the gNB-CU or OAM.

- Capture the deployment for AI/ML assisted positioning in the TS 38.401.

- Conclusion

Following is our proposals and observations.

- For AI/ML-based positioning, the NRPPa transaction must also be initiated by the LMF, following the principle of legacy positioning mechanism.

- For case 3a, it is recommended that LMF perform the UE selection for data collection.

- The LMF can trigger a new Class 2 NRPPa procedure, such as a NRPPa DATA COLLECTION NOTIFICATION procedure, to inform NG-RAN nodes that training data collection is available.

- The NG-RAN node can inform the LMF about its data collection requirements via an new Class 1 procedure, such as NRPPa DATA COLLECTION INITIATION procedure to request the training data and is requirement.

- Introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP

- RAN3 is not required to further discuss the correlation between Part A and Part B.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

- Capture the deployment for AI/ML assisted positioning in the TS 38.401.

- Text Proposal – TS 38.401

7.6	Positioning

The NG-RAN supports the positioning functionality as specified in TS 38.305 [25].

To support of AI/ML assisted positioning, in case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

- Text Proposal to TS 38.455

Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Data Collection Transfer. This function allows the NG-RAN node to request the data information to support AI/ML assisted positioning.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Transfer | Data Collection Request Data Collection Response Data Collection Update |

8	NRPPa procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Data Collection Transfer | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Update | DATA COLLECTION UPDATE |

8.2	Location Information Transfer Procedures

8.5.X Data Collection Transfer

8.5.1	Data Collection

8.5.1.1	General

The Data Collection procedure allows the NG-RAN node to request AI/ML positioning measurements from LMF. This procedure applies only if the NG-RAN node is a gNB.

8.5.1.2	Successful Operation

Figure 8.5.X: Measurement procedure. Successful operation.

8.5.1.3	Unsuccessful Operation

Figure 8.5.X: Data Collection Procedure. Unsuccessful operation.

8.5.1.4	Abnormal Conditions

8.5.2	Data Collection Update

8.5.2.1	General

The Data Collection Update procedure allows the LMF to report AI/ML assisted positioning measurements to the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

8.5.2.3	Abnormal Conditions

Void.

- Draft LS to RAN1

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, Greece, 17-21 Feb 2025

Title:	[Draft] LS on AI/ML assisted Positioning about model performance monitoring

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

- Overall description

RAN3 discussed the following options for model performance monitoring in the case of case 3a,

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision

RAN3 prefers the option 1 that gNB performs model monitoring metric calculation and makes monitoring decision to avoid transmitting model performance metrics over NG-RAN interfaces.

- Actions

To RAN1:

ACTION: 	RAN3 kindly asks RAN1 to take the above information into consideration.

3	Dates of next TSG SA WG 5 meetings

RAN3#128	17 February 	- 21 February   2025		China