3GPP TSG-RAN WG3 Meeting #127bis                                                           R3-251623

Wuhan China, April 7 – 11, 2025

Agenda item: 		17.2

Source: 		Qualcomm Incorporated

Title: 	Aspects of on-demand SSB for NES enhancements

Document for:		Discussion

# 1	Introduction

In last meeting, RAN3 discussed aspects related to OD SIB1. Chair nodes capture the following topic for further discussion [1]:

The CU sends the allowed cells for on-demand SSB operation to the DU?

This contribution aims to make further progress on aspects related to on-demand SIB1.

# 2	Discussion

RAN1/2 discuss two cases for OD SSB:

Case 1: The cell intermittently broadcasts NCD SSB.

Case 2: The cell periodically broadcasts CD (or NCD) SSB. Additional transmissions can be added OD.

Impact on Xn

Case 1: UEs connected to or camping on cells of peer gNBs do not measure the intermittently broadcasted SSB since it is a NCD SSB. Therefore, there is no Xn impact.

Case 2: Since RAN2 agreed to use legacy SMTC, UEs connected to or camping on cells of peer gNBs can measure the periodic CD SSB broadcast. The measurements will not be affected by additional OD SSB transmissions. Therefore, no coordination is needed between gNBs.

Proposal 1: No Xn enhancements anticipated since for Case 1, only NCD SSB is used, and for Case 2, no coordination is required between gNBs.

Impact on F1

Case 1 and 2: SSB is owned by the DU. Since there is no need for inter-CU coordination, there is also no obvious need for the CU to restrict the cells for on-demand SSB operation.

Proposal 2: No need for the CU to send the allowed cells for OD SSB to the DU.

# 3	Conclusion

This contribution discussed aspects related to on-demand SSB. The following observations and proposals have been made:

Proposal 1: No Xn enhancements anticipated since for Case 1, only NCD SSB is used, and for Case 2, no coordination is required between gNBs.

Proposal 2: No need for the CU to send the allowed cells for OD SSB to the DU.

# Reference

[1]  Chair notes, RAN WG3 meeting #127, Athens, Greece, Feb 2025