3GPP TSG-RAN3 Meeting #127bis	R3-251625

China, CN, April 7th – 11th 2025

Agenda Item:	18.2

Source: 	Qualcomm Incorporated

Title: 	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE

Document for:	Discussion/Decision

# Background

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

WID of Low-power wake-up signal and receiver for NR (LP-WUS/WUR) (RP-240801) has the below objective for RAN3:

In this contribution, we discuss the details on supporting LP-WUS subgrouping to indicate paging via LP-WUS for a UE in RRC_IDLE/RRC_INACTIVE states.

# Discussion

## 2.1 LP-WUS Extended UE Identity Index Value IE in F1 Paging

In RAN3#125bis [1], RAN3 discussed whether to introduce a new IE to include extra bits for UE_ID based LP-WUS subgrouping, i.e. the Extended UE Identity Index Value IE into F1 paging. However, no agreement has been made.

For PEI, Extended UE Identity Index Value IE was introduced in the “Core Network Assistance Information for RRC INACTIVE” sent by AMF to NG-RAN. The format of the IE and description is below.

Extended UE Identity Index Value IE

This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UE Identity Index Value | M |  | BIT STRING (SIZE(16)) | Encoded as 5G-S-TMSI mod 65536. |

The Extended UE Identity Index Value IE was introduced because the 10-bit size of the UE Identity Index value IE was insufficient for deriving UE_IDs for various scenarios, such as normal paging, PEI with UE_ID-based subgrouping, and with/without eDRX, without any overlap between the UE_IDs. The 16-bit size for the Extended UE Identity Index Value IE was determined based on the UE_ID formula (e.g., 5G-S-TMSI mod 1024, 5G-S-TMSI mod 4096, etc.) for these different scenarios as defined in TS38.304[2].

Observation 1: For PEI, Extended UE Identity Index Value IE was introduced in the “Core Network Assistance Information for RRC INACTIVE” sent by AMF to NG-RAN.

Observation 2: For PEI, the size of the Extended UE Identity Index Value, is dependent on the formula defined for PEI UE ID computation.

gNB-CU forwards the received Extended UE Identity Index Value IE to the gNB-DU via F1 AP paging message, to assist the gNB-DU determine UE_ID.

Observation 3: gNB-CU sends the Extended UE Identity Index Value IE in the F1 AP paging message sent to gNB-DU

In RAN2, it was agreed that a similar formula as PEI subgrouping can be applied for LP-WUS. However, it has not yet been decided whether the same UE_ID formula will be reused for LP-WUS. Without knowing the exact formula for LP-WUS-based UE_ID computation, it is difficult to determine whether the existing Extended UE Identity Index Value IE size is sufficient.

Observation 4: RAN2 has not yet defined the formula for LP-WUS UE_ID based subgrouping ID computation.

Proposal 1: RAN3 wait for RAN2/RAN1 progress to decide which values of X are used for UE_ID computation in LP-WUS UE_ID based subgroup ID calculation, to determine whether there is a need to extend the ‘Extended UE Identity Index Value IE’ size further.

## 2.2 LP-WUS during emergency PDU Session

In RAN3#126 meeting, there was discussion on whether to disable LP-WUS during an emergency PDU Session, with no agreements made.

In SA2 #167, 17-21 February 2025, meeting there were discussions based on [3] on whether it is necessary to disable paging mechanisms like PEI, eDRX for a UE with an active emergency PDU session. SA2 agreed to remove the emergency PDU session restrictions for Paging Early Indication and LP-WUS from Rel.19 version of the specifications, as PSAP callback is delivered as normal call in IMS PDU session. SA2 prepared CR S2-2502428, [4], for this agreement.

Proposal 2: RAN3 agree to not disable PEI or LP-WUS when emergency PDU session is active, following SA2 agreement.

# Summary

Below are the observations and proposals discussed in this document.

Observation 1: For PEI, Extended UE Identity Index Value IE was introduced in the “Core Network Assistance Information for RRC INACTIVE” sent by AMF to NG-RAN.

Observation 2: For PEI, the size of the Extended UE Identity Index Value, is dependent on the formula defined for PEI UE ID computation.

Observation 3: gNB-CU sends the Extended UE Identity Index Value IE in the F1 AP paging message sent to gNB-DU

Observation 4: RAN2 has not yet defined the formula for LP-WUS UE_ID based subgrouping ID computation.

Proposal 1: RAN3 wait for RAN2/RAN1 progress to know which values of X are used for UE_ID computation in LP-WUS UE_ID based subgroup ID calculation, to determine whether there is a need to extend the ‘Extended UE Identity Index Value IE’ size further.

Proposal 2: RAN3 agree to not disable PEI or LP-WUS when emergency PDU session is active, following SA2 agreement .

# References

[1] (draft) RAN3 #125-bis Meeting Report, Hefei, China

[2] TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 18)

[3] S2-2500133, Emergency call back and “delayed” paging, Qualcomm Incorporated, 3GPP TSG-SA WG2#166-AdHoc-e

[4] S2-2502428, 23.501 CR6083 (Rel-19, 'F'): Removing unnecessary restrictions for emergency PDU sessions	Qualcomm	Rel-19