3GPP TSG-RAN WG3 #127bis	R3-251626

# Wuhan, China, April 7 – 11, 2025

Agenda Item:	7

Source:	Ericsson

Title: 	State of the art and perspectives on methods and tools for managing CRs and TSs in 3GPP

Document for:	Information

# Introduction

In the past decades the 3GPP organization never stopped to grow while the methods and tools used for managing CRs and TSs/TRs remained unchanged. Today, the ever-increasing number of contributions and CRs, and the large sizes of our TSs/TRs reveal more than ever the shortcomings of the tools we are currently using. The top 3 of these short comes are:

      - manual implementation of the CRs,

- browsing large TSs files,

- writing/checking proper ASN.1 code.

In this contribution we take a close look at each one of the above-mentioned shortcomings and we try to foresee possible solutions to overcome them.

# Discussion

## 2.1	Manual implantation of CRs

The drawback of manually implementing CRs was already pointed out in the email sent on the RAN3 reflector on November 1st, 2022, by the New Working Methods (NWM) Project oversight Committee (PoC).

Subject:3GPP stakeholder survey for 'next generation specification development' Date:Tue, 1 Nov 2022 19:39:22 +0100 Dear 3GPP Stakeholder, 3GPP has been extremely successful in developing generation after generation of mobile telecommunication standards. Our way of working has remained the same now for decades, and it is time to reexamine the tools we use for creating, revising and implementing CRs (and ideally pCRs as well.) The approach used now requires manual implementation and checks of each CR. As our organization grows this becomes difficult perhaps impossible to manage. In anticipation of 6G, it important to determine how to adopt a ‘next generation tool. …’

The problem of merging changes from different sources into a common base while resolving conflicts generated by conflicting changes, is not limited to our industry. Other industries, especially the software development industry faces the same problem but at a scale tens or hundreds times bigger than the one our organization is dealing with.

The solution adopted by the software industry, which later became a de facto standard solution for version control in many other industries, is so called “Git”. Today Git enables large and small projects to merge hundreds, even thousands of changes from different sources into a common base on a daily basis.

From https://git-scm.com/: Git is a free and open source distributed version control system designed to handle everything from small to very large projects with speed and efficiency.

When referring to Git it is also worth mentioning that:

- besides being an open-source software, Git works in exactly the same way on every possible operating system, e.g. Windows, MacOS, Linux, etc.

- Git primarily handles text files (e.g., .txt, .md, .json, .yaml, .xml, etc), but it can also manage binary files (e.g., .png, .jpg, .gif, .pdf, .docx, .xlsx) .

- While Git can handle binary files, it is generally more efficient with text files because it can track changes line by line. For binary files, Git can only detect changes in the entire file, making version control less efficient.

As a consequence of the above, for an efficient utilization of Git in 3GPP the format of our specifications would need to change from binary files (.docx) to text files (e.g. ,md, txt, .json, etc).

## 2.2	MS Word: Pain vs gain in documents formatting

MS Word is the tool used with predilection in 3GPP for writing contributions, CRs and specifications. MS word is used for its capability to cope with changes tracking within a file and especially for its rich text formatting functionalities. For specifications’ drafting and text formatting, 3GPP has defined a set of rules encompassed in TS 21.801 but the fact is that only a handful of people know these rules and correctly apply them in a process of drafting the specifications. Thus, at the end, the entire weight of making the specifications complaint with the formatting rules defined by 3GPP falls on the specifications’ rapporteurs shoulders.

One possible solution to overcome this is to separate the text editing and text formatting tasks either by using tools such as LaTeX or by using a markup language such as Markdown, alone or together with stylesheets that define the text formatting rules. The simplest, yet sufficient, solution is the later one because it requires no new tool installation as markdown files can be written with any text editor raging from the simplest ones such as Notepad++ to the powerful IDEs (Intelligent Development Environment) such as MS Visual Studio Code.

Being able to use any other tool for specifications drafting is yet another big advantage over MS Word as, lately MS Words struggles to cope with the large size of our specifications forcing the delegates to find tops and tricks such as “open in draft mode” which work more or less.

Figure 1 What is Markdown by Mistral AI

Figure 2: Markdown file with preview in VS Code

## 2.3	Writing and checking ASN.1 code

Another source of trouble when using MS Word for drafting our specifications is the ASN1 Part of our specifications. As MS Word does not offer any syntax verification for the ASN1 code, in every company, delegate who want to correctly edit the ASN1 code, need to, first extract the code from the Word document, then import or paste the code in some specialized ASN1 code editor, then make the changes and finally paste back the code in the word document while manually marking the changes if the Word document is a CR.

The result of this tedious process is the majority of CRs being submitted with incorrect ASN1 code. Here again, as for the formatting misalignments, the burden of correcting the ASN.1 part of the specification falls on the specifications’ rapporteurs and this could be easily avoided by using the appropriate tools for editing the ASN1 part of our specifications.

For example, a free and open source tool which offers ASN1 syntax verification and many other code related functionalities at a “lightning” speed is Microsoft Visual Studio Code together with the “ASN.1, edit, encode and decode” extension from OSS Nokalva Inc which is already used by many companies in 3GPP.

Figure 3: “ASN.1, edit, encode and decode” extension from OSS Nokalva Inc for VS Code

Figure 4: Example of ASN1 error highlighting in VS Code

## 2.4	Git landscape in 3GPP

As far as we know, the following can be mentioned about the utilization of Git in 3GPP:

- From our recollection of the “3GPP stakeholder survey for 'next generation specification development” survey from 2022, Git is a strong candidate for the “next CR tool” position.

- 3GPP already hosts and manages a GitLab server (https://forge.3gpp.org/) on which, every delegate with an ETSI account, can create and work on Git projects.

- Interface definitions for the 5G service-based architecture specified by SA and CT working groups are stored in the 3GPP Forge Gitlab repository (https://forge.3gpp.org/rep/all/5G_APIs).

- RAN2 is currently running a trial for using a Git-based environment for specification drafting and CRs handling and it considers using it also for drafting the upcoming 6G Technical Report.

# 3	Concluding Considerations

In this contribution we discuss some of the shortcomings of the specifications drafting and CRs handling methods and tools, and we draw the basic principles for a solution that could enable the 3GPP Working Groups to overcome the drawbacks of the existing tools.

These principles are:

- Use Git for CR handling,

- Use Markdown for drafting the procedural text part of the specifications

- Use separate text files for the ASN1 parts of the specifications and use specialized tools to edit these files.

Proposal 1: RAN3 companies to consider the above and engage in the corresponding discussions e.g. in TSG RAN.

We would like to stress out that we don’t have the intention to bring changes to the contributions drafting methods and tools. MS Word remains a great tool for drafting contributions as nice and small and being not under version control like this one.