3GPP TSG-RAN WG3 Meeting #127                                                                   R3-251634

Athens, Greece, Feb 17th – 21st, 2025

Source:	CATT, China Telecom, CMCC

Title:	Discussion on L3-report based SCell activation

Agenda Item:	9.2

Document for:	Discussion and Approval

1 Introduction

In the Rel-18 NR RRM enhancement topic, RAN4 had some conclusions on unknown FR2 SCell activation enhancement with L3 reporting. RAN4 sent a LS[1] to RAN2 in RAN4#108 meeting (August 2023) for designing the corresponding signaling to support the enhancements.

Following details are captured in the LS to RAN2:

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation

Enhancement for multiple FR2 unknown SCells’ activation Agreements: - Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band. - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized TCI activation command transmitted is based on L3 measurement results or L1 measurement results? Agreements: - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

RAN4 introduced an enhancement to support L3-report based TCI activation during FR2 unknown SCell activation. Later RAN4 extended the same framework for both FR1 and FR2 Unknown SCells.

When SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported for the unknown SCell, the L3 measurement report containing SCell measurements will be sent to gNB including the beam level report. In this case, the gNB will receive the L3 SCell measurement report quickly within few milli seconds (note that UE already performed these L3 deactivated SCell measurements at low periodicity but were not reported to gNB before Aperiodic Scell activation) after sending SCell activation MAC-CE command, the gNB can activate TCI state (i.e., to activate a specific beam) for UE’s SCell based on the L3 measurement report so that it can significantly reduce the latency of SCell activation. Without this enhancement, UE will start legacy L1 measurements after receiving the SCell activation command and report L1 measurements, but result in significant delay for SCell TCI state activation, CSI reporting and subsequent scheduling from gNB.

To support this enhancement, RAN2 has agreed to introduce a new report type “reportOnScellActivation”. According to the CR[2] to RRC spec and CR[3] to MAC spec of RAN2, for the unknown SCell, L3 measurement results are reported upon UE receiving the SCell activation command MAC CE in case the new report type is configured to UE.

2 Discussion

According to RAN4, if the L3 report is triggered upon UE receiving the SCell activation command for the unknown SCell, gNB can activate the TCI state based on the L3 report instead of waiting for L1 report, in this way the activation of unknown SCell can be significantly accelerated. If both L3 and L1 measurement reports are received by gNB then it is left to implementation which measurements to be used for Scell TCI state activation. However, in split gNB architecture, the node receiving the L3 measurement report is not that transmitting the MAC-CE command of TCI activation, it is CU to receive the L3 measurement report while DU MAC is responsible for MAC CE based SCell activation and MAC CE based TCI state activation to UE. So, the L3-report based SCell activation in split architecture is not supported at present. RAN3 need to study whether and how to support it.

To support this enhancement, coordination between CU and DU is needed, there would be two potential options. One option is CU using the L3 report to select the beam of TCI activation and indicating the selected beam to DU, another option is CU delivering the L3 report to DU for DU to determine the beam of TCI activation.

Option 1: CU use L3 report to select the beam of TCI activation

When gNB-CU receives the L3 report from UE and is trigger by reportOnScellActivation, it knows the report is triggered due to SCell activation command. The gNB-CU can use the L3 report received from UE to determine the SCell beam, then indicates the selected beam to gNB-DU. We understand option 1 has these characteristics:

- New F1AP signalling is needed to deliver the selected beam from gNB-CU to gNB-DU.

- Per RAN4’s agreements, DU is assumed to send TCI activation command based on:

- Beam indicated by CU, if no L1-RSRP report is received by DU before transmitting the TCI activation command.

- Beam indicated by CU or L1-RSRP report (up to DU), if L1-RSRP report is also received by DU before transmitting the TCI activation command.

- According to legacy function split, CU is responsible for cell level configuration, while all the functions related to beam are located in DU. So, option 1 will potentially introduce a function out of the legacy to gNB-CU.

- CU knows when to send the beam indication to DU, but it does not know to which SCell(s) the DU needs to send TCI activation, as the L3 report trigger by SCell activation command MAC CE contains the results for all the UE’s serving cells. So, CU has to do the beam selection to all SCells and indicate to DU, which is not necessary.

- As gNB-CU is not aware of the information of TCI configuration, it can only carry out SSB level beam selection. Thus, gNB-DU shall probably send SSB level TCI activation command.

Option 2: DU use L3 report to select the beam of TCI activation

When gNB-CU receives L3 report with the type reportOnScellActivation, it can simply deliver the L3 report to gNB-DU. Then gNB-DU determines the beam of SCell based on the L3 report. We understand option 2 has these characteristics:

- New F1AP signalling is needed for delivering the L3 report of SCell from gNB-CU to gNB-DU.

- Based on RAN4’s agreements, DU is assumed to send TCI activation command based on:

- L3 report, if no L1-RSRP report is received by DU before transmitting the TCI activation command.

- L3 report or L1-RSRP report (up to DU schedular implementation), if L1-RSRP report is also received by DU before transmitting the TCI activation command.

- The function for gNB-DU determining the beam of serving cell based on L3 measurement report is already supported in Rel-18. In Rel-18 RACH-less HO, the RRC handover command may contain a beam identifier for the the target cell. The beam is determined based on L3 beam level measurement results for the target cell reported by UE. In split architecture, the RRC handover command is generated by gNB-DU and gNB-DU can select the beam used in UE’s target cell based on the L3 report transferred from CU to DU via the Handover Preparation Information IE. At this point, option 2 is aligned with present DU functionality.

- The gNB-DU can carry out both SSB level and finer beam selection for TCI activation.

Definition of RACH-less HO command in TS 38.331[4]:

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

Note that it is important to support this fast SCell activation enhancement, otherwise enhancements to Unknown Scell activation (for both FR1 and FR2) does not work in split gNB architecture and RAN4/RAN2 work will become useless. Without this enhancement, it will take few seconds to activate SCell and start scheduling.

From above analysis, it can be observed that both options are workable, we suggest RAN3 making the way forward to specify new F1AP signalling for the L3 report based SCell activation in split architecture based on one of above options.

Proposal: RAN3 agrees to support L3 report based SCell activation in split gNB architecture based on one of the following options:

Option 1: CU use L3 report to select the beam of TCI activation

Option 2: DU use L3 report to select the beam of TCI activation

3 Conclusion

In the previous sections we made the following proposals:

Proposal: RAN3 agrees to support L3 report based SCell activation in split gNB architecture based on one of the following options:

Option 1: CU use L3 report to select the beam of TCI activation

Option 2: DU use L3 report to select the beam of TCI activation

4 Reference

- R2-2309450, LS on FR2 SCell activation enhancements, RAN2#123bis, RAN4, to RAN2, cc RAN1

- R2-2313936, Introduction of FR2 SCell enhancements, RAN2#124, Apple, CATT, Ericsson, Xiaomi, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- R2-2313937, Introduction of FR2 SCell enhancements, RAN2#124, Xiaomi, Apple, CATT, Ericsson, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.4.0 (2024-12)