3GPP TSG-RAN WG3 Meeting #127bis                                                               R3-251639

Wuhan, China, 7th – 11th, April 2025

Source:	CATT

Title:	Discussion on remaining issue of NR Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting [1], NR Femto was studied and large progress was made. The architecture and functions for NR Femto have been captured in the BLCR of TS 38.300 [2].

In this contribution, we will discuss the remaining issue of NR Femto.

2 Discussion

In the last meeting, one issue was remained as follow [1]:

The Femto GW hosts the following function: - Selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions. To be continued…

In LTE, HeNBs may be mixed deployed via a HeNB GW, some HeNBs support IPv4 only, and other HeNBs support IPv6. When S1 signaling is terminated at HeNB GW, if HeNB GW does not terminate user plane, MME cannot determine the IP address version for S1-U according to the IP version of the S1-C interface. In that case, the MME sends two transport layer addresses of different versions, the HeNB GW takes the role of selection of an IP version to be used for S1-U.

In NR Femto, the situation is similar. According to the architecture in BLCR [2], when the Femto GW is deployed, the NG signaling is terminated at Femto GW while the IP layer of NG-U may not be terminated by Femto GW. The issue is that the AMF cannot determine the IP version of NG-U according to the IP version used by NG-C in that case.

Observation: The AMF cannot determine the IP version of NG-U according to the IP version used by NG-C in case the Femto GW is deployed while the IP layer of NG-U is not terminated by the Femto GW.

Referring to TS 36.413 [3], Transport Layer Address IE is included in the E-RAB SETUP REQUEST message during E-RAB setup for transport layer information. Transport Layer Address IE is a bit string of 32 bits, 128 bits, or 160 bits length. When the bit string is 32 bits, it includes an IPv4 address. When the bit string is 128 bits, it includes an IPv6 address. When the bit string is 160 bits, it includes an IPv4 and an IPv6 address, in which case the IPv4 address is contained in the first 32 bits. In case the HeNB GW does not terminate S1-U, the MME can sends the 160 bits Transport Layer Address IE of different IP versions contained in E-RAB configuration to HeNB GW, and the HeNB GW can select the appropriate IP version used for the E-RAB according to the IP version of the S1-C interface with the HeNB.

According to TS 38.413 [4], the PDU SESSION RESOURCE SETUP REQUEST message includes PDU Session Resource Setup Request Transfer IE. The PDU Session Resource Setup Request Transfer IE includes UL NG-U UP TNL Information IE, which indicates the UPF endpoint of the NG-U transport bearer. The UP Transport Layer Information IE is used to provide the NG UP transport layer information associated with a PDU session, and it includes the Endpoint IP Address IE, which contains a bit string up to 160 bits to support providing two different versions of IP address.

Thus, AMF sending two IP Addresses of different versions in the NR-U UP transport layer information can be supported, in case the Femto GW is deployed but it does not terminate the IP layer of NG-U. Then the Femto GW can take the role of IP version selection for NG-U if the NG-U UP transport layer information contains two IP addresses of different versions.

We propose that the NR Femto GW and the AMF host corresponding functions in TS 38.300.

Proposal 1: AMF hosts the function of send two IP Addresses of different versions in PDU session configuration, if the deployed Femto GW does not terminate the IP layer of NG-U.

Proposal 2: NR Femto GW hosts the function of selection of an IP version to be used for NG-U, if a PDU session configuration contains two IP Addresses of different versions.

Proposal 3: Capture above functions of NR Femto GW and functions of AMF in TS 38.300.

In the RAN3#126 meeting, access control was discussed and three modes were agreed [5] as follow.

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription. NOTE: The NR Femto node may use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

However, in the SA2#167 meeting, SA2 sent a LS[6] saying that the hybrid access mode (i.e., the second mode) is not supported by CAG according to present SA2 spec:

Among above three options, SA2 seeks some clarifications regarding the second option. As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list. As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration.

It can be observed that the second access mode for Femto which is captured in present stage-2 BLCR contradicts with the existing SA2 specifications. RAN3 should conclude that Femto does not support the hybrid access mode. The second access mode should be removed from present BLCR.

Proposal 4: Remove the second access control mode in TS 38.300 BLCR.

3 Conclusion

In the previous sections we made the following observations and proposals:

Observation: The AMF cannot determine the IP version of NG-U according to the IP version used by NG-C in case the Femto GW is deployed while the IP layer of NG-U is not terminated by the Femto GW.

Proposal 1: AMF hosts the function of send two IP Addresses of different versions in PDU session configuration, if the deployed Femto GW does not terminate the IP layer of NG-U.

Proposal 2: NR Femto GW hosts the function of selection of an IP version to be used for NG-U, if a PDU session configuration contains two IP Addresses of different versions.

Proposal 3: Capture above functions of NR Femto GW and functions of AMF in TS 38.300.

Proposal 4: Remove the second access control mode in TS 38.300 BLCR.

4 Reference

- RAN3#127 meeting report, RAN3 Chair

- R3-250943, (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects, Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, RAN3#127

- 3GPP TS 36.413, S1 Application Protocol (S1AP), V18.3.0 (2024-12)

- 3GPP TS 38.413, NG Application Protocol (NGAP), V18.4.0 (2024-12)

- R3-247829, Access Control with NR Femto – Stage 2 TP, Ericsson, Huawei, Nokia, China Telecom, Lenovo, Samsung, NTT DOCOMO, ZTE, Verizon, BT, RAN3#126

- R3-251524, LS on NR Femto node shared by PLMN and PNI-NPN, SA2