3GPP TSG-RAN WG3 Meeting #127bis                                                                R3-25xxxx

Wuhan, China, Apr 7th – 11th, 2025

Source:	CATT, Fujitsu

Title:	(TP to BLCR for XnAP) On support on-demand SIB1 for idle UE

Agenda Item:	17.3

Document for:	Discussion and Approval

1 Introduction

During RAN3#126 meetings[1], following agreements were achieved for on-demand SIB1:

RAN3#126 The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In the last meeting[2], agreements were made as below:

RAN3#127 It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

In this contribution, we will continue to discuss the on-demand SIB1 topic.

2 Discussion

2.1 XnAP signalling

Per agreements, NES gNB can instruct Cell A gNB to broadcast the UL WUS configuration by delivering the UL WUS configuration to Cell A gNB. But two issues are leftover. One is whether the request and the instruction of UL WUS configuration broadcasting from NES gNB to Cell A gNB can be separately delivered, another is whether the “start” instruction and UL WUS configuration can be de-coupled.

Whether the request and the instruction of broadcasting can be separately delivered over Xn?

The point to support the separate delivery:

When several Cell A are configured, the NES gNB would need to collect more than one Cell A gNBs’ acceptance to start OD-SIB1 operation. Cell A gNB(s) may not need to broadcast UL WUS configuration even if it has responsed the acceptance to NES gNB, that is because NES gNB may decide not to start OD-SIB1 operation. Thus, the NES gNB can first send a request to Cell A gNBs in order to collect “willingness” of each Cell A gNB, after which the NES gNB instructs Cell A gNBs to start broadcasting the UL WUS configuration if it decides to start OD-SIB1 operation.

Considerations on this issue:

- Cell A corresponding to a NES cell is basically pre-configured by OAM up to the operator, so Cell A is expected to have the capability of broadcasting UL WUS configuration and be willing to do that. On the other hand, the WUS configuration is broadcasted by SIBxx, it will occupy limited radio resource of Cell A, so Cell A would probably not refuse the request of NES cell. Thus, the refusion from Cell A gNB is quite seldom.

- There is no criterion on how many or which Cell A should accept the request before the NES cell entering OD-SIB1. According to RAN2’s running CR, a Cell A can be “any cell”, typically neighboring or overlapping with the NES cell. So, NES cell can enter OD-SIB1 status even if there is only one Cell A accepting the request, and thus the issue may not exist.

- Even if the issue exists, the NES cell can just indicate the Cell A which has accepted the request to stop broadcasting if the NES cell does not enter OD-SIB1.

Whether “start” instruction and UL WUS configuration delivery can be de-coupled over Xn?

The reason to support the decoupling:

If Cell A gNB can store the UL WUS configuration upon the first reception and keep storage even if the stop instruction is received from the NES gNB, NES gNB does not need to deliver the UL WUS configuration for the subsequent “start” instruction.

Considerations on this issue:

- Whether to store or remove storage is a pure implementation matter, it’s unusual for a gNB to expect other gNBs keep something stored. And it’s not suitable for a gNB to ask other gNBs to store or remove something. At this point, NES gNB can hardly have the knowledge of UL-WUS configuration storage in Cell A gNB, it is better to include the UL WUS configuration with each start instruction.

- Assuming that the UL WUS configuration storage in Cell A gNB is controlled by NES gNB, the NES gNB should always keep in mind the status of UL WUS configuration storage in Cell A gNB, which will certainly complicate the whole procedure. For example, besides the “stop UL WUS Configuration” instruction, the indication of “Cancel storage of UL WUS configuration” from Cell A gNB to NES gNB would be necessary.

- If the start instruction and WUS configuration are decoupled and the NES gNB sends “start” instruction to trigger Cell A to broadcast WUL WUS configuration, ambiguity may be caused. Because there would be multiple versions of WUL WUS configuration stored in Cell A gNB, it’s not clear which one to be used.

It can be observed that there would be no concrete reason to support the separate delivery of request and instruction of broadcast or decoupling the “start” instruction and UL WUS configuration delivery.

Observation 1: There is no concrete reason to support the separate delivery of request and instruction of UL WUS configuration or the decoupling of “start” instruction and UL WUS configuration.

Proposal 1: RAN3 to confirm that NES gNB always requests Cell A gNB to start UL WUS configuration broadcast with UL WUS configuration included in the request message.

A UL WUS Config IE should be introduced in each item of the list of NES cells in the request message. For each NES cell, Cell A gNB evaluates whether to accept the UL WUS configuration provision request. Cell A shall broadcast the UL WUS config to UE via SIBxx for the NES cells whose UL WUS config is accepted, upon reception of the request message. And Cell A should broadcast the UL WUS config exactly included in the request message, no matter whether it has stored the UL WUS config before.

If the Cell A gNB cannot accept UL WUS config for any NES cell, it can respond with a failure message with appropriate cause.

Proposal 2: Cell A broadcasts the UL WUS config included in the request message, no matter whether it has stored the UL WUS config before.

Proposal 3: Cell A gNB responds NES gNB with a failure message if it cannot accept UL WUS config for any NES cell.

To distinguish different UL WUS configuration provision procedures between NES gNB and Cell A gNB, a Provision ID IE can be added to the request message, response message and failure message.

Proposal 4: Provision ID is included in the request message, response message and failure message.

If the UL WUS configuration is modified, the new configuration should be delivered from NES cell to Cell A and broadcasted by Cell A. The new configuration delivery can reuse the same Xn procedure (i.e., UL WUS configuration provision procedure), i.e., NES cell provides the new WUS config to Cell A using the request message. Cell A which has accepted the request before can response whether it still accepts the new WUS config for the same NES cell via the response message. If Cell A accepts the new WUS config, it updates the new WUS config provision in SIBxx using SI modification procedure.

Proposal 5: If WUS configuration is modified, the new Xn procedure is reused for delivering the new WUS configuration from NES cell to Cell A for Cell A broadcasting the updated UL WUS configuration.

When UL WUS configuration is modified, because NES cell should broadcast its own UL WUS config, it needs at least one whole SI modification period to make the new UL WUS config take effect, and Cell A also needs at least one whole SI modification period to provide the new UL WUS config to UE. There is an issue that the time when the new UL WUS config takes effect in NES cell and the time when UEs camping on Cell A start to use the new UL WUS config may not be aligned with each other, because the SI modification period for NES cell and for Cell A may be different. As the longest SI modification period is 40.96s, the mis-aligned period between the two can be around 80s. If the UE camping on Cell A sends the UL WUS to the NES cell during the mis-aligned period, it would fail to receive OD-SIB1.

Take following figure as example, the SI modification period is 40.96s for Cell A, while it’s 10.24s for NES cell. When the new WUS config is provided to Cell A, the new WUS configuration is expected to be provided to UE in Cell A from point A, whereas it will take effect after point B in NES cell. If the UE camping on Cell A triggers OD-SIB1 during the period between A and B, it would fail because the new UL WUS config which the UE uses to trigger the OD-SIB1 has not taken effect in NES cell.

Observation 2: If WUS configuration is modified, the UE camping on Cell A may fail to receive OD-SIB1 if it triggers the OD-SIB1 during the mis-aligned period between Cell A and NES cell.

The potential solution is that Cell A and NES cell can exchange information on the time of new UL WUS configuration taking effect. There are three potential solutions:

- Solution 1: NES cell informs Cell A the time when the new WUS config will take effect (e.g., when the SI modification procedure finishes in NES cell)

By this solution, if point B is behind point A, Cell A can delay the time broadcasting the new WUS config from point A to point B to achieve the time alignment with NES cell.

- Solution 2: Cell A notifies the time when the new WUS config will be provided to UE (e.g., when SI modification procedure for the new WUS config finishes)

By this solution, if point A is behind point B, NES cell can delay the time of new UL WUS config taking effect from point A to point B to achieve the time alignment with Cell A.

- Solution 3: NES cell informs Cell A the time when the new WUS config will take effect and Cell A notifies the time when the new WUS config will be provided to UE

Solution 3 combines Solution 1 and 2. By solution 3, the time can be aligned anyway.

RAN3 can discuss how Cell A and NES cell exchange the time information on new UL WUS configuration taking effect during the new Xn procedure.

Proposal 6: RAN3 to discuss how Cell A and NES cell exchange time information on new UL WUS configuration taking effect via the new Xn procedure.

In the RAN3#126 meeting, RAN3 agreed that Cell A can initiate a class 2 Xn procedure to inform NES Cell gNB that it stops the UL WUS configuration provision.In the last meeting, it was agreed that NES cell can request Cell A to stop provision the UL WUS configuration.

RAN3#126: Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. RAN3#127: It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

Upon receiving the indication of stopping UL WUS config broadcast from Cell A, NES cell can consider the UL WUS config is no longer broadcasted by Cell A.

NES cell may request Cell A to stop broadcasting the WUS configuration, e.g., when the NES cell plans to get out of OD-SIB1 operation. The new class 1 Xn procedure can be reused. A per-NES-cell stop indicator can be introduced to the request message to indicate stopping UL WUS configuration provision for the NES cell. Upon reception of the request, Cell A gNB should stop broadcasting the WUS config in Cell A’s SIBxx, the SI modification procedure is used to update SIBxx.

Proposal 7: The new Class-1 Xn procedure is reused for NES cell requesting Cell A not to broadcast UL WUS config. A per-cell stop indicator is introduced to the request message.

2.2 F1AP signalling

As NES gNB-CU decides whether OD-SIB1 can be operated in the NES cell based on the response from Cell A(s), the NES gNB-CU should indicate its decision to the NES gNB-DU. NES gNB-CU can indicate the allowed cell(s) for OD-SIB1 to the NES gNB-DU so that the NES gNB-DU can activate OD-SIB1 to those allowed cells.

An allowed list of OD-SIB1 IE can be introduced to gNB-CU Configuration Update message, the list contains the identities of cells allowed to operate OD-SIB1.

Proposal 8: gNB-CU can indicate the allowed cell(s) for OD-SIB1 to gNB-DU. Add the Cells allowed for OD-SIB1 List IE in gNB-CU Configuration Update message.

If a NES cell previously allowed to operation SIB1 is no longer allowed, NES gNB-CU should indicate to gNB-DU.

Since the allowed list for OD-SIB1 IE is assumed to include identities of all the cells allowed to operate OD-SIB1, if any cell cannot keep OD-SIB1 operation, gNB-CU can exclude the cell from the allowed list for OD-SIB1 and send the updated list by gNB-CU Configuration Update message. There is no extra F1 impact.

Proposal 9: gNB-CU can update the allowed list of OD-SIB1 IE to indicate gNB-DU which cell is no longer allowed to operate OD-SIB1.

In RAN3#126 meeting[4], it was agreed that WUS configuration is provided from DU to CU via Serving Cell Information IE:

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

The gNB-DU plans to operate OD-SIB1 can provide the WUS config to gNB-CU. When the WUS configuration is modified by gNB-DU, NES gNB-DU also provides the new configuration to gNB-CU, so that NES gNB can provide the new configuration to Cell A gNB. gNB-DU can send the modified WUS configuration to gNB-CU via the UL WUS Configuration Information IE in Served Cell Information IE.

Proposal 10: The UL WUS Configuration Information IE in Served Cell Information IE is used to send the modified UL WUS config from gNB-DU to gNB-CU.

When the NES gNB-DU stops broadcasting the WUS configuration in NES cell, the WUS resource is no longer valid in the NES cell. In that case, cellA gNB should not broadcast the WUS configuration either. gNB-DU should be able to indicate gNB-CU that the WUS configuration is released, so that NES gNB can request CellA gNB to stop broadcasting the WUS config.

As discussed above, for the first WUS config broadcast or WUS config modification, UL WUS config info IE included in Served Cell Information IE can be used to convey the WUS configuration from DU to CU. So, there are two methods over F1:

- Using explicit indicator

- No explicit indicator, using the absence of WUS Config Info IE

For method B), given no explicit indicator, gNB-DU should always include the WUS Config Info IE in Served Cell Information IE when delivering the gNB-DU Configuration Update message to gNB-CU (otherwise the WUS config broadcast will be assumed not needed). If that’s the case, gNB-CU cannot learn whether the WUS config is updated or not. Thus, method B) is not feasible. A) is preferred, i.e., an explicit indicator should be introduced to indicate that the WUS config broadcast is not needed.

Proposal 11: A new indicator is introduced to Served Cell Information IE to indicate that UL WUS config provision is not needed.

3 Conclusion

In the previous sections we made the following observations and proposals:

XnAP siganlling

Observation 1: There is no concrete reason to support the separate delivery of request and instruction of UL WUS configuration or the decoupling of “start” instruction and UL WUS configuration.

Proposal 1: RAN3 to confirm that NES gNB always requests Cell A gNB to start UL WUS configuration broadcast with UL WUS configuration included in the request message.

Proposal 2: Cell A broadcasts the UL WUS config included in the request message, no matter whether it has stored the UL WUS config before.

Proposal 3: Cell A gNB responds NES gNB with a failure message if it cannot accept UL WUS config for any NES cell.

Proposal 4: Provision ID is included in the request message, response message and failure message.

Proposal 5: If WUS configuration is modified, the new Xn procedure is reused for delivering the new WUS configuration from NES cell to Cell A for Cell A broadcasting the updated UL WUS configuration.

Observation 2: If WUS configuration is modified, the UE camping on Cell A may fail to receive OD-SIB1 if it triggers the OD-SIB1 during the mis-aligned period between Cell A and NES cell.

Proposal 6: RAN3 to discuss how Cell A and NES cell exchange time information on new UL WUS configuration taking effect via the new Xn procedure.

Proposal 7: The new Class-1 Xn procedure is reused for NES cell requesting Cell A not to broadcast UL WUS config. A per-cell stop indicator is introduced to the request message.

F1AP signalling

Proposal 8: gNB-CU can indicate the allowed cell(s) for OD-SIB1 to gNB-DU. Add the Cells allowed for OD-SIB1 List IE in gNB-CU Configuration Update message.

Proposal 9: gNB-CU can update the allowed list of OD-SIB1 IE to indicate gNB-DU which cell is no longer allowed to operate OD-SIB1.

Proposal 10: The UL WUS Configuration Information IE in Served Cell Information IE is used to send the modified UL WUS config from gNB-DU to gNB-CU.

Proposal 11: A new indicator is introduced to Served Cell Information IE to indicate that UL WUS config provision is not needed.

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #126, MCC

- Draft Report of 3GPP TSG RAN3 meeting #127, MCC

5 Annex (TP to BLCR of 38.473)

-------------------------------------Start of TP---------------------------------------------

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells allowed for on-demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell(s) indicated by this IE is allowed for OD-SIB1.

>>>>>>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-----------------------------------------------------Next Change-----------------------------------------------------

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Unchanged parts are skipped |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed for On-demand SIB1 |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed for On-demand SIB1 Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

-------------------------------------Next Change---------------------------------------------

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| UL WUS Configuration Information | O |  | 9.3.1.xxx | Includes the information of UL WUS for OD-SIB1. | YES | ignore |

| - Range bound | - Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

-------------------------------------Next Change---------------------------------------------

#### 9.3.1.xxx	UL WUS Configuration Information

This IE contains information of UL WUS configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL WUS Config | O |  | Octet String | Includes the UL-WUS-Config-r19 in the SIBxx IE, as defined in TS 38.331 [8]. |
| Release Indicator | O |  | ENUMERATED (true, ...) | Indicates that the UL WUS configuration is released. |

-------------------------------------End of TP---------------------------------------------