3GPP TSG-RAN WG3 Meeting #127bis                                                                         R3-251644

Wuhan, China, 7th – 11st, April, 2025

Source:	CATT

Title:	Potential impacts of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and Decision

1 Introduction

In the last meeting[1], RAN3 had following conclusion:

For the following issues, including Extended UE Identity Index、The maximum value of CN subgroup ID、Emergency call back、the terminology of “LP-WUS”、 the LP-WUS node capability need more inputs from other WGs.

In this contribution, we will continue to discuss the impacts of LP-WUS, including aspects as below:

- Extended UE identity index

- Terminology and UE capability missing

- Emergency call back

- TP to BL CR for TS 38.410

2 Discussion

2.1 Extended UE identity index

In the last RAN2 meeting[2], it was agreed that the similar formular defined for PEI subgrouping is reused for LP-WUS subgroup:

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

According to the agreement above, it can be observed that:

- The number of subgroups for UE_ID based PEI subgrouping is used for deriving the UE_ID based LP-WUS subgroup

- If both UE_ID based LP-WUS subgrouping and UE_ID based PEI subgrouping are configured, the UEs belonging to the same PEI subgroup can be distributed to different LP-WUS subgroups. UEs monitoring the same PEI (i.e., belonging to the same PEI subgroup) can be distributed to subgroupsNumForUEID LP-WUS subgroups, which reduces the false paging alarms significantly.

- UE_ID should equal to 5G-S-TMSI mod X, but X has not been concluded by RAN2

According to the formulation, X should be determined by max(N*Ns*Np*subgroupsNumForUEID). Based on agreement in RAN1#120, the maximum value of subgroupsNumForUEID is 31. The maximum value of Np is 8. The maximum value of Ns is 4. The maximum value of N is 256 if eDRX is not used, while it’s 1024 for eDRX. So, X can be calculated as:

- If eDRX is not used, X = 210 *23*25=262144

- If eDRX is used, X= 212 *23*25=1048576

So, UE_ID occupying the LSBs of 5G-S-TMSI can be illustrated as:

Figure The illustration of UE_ID occupying 5G-S-TMSI

It can be observed that UE_ID needs 18 or 20 bits truncated 5G-S-TMSI for non-eDRX and eDRX respectively.

Observation 1: According to the formulation agreed by RAN2 for UE_ID based LP-WUS subgrouping, the length of UE_ID needs to be 18 or 20 bits for non-eDRX and eDRX respectively.

For RAN paging, the UE_ID should be sent between gNBs or from CU to DU for deriving the PF, PO, UE_ID based PEI subgroup ID as well as LP_WUS subgroup ID. However, present IE (Extended UE Identity Index Value) in Paging message used to deliver UE_ID for deriving PF, PO and PEI subgroup ID is 16 bits. So, present IE is not long enough to deliver the UE_ID which is used for calculating the LP-WUS subgroup ID. The LP-WUS subgroup ID calculated using the present IE would be wrong.

#### 9.3.1.285 Extended UE Identity Index Value This IE is used by the gNB-DU to calculate the Paging Frame and Paging Occasion for eDRX, and the UE_ID based subgroup ID as specified in TS 38.304 [24]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

Based on present formula, new IE needs to be introduced, the exact length can be checked with RAN2, depending on, e.g., whether eDRX is co-exist with LP-WUS.

Observation 2: The present IE in F1AP/XnAP paging message is not long enough for delivering the UE_ID for calculating the LP-WUS subgrouping ID. New IE needs to be introduced.

However, the increased length of UE_ID (i.e., truncated 5G-S-TMSI) may lead to security risk. Note that delivering the entire 5G-S-TMSI over F1/Xn interface may cause some security issues, and that is the reason why the truncated 5G-S-TMSI is used from R15. Thus, if RAN3 assumes increasing the length of truncated 5G-S-TMSI for LP-WUS subgrouping is needed, it should ask SA3 whether there is any concern.

Observation 3: Further increasing the length of UE_ID delivered over F1/Xn interfaces may have security risk.

Proposal 1: RAN3 should send an LS to SA3 to ask if there is any concern on further increasing the length of UE_ID over F1/Xn interfaces for LP-WUS subgrouping.

2.2 Terminology and UE capability missing

In the last RAN2 meeting[2], RAN2 discussed the questions from SA2 in R3-250021(S2-2412876)[3] and provided the feedback in reply LS [4] as below:

- On the terminology and naming of “LP-WUS”: - It’s not necessary that LP-WUS is transmitted with high power. Higher power is just one possible way to ensure the coverage performance. The terminology has been used since R18 Study Item on LP-WUS and the terminology is clear in RAN without any misunderstanding. Thus, RAN2 confirmed that the “LP-WUS” terminology can be used. - On the issue of UE radio capability for paging information, RAN2 had the following agreement: - Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for Release 15 gNB and Release 16 gNB and for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the coming RAN2 meetings based on company contributions.

It can be observed that RAN2 has confirmed the terminology “LP-WUS” is clear and can be used.

On the issue of UE paging capability missing due to heterogeneous network deployment, RAN2 has the agreement that the issue does not exist for network after R17 given the LP-WUS capability is included in R17 RRC container rather than R19 IE in the UE capability RRC message.

For other scenarios, i.e., R15/16 gNBs and R17 (or beyond) gNBs heterogeneous deployment, RAN2 decided to continue the discussion. It is suggested RAN3 continue to wait for RAN2’s progress.

Proposal 2: On the issue of UE paging capability missing due to heterogeneous deployment with R15/16 gNBs and R17 (or beyond) gNBs, RAN3 continue to wait for RAN2’s progress.

2.2 Emergency call back

In RAN2 LS to SA2 on emergency services and eDRX (S2-2500044/R2-2411041)[5], an issue was raised that a UE with emergency PDU session may be released to RRC_INACTIVE state by RAN node with eDRX cycle due to RAN node is not aware of UE’s emergency PDU session, because UE with emergency PDU session in RRC_IDLE or INACTIVE state should not use eDRX.

SA2 discussed this issue during SA2#167 and gave following feedback in reply LS[6] to RAN2:

SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

From this feedback, SA2 confirmed that RAN node can be aware of UE’s emergency PDU session based on specific ARP value, and RAN node may be configured not to release a UE to RRC_IDLE or INACTIVE state when the UE has emergency session. It can be observed that:

- If RAN node is configured to not release a UE with emergency session, the issue of latency caused by LP-WUS/eDRX for emergency call does not exist

- PDU session context with ARP value is saved in RAN node for UE in RRC_INACTIVE or CONNECTED state, so, the RAN node is aware of UE’s emergency PDU session for UEs in RRC_INACTIVE or CONNECTED state. Therefore, RAN node can prevent to release the UE to RRC_INACTIVE state with eDRX. RAN node can disable LP-WUS for UEs in RRC_INACTIVE state with emergency PDU session, and UE can also disable LP-WUS when there is emergency session, that the latency issue caused by LP-WUS to emergency call can be addressed for RAN paging.

- The RAN node is not aware of UE’s emergency PDU session when UE is in RRC_IDLE state, so, the RAN node cannot disable LP-WUS for RRC_IDLE UE for emergency PDU session.

Observation 3: To reduce the paging latency for UE with emergence session, RAN-node can disable LP-WUS for UE in RRC_INACTIVE state, but it cannot disable LP-WUS for UE in RRC_IDLE state.

To resolve the issue for RRC_IDLE UE, one possible solution may be considered that an indicator is introduced to NG paging message to indicate a UE with emergency session. Then RAN-node can be aware of UE with emergency service and disable LP-WUS.

Whether introducing an indicator to NG paging message is needed?

When LP-WUS is disabled, RAN node stops sending LP-WUS and it can send paging from the first available PO, UE also stops monitoring LP-WUS and starts to monitor each PO. However, even if the RAN-node is not aware of UE with emergency session, in which case LP-WUS is not disabled, the RAN node still can send paging in PO(s) from the first available PO for UE. In our opinion, it should be up to RAN node’s implementation to start sending paging before the PO associated with the first LP-WUS. By this implementation, UEs which disable LP-WUS by themselves due to emergency session can receive paging before the PO associated with the first LP-WUS.

On the other hand, emergency session always come with the highest paging priority. So, for optimization, RAN node can start sending paging from the first available PO just for the high paging priority UEs, even if LP-WUS is used.

It can be observed the latency caused by LP-WUS to UE’s emergency session can be addressed by RAN implementation. There is no need to introduce indicator for UE with emergency session to NG paging message.

Observation 4: Introducing an indicator to NG paging message for emergency service is not necessary.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Observation 1: According to the formulation agreed by RAN2 for UE_ID based LP-WUS subgrouping, the length of UE_ID needs to be 18 or 20 bits for non-eDRX and eDRX respectively.

Observation 2: The present IE in F1AP paging message is not long enough for delivering the UE_ID for calculating the LP-WUS subgrouping ID. New IE needs to be introduced.

Observation 3: Further increasing the length of UE_ID delivered over F1/Xn interfaces may have security risk.

Proposal 1: RAN3 should send an LS to SA3 to ask if there is any concern on further increasing the length of UE_ID over F1/Xn interfaces for LP-WUS subgrouping.

Proposal 2: On the issue of UE paging capability missing due to heterogeneous deployment with R15/16 gNBs and R17 (or beyond) gNBs, RAN3 continue to wait for RAN2’s progress.

Observation 3: To reduce the paging latency for UE with emergence session, RAN-node can disable LP-WUS for UE in RRC_INACTIVE state, but it cannot disable LP-WUS for UE in RRC_IDLE state.

Observation 4: Introducing an indicator to NG paging message for emergency service is not necessary.

4 Reference

- RAN3 Chair’s Notes, RAN3#127

- draft_MeetingReport_RAN2#129, MCC

- R3-250021(S2-2412876), LS Reply on LP-WUS subgrouping, SA2, RAN3#127

- R3-251515(R2-2501388), Reply LS on LP-WUS subgrouping, RAN2, RAN3#127bis

- S2-2500044(R2-2411041), LS on emergency services and eDRX, RAN2, SA2#167

- R3-251521(S2-2502427), Reply LS on emergency call back and paging, SA2, RAN3#127bis

Annex: TP for BLCR to TS 38.410

Start of TP

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

CN	Core Network

DRX	Discontinuous Reception

LP-WUS	Low Power Wake Up Signal

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PEI	Paging Early Indication

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

Next Change

# 5 	Functions of the NG interface

## 5.1	General

The following clauses describe the functions supported over the NG interface.

## 5.2	Paging function

The paging function supports the sending of paging requests to the NG-RAN nodes involved in the paging area e.g. the NG-RAN nodes of the TA(s) the UE is registered.

The function also supports CN controlled subgrouping paging for UE Power Saving for PEI and LP-WUS.

End of TP