3GPP TSG-RAN WG3 Meeting #127bis	R3-251650

Wuhan, China, 7 – 11 Avril, 2025

Agenda item:	17.2 and 17.4

Source:	Nokia

Title:	Open points on OD-SSB. Adaptation of common signals/channels

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses:

- open points on OD-SSB (AI 17.2)

- Adaptation of common signals/channels (AI 17.4)

Open points on OD-SSB: Discussions at last meeting were resumed as follows in the summary of discussion document R3-250800:

For the allowing On-demand SSB SCell, we discussed the two way-forward as follows:

- WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general.

- WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”.

But the above is not agreed, there are two concerns raised during the discussion:

- One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation.

- Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation.

To be continued at the next meeting on the above basis.

We here provide our views on the listed options WF1 and WF2, taking into account the captured concerns.

# 2	Open points on OD-SSB

RAN1 has agreed two cases for OD-SSB operation:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Here are some examples of OD-SSB operation:

Figure 1: Examples of OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell) and case #1 (only on-demand SSB is transmitted on the cell)

For case #2, the cell will keep always-on SSB operation in addition to the OD-SSB operation. Case #2 operation therefore doesn’t alter the capability of the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

Observation 1:  OD-SSB case #2 operation doesn’t prevent the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

It therefore seems clear that there is no need for the gNB-CU to be involved in activation and deactivation of OD-SSB operation in case #2.

Proposal 1: For OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell), it is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. gNB-CU assistance is not needed.

The second point to very is then whether the gNB-DU needs the “CU to DU” assistance in order to switch from regular SSB transmission to OD-SSB SCell operation case #1. We first observe that such transmission can’t happen while there are UEs camping on the cell. This means that the cell has to be barred. Similary, the transition can’t happen while the cell is serving UEs as PCell or PSCell. However, such transition can happen while the cell is serving UEs as SCell, because the OD-SSB pattern can be provided both at the time of SCell addition and at any time during Scell operation.

We therefore conclude that the gNB-DU has sufficient information to determine whether it can switch from always-on SSB operation to OD-SSB operation case #1 because:

- The DU owns the MIB, so it will know whether the cell is barred (or e.g. DU-OAM could take the decision to bar the cell, but there is no CU involvement)

- The DU knows whether the cell serves UEs as PCell / PSCell

Proposal 2: For OD-SSB operation case #1 (no always-on SSB on the cell), the gNB-DU has sufficient information to determine whether it can switch from regular SSB transmission to OD-SSB SCell operation. Additional gNB-CU assistance is therefore not needed.

# 3	Adaptation of common signals/channels

## 3.1	Adaptation of SSB in time domain

The Rel-19 outcome from RAN1 relative to adaptation mechanism(s) of SSB in time-domain is as follows:

•	For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

•	Adaptation for SSB that is not CD-SSB is not supported (A2)

•	Adaptation for SSB not on sync raster is not supported (A3)

•	For Rel-19 NES-capable UE’s SCell, adaptation of SSB configured for the SCell is supported for the following cases

•	Adaptation for SSB that is not CD-SSB on sync raster (B2’)

•	Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Also, for SCell, adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec has been discussed in RAN1 but is not supported.

Based on this, we believe there is no RAN3 impact on adaptation of SSB in time domain in Rel-19.

Proposal 2: RAN3 identifies no impact on RAN3 specification coming from adaptation of SSB in time domain in Rel-19.

## 3.2	Adaptation of PRACH in time domain

In previous RAN1 meetings, a list of candidate mechanisms for PRACH adaptation in time domain was discussed. The agreed mechanism is based on additional PRACH resources configured and activated for NES-capable UEs only when needed, hence allowing the network to enter longer sleeping periods in low load periods.

Agreement from RAN1#116 (Feb. 2024):

For adaptation of PRACH in time-domain, support at least the following:

-	Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

Observation 2:

For PRACH adaptation, two types of PRACH resources will be defined in a cell:

•	Legacy (Rel-15) PRACH resources (minimum set of PRACH resources) – always activated

•	Additional PRACH resources (new in Rel-19 for NES-capable UEs) – activated dynamically

In subsequent RAN1 meetings, the mechanism (with the associated details) to support the additional PRACH resources for NES-capable UEs has been discussed. We assume that from configuration perspective the OAM will configure the new additional PRACH resources in the gNB-DU.

Proposal 3: OAM configures the new additional PRACH resources to the gNB-DU

We also observe that RAN1 has designed the additional PRACH resources to be activated or deactivated in a dynamic manner. RAN3 needs to determine which entity that decides this activation and deactivation. From our side we believe this can be done autonomously by the gNB-DU.

Proposal 4: Activation and deactivation of additional PRACH resources to be decided autonomously by the gNB-DU.

RACH optimization was introduced in Rel-16, based on Rel-15 PRACH resource definition. RAN3 is tasked by the Rel-19 NES WID [1] to enable NG-RAN support of the new Rel-19 PRACH resources. Support of RACH optimization is part of the NG-RAN functionality, and it therefore seems that RAN3 within the present work item should evaluate whether there is impact on the RACH optimization feature and determine enhancements needed, if any.

At the present stage, we observe that in the legacy case the gNB-CU has the full view of the PRACH resources configured in the gNB-DU, and these resources are considered to be statically allocated and activated. Information about these resources may today also be transferred over F1 to gNB-DUs serving neighbour cells, and transferred over Xn to neighbour gNBs. But, due to dynamic activation and deactivation of the new additional PRACH resources defined in Rel-19, the network entities in charge of RACH optimization (gNB-CU, gNB-DU) may, based on the existing information exchange, have incomplete information available about the PRACH resources actually used in different cells.

We therefore propose that RAN3 further works on the analysis of impact of Rel-19 additional PRACH resources on the legacy RACH optimization feature.

Proposal 5: RAN3 to further work on the analysis of impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

## 3.3	Adaptation of paging occasions

RAN1 and RAN2 have made the following key agreements on adaptation of paging occasions:

- The solution option b) – where the paging frame interval is extended – has been agreed as the baseline solution to be worked upon

- RAN2 to aim for signaling overhead minimization

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration

- Legacy UE is not barred

The agreed option b) can be illustrated as in Figure 1:

Figure 1: Illustration of adaptation of paging occasions based on extension of the paging frame interval as per RAN1’s agreement.

As can be observed, this solution targets to increase energy savings by extending the paging time interval, thereby allowing for paging resource configuration over a longer period of time. While this is desirable from the energy savings perspective, it also has the effect to reduce the resources allocated for paging. In a legacy network, paging resources will be dimensioned in order to reasonably support periods of high load. If for some reason the paging load becomes particularly high in a given cell or beam for a given Paging Occasion (PO), some of the paging messages will not be transmitted in that PO and in case the UE is located in that cell or beam the network will have to escalate the paging. However, although paging escalation is based on implementation dependent strategies, at one point the mechanism requires extension of the paging to wider areas (additional cells/nodes) so that the full registration area of the UE can eventually be paged. Paging escalation is therefore resource consuming and should be avoided as far as possible, and this aspect is therefore given much attention in deployed networks.

However, the need for network energy saving is equally important, and the reduction of POs based on  extension of the paging frame interval as per RAN1’s agreement allows to increase sleep opportunities for the transmitter (due to reduced number of POs), as well as for the receiver due to reduced number of PRACH resources as illustrated in Figure 1. It can be noted that as per RAN2’s agreement “Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration”, and an energy saving action based on paging adaptation will therefore unavoidably create similar risk for paging overload (and hence skipped paging messages) in periods of low paging load as the risk currently existing in high-load periods in legacy networks.

RAN3 will therefore first need to determine the entity to be in charge of the decision of activation and deactivation of adaptation of the POs for the purpose of NES (paging energy saving). In non-split architecture this could be the OAM or the gNB. We here assume that the gNB will need to be able to at least take decision of deactivation of paging energy saving in order to cope with situation of paging overload as fast as possible. And in order to optimize opportunities for micro-sleep modes, the gNB then also needs to take the decision of activation of paging energy saving. Cell level granularity seems to us to be the good granularity for this decision, hence enabling same paging capacity in all beams within the cell which is needed e.g. for escalated paging. We here derive the following principles for paging energy saving:

Proposal 6: Paging energy saving is done on a per cell granularity

Proposal 7: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB takes the decision to enter paging energy saving mode

- The gNB takes the decision to resume regular paging operation (exit paging energy saving mode)

In split architecture, the gNB-DU will be the entity that can determine potential energy saving gains from paging energy saving. Both the gNB-CU and the gNB-DU know the current paging load. The gNB-CU has a better knowledge to determine upcoming paging load based on information about UEs in RRC_Inactive for which it is anchor gNB, and RAN paging represents a significant part of the paging load in deployed networks. Of course, still the gNB-CU will just transparently forward Paging requests for CN paging and RAN paging requests received over Xn. Also, entering/leaving paging energy saving mode will not be instantaneous but rely on the delay for update of SIB1. Based on this it seems that the gNB-DU could take the decision to enter/exit paging energy saving mode, but the gNB-CU should be aware of this mode in order to take the energy saving mode into account in its paging strategy and paging escalation strategy for RAN paging. It therefore seems the following principles can be taken as base-line for paging energy saving in split architecture:

Proposal 8: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB-DU takes the decision to enter paging energy saving mode and informs the gNB-CU.

- The gNB-DU takes the decision to resume regular paging operation (exit paging energy saving mode) and informs the gNB-CU.

On top of this, in order to enable more precise handling of gNB-CU decided paging escalation for RAN paging, the gNB-DU could inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

Proposal 9: RAN3 to discuss whether the gNB-DU should inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

# 4	Conclusion

We have made the following observations and proposals:

For OD-SSB:

Observation 1:  OD-SSB case #2 operation doesn’t prevent the cell to operate as a suitable or acceptable cell for idle-mode UEs, or to serve UEs as PCell or PSCell.

Proposal 1: For OD-SSB operation case #2 (always-on SSB is periodically transmitted on the cell), it is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. gNB-CU assistance is not needed.

Proposal 2: For OD-SSB operation case #1 (no always-on SSB on the cell), the gNB-DU has sufficient information to determine whether it can switch from regular SSB transmission to OD-SSB SCell operation. Additional gNB-CU assistance is therefore not needed.

For adaptation of common signals/channels:

Observation 1: Use of RFSP index as alternative to an explicit Energy Saving Indicator will not provide sufficient implementation freedom for energy saving techniques in the NG-RAN.

Proposal 1: RAN3 sees that the use of S-NSSAI is sufficient and no new indicator is needed over NGAP, i.e. the subscribed slices behaviour will determine the RAN behaviour also for energy.

Proposal 2: RAN3 identifies no impact on RAN3 specification coming from adaptation of SSB in time domain in Rel-19.

Observation-2: For PRACH adaptation, two types of PRACH resources will be defined in a cell:

•	Legacy (Rel-15) PRACH resources (minimum set of PRACH resources) – always activated

•	Additional PRACH resources (new in Rel-19 for NES-capable UEs) – activated dynamically

Proposal 3: OAM configures the new additional PRACH resources to the gNB-DU

Proposal 4: Activation and deactivation of additional PRACH resources to be decided autonomously by the gNB-DU.

Proposal 5: RAN3 to further work on the analysis of impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

Proposal 6: Paging energy saving is done on a per cell granularity

Proposal 7: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB takes the decision to enter paging energy saving mode

- The gNB takes the decision to resume regular paging operation (exit paging energy saving mode)

Proposal 8: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB-DU takes the decision to enter paging energy saving mode and informs the gNB-CU.

- The gNB-DU takes the decision to resume regular paging operation (exit paging energy saving mode) and informs the gNB-CU.

Proposal 9: RAN3 to discuss whether the gNB-DU should inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

# References

[1]		R3-250800, Summary of Offline Discussion on on-demand SSB SCell operation, Huawei (moderator)