3GPP TSG-RAN WG3 Meeting #127bis	R3-251666

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.1

Source:	Nokia

Title:	Emergency Callback and Paging

WID/SID:	NR_newRAT-Core, NR_redcap-Core

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#127 meeting RAN3 received the LS in [1] from RAN2 reporting an issue when eDRX feature is combined with RRC_INACTIVE feature.

The LS states:

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

# 2	Actions

To SA2, CT1 group

ACTION: 	RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

At this RAN3#127bis meeting, the reply from SA2 in [2] has been received:

SA2 analysed the identified issue above and would like to provide the following feedback:

- Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node.

- In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10.

Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

Even though the action is directed to SA2/CT1, this issue seems primarily a RAN3 issue since the gNB is in charge of deciding whether/when to send a UE in RRC INACTIVE state.

This is why this paper investigates this issue from RAN3 standpoint and potential impact to RAN3 specifications.

# 2	Discussion

Even though the action is directed to SA2/CT1, this issue seems primarily a RAN3 issue since the gNB is in charge of deciding whether/when to send a UE in RRC INACTIVE state.

We already commented at last RAN3#127 that the problem should be split in two parts: “Emergency PDU session activated” or “Emergency PDU session deactivated”. The response from SA2 seems to acknowledge this.

In our view we should differentiate two cases:

- Case 1: Emergency call on Emergency PDU session

This case mostly addresses non voice-centric UEs which would not have an IMS PDU session established.

The emergency call takes place on the Emergency PDU session. After the emergency call ends, for the callback, this case is covered by TS 23.503: the SMF starts a timer at the end of the emergency call and the SMF keeps the emergency PDU session setup and active as long as the timer is running i.e. the SMF does not deactivate the PDU session. When the call back takes place the emergency PDU session is still there and the callback takes place over the emergency PDU session.

As long as the timer is running, the emergency PDU session is setup and active with the UE context in the gNB. This means that the gNB is aware of this emergency PDU session through the specific ARP as explained by SA2 and can refrain from using the inactive eDRX until the callback happens or timer expires.

NOTE: it should be noticed that this does not prevent the gNB to send the UE to RRC_IDLE or RRC_INACTIVE.

There is therefore no issue for this case 1 if gNB implementation is to not use inactive eDRX as long as it is aware of the emergency PDU session ongoing with the ARP. Still, it could be worth adding some specification text to clarify this expected gNB behavior in order to prevent any misbehaving.

Proposal 1: for case 1, specify that gNB shall not use inactive eDRX as long as an emergency PDU session is ongoing.

- Case 2: Emergency call on IMS PDU session

A different case is when the emergency call takes place over the normal IMS PDU session.

Typically, Voice centric UEs have a permanent IMS PDU session setup. The emergency call then takes place over this “normal” PDU session. For the callback over this “normal” PDU session, the problem can exist that gNB has sent the UE to inactive eDRX in-between because the solution of case 1 based on ARP does not work anymore.

RAN3 therefore needs to consider this case 2 and take a solution. In general voice centric UEs don’t need eDRX or long eDRX. 5GC is aware if there is an IMS PDU session and a reasonable solution can be in this case that AMF avoids sending the eDRX configuration to NG-RAN or sends only the short eDRX configuration (e.g. up to 10.24 seconds).

Here the solution can be in the AMF. Again, it seems preferable to specify the expected AMF behavior for this case in order to prevent any issue. AMF behavior would be to be specified in SA2 specifications.

Proposal 2: for case 2, specify that AMF shall not send the long eDRX configuration to gNB. We assume that AMF behavior would be to be specified in SA2 specifications.

# 3	Conclusion and Proposals

This paper has explained why the issue brought up by RAN2 of disabling inactive eDRX in some specific scenario involves RAN3. The paper has further analysed SA2 answer and proposed two different solutions:

Proposal 1: for case 1 (emergency call on emergency PDU session), specify that gNB shall not use inactive eDRX as long as an emergency PDU session is ongoing.

Proposal 2: for case 2 (emergency call on IMS PDU session), specify that AMF shall not send the long eDRX configuration to gNB. We assume that AMF behavior would be to be specified in SA2 specifications.

Proposal 3: agree the corresponding response LS in [3] to inform SA2/RAN2 and agree the draft CR in [4], [5] for TS 38.300.

# 4	References

- R3-250006, LS on Emergency Services and eDRX

- R3-251521, Reply LS on Emergency Callback and Paging (S2-2502427)

- R3-251667, Response LS on Emergency Callback and Paging

- R3-251668, Correction of emergency call and eDRX, draft CR R17

- R3-251669, Correction of emergency call and eDRX, draft CR R18