3GPP TSG-RAN WG3 #127bis	R3-251671

Wuhan, China, 7th – 11th April 2025

Agenda item:		13.2 Support of inter-CU LTM

Title:		discussion Rel-19 inter-CU LTM in DC scenario

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Last RAN3#127 meeting the inter-CU LTM with DC configuration was discussed and reached a status that a baseline CR to 37.340 (R3-250931) was endorsed , and TP to 38.423 (R3-250935) was endorsed.

This contribution we continue to discuss the remaining issue.

RAN3#127: ….. RAN3 agree the following scenarios to support LTM with NR-DC: 1.SN initiated inter-CU SCG LTM without MCG changes (high priority) 2.Inter-CU MCG LTM without SN release 3.Inter-CU MCG LTM with SN release 4.Inter-CU MCG LTM with SN addition …. For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN. The MN requests each candidate SN to allocate resources for the UE via SN Addition request message. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message. The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed. Continue to work on the solutions comparison and details…

2. Discussion

2.1	To avoid inter-CU MCG LTM and inter-CU SCG LTM be configured at the same time

RAN3 has confirmed that ““Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time”.”

For the Inter-CU LTM with DC configuration, to avoid both MCG and SCG are configured at the same time, the MN and SN need to coordinate before initiating the inter-CU MCG LTM and inter-CU-SCG LTM respectively.

One principle to draw is that, which node will be the canter control of such situation. It has been more or less understood that the MN is the node who will control for the dual connectivity configure to the UE, e.g. whether to add SCG for the UE or not. Therefore to have MN to be the centre control, it would natural to do like:

- In case of NR-DC, for the inter-CU MCG LTM, MN indicate/Notify to the SN,

- In case of NR-DC, for the inter-CU SCG LTM, SN send a request to MN which may be accepted or rejected by the MN.

Proposal 1: To avoid inter-CU MCG LTM and inter-CU SCG LTM be configured at the same time, the MN is the centre control / decision so

- for the inter-CU SCG LTM the SN need to request to the MN if can initiating inter-CU SCG LTM

- for the inter-CU MCG LTM, the MN indicate/Notify to the SN;

- in case of crossing, the inter-CU MCG LTM take the precedence.

2.2	Early Sync procedures

For the transferring of TA value from target SN via MN to source SN, it can simply reuse the XnAP: TA Information Transfer procedure that will be introduced for inter-CU LTM. therefore the Editor Note after step 14b in the baseline CR for 37.340 can be removed.

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer procedure.

Proposal 2: Since the to be introduced in XnAP: TA Information Transfer procedure can be reused for inter-CU SCG LTM to transfer the TA values from target SN via MN to source SN, can remove the Editor’s Note: Detail for TA information transfer procedure is up to RAN3 discussion.

3. Conclusion and proposals

Proposal 1: To avoid inter-CU MCG LTM and inter-CU SCG LTM be configured at the same time, the MN is the centre control / decision so

- for the inter-CU SCG LTM the SN need to request to the MN if can initiating inter-CU SCG LTM

- for the inter-CU MCG LTM, the MN indicate/Notify to the SN;

- in case of crossing, the inter-CU MCG LTM take the precedence.

Proposal 2: Since the to be introduced in XnAP: TA Information Transfer procedure can be reused for inter-CU SCG LTM to transfer the TA values from target SN via MN to source SN, can remove the Editor’s Note: Detail for TA information transfer procedure is up to RAN3 discussion.

# References

[1] RP-250339 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-250807 Summary of offline discussion on MobilityEnh

[3] R3-250049 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

[4] R3-251553 (BL CR to 38.420) Support for Inter-CU LTM

[5] R3-250931 => R3-251554 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

[6] R3-250932 => R3-251555 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[7] R3-250933 => R3-251556 (BL CR to 38.300) Inter-CU LTM

[8] R3-250934 => R3-251557 (BL CR to 38.401) on conditional intra-CU LTM

[9] R3-250935 => R3-251558 (BL CR to 38.423) Xn Support for Inter-CU LTM

[10] R3-250936 => R3-251559 (BL CR to 38.473) Inter-CU LTM