3GPP TSG-RAN WG3 Meeting #127-bis	R3-251682

Wuhan, China, 7th – 11st April 2025

Agenda Item:		12.3

Source:		NEC

Title:			Mobility for NR Femto

Document for:		Discussion and decision

# Introduction

In RAN3#127 meeting, functional aspects of NR Femto Gateway and AMF were agreed and captured in [1, 2].

Besides, RAN3 received an LS from SA2 [3] to ask RAN3 for some clarifications on the access control when access to a PLMN and PNI-NPN shared cell.

In this contribution, we further discuss the mobility issues for NR femto, one of which is also related to the above LS.

# Discussion

## 2.1	Mobility optimization

5G Femto is analogous to the LTE concept of HeNB and is deployed to provide NR access at home or at enterprise premises. Considering large scale femto deployment, the same problem for measurement configuration for handover as in HeNB still exists.

While the UE is in RRC_CONNECTED state, the UE performs normal measurement and mobility procedures based on configuration provided by the network. But UEs cannot continuously preform measurements and SI acquisition of a large number of femto cells. In order to allow the UE to perform measurements efficiently and to support proper handover to a NR femto, it is preferred to let the gNB know that the UE is entering or leaving the proximity of one or more CAG member cells and configure UEs in an efficient way.

Unlike defining three access modes for LTE femto, access control in NR femto can be performed as described in TS38.300:

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE: The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

During Xn setup procedure with a NR Femto, NR Femto can exchange its supported CAG ID and PCI of its CAG cells to neighbour node, and the neighbour node stores this information.

After UE connecting to a gNB, based on UE’s Allowed CAG ID List from AMF or neighbor gNB, the gNB can send the PCI of the CAG cells which are in the UE’s Allowed CAG ID List to UE in advance, e.g. together with the other measurement configuration, for CGI acquisition. When UE determines it may be near a CAG member cell, UE performs SI acquisition using autonomous gaps to acquire CGI from the target CAG cell and then sends to gNB in a measurement report. This can reduce the signaling overhead and the handover delay.

As mentioned above, UE can be configured the PCI of the UE’s Allowed CAG cells by RRC signaling, but this needs to involve RAN2 to update the measurement configuration. Therefore, we suggest to LS RAN2 on this issue.

A draft LS to RAN2 capturing the abovementioned issues is included in Annex.

Proposal 1: To agree the LS to RAN2 on PCI configuration for 5G femto in Annex.

In the agreed CR [1], a Note is captured to explain how to use CAG mechanism to realize hybrid access as below:

-	The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

Regarding access control to a PNI-NPN cell and to a shared cell, the legacy way is captured in the SA2’s LS as below:

As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list.

As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration.

In legacy, during the handover procedure, if a UE is tried to handover to a CAG member cell but the membership check is failed, AMF or target NR femto shall reject this handover preparation procedure and inform the source node with a cause. However, if the target cell is a shared cell as the abovementioned new deployment scenario we just agreed, which can accept a UE as either a member or a non-member, so in this case, we think the AMF or the target NR femto can directly decide whether to continue handover preparation procedure with an acknowledgement message instead of a failure message, which can increase the handover success rate and improve UE experience.

Proposal 2: When handover a UE to a PLMN and PNI-NPN shared cell, even if the membership check for the UE is failed due to not-a-member, AMF or target NR femto can accept the UE as a non-member and continue the handover preparation procedure with an acknowledgement message.

Based on proposal 2, a draft LS to SA2 capturing the abovementioned issues is included in Annex.

Proposal 3: To agree the LS to SA2 on NR Femto node shared by PLMN and PNI-NPN in Annex.

## 2.2	DC support

As RAN3 agreed that an NPN cell can be activated in a NR Femto, and considering in legacy DC is supported for PNI-NPN and may involve both PNI-NPN and PLMN cells, NR Femto can also support DC, e.g. a NR Femto cell can be used as a PCell or can be added as a SCell.

Proposal 4: DC can be supported in NR Femto.

# Proposals

In this contribution, we provided considerations on mobility for NR femto and the following proposals were drawn:

Proposal 1: To agree the LS to RAN2 on PCI configuration for NR femto in Annex.

Proposal 2: When handover a UE to a PLMN and PNI-NPN shared cell, even if the membership check for the UE is failed due to not-a-member, AMF or target NR femto can accept the UE as a non-member and continue the handover preparation procedure with an acknowledgement message.

Proposal 3: To agree the LS to SA2 on NR Femto node shared by PLMN and PNI-NPN in Annex.

Proposal 4: DC can be supported in NR Femto.

# Reference

[1] R3-250821, (TP for BL CR for TS 38.300) Functional of AMF for NR Femto

[2] R3-250817, TP for BL CR NR Femto 38.300] Functional aspects of NR Femto Gateway

[3] R3-251524, LS on NR Femto node shared by PLMN and PNI-NPN

# Annex LS to RAN2

3GPP TSG-RAN WG3 Meeting #127-bis	R3-25xxxx

Wuhan, China, 7th – 11st April 2025

Title:	[draft] LS on PCI configuration for 5G femto

Response to:

Release:	Rel-19

Work Item:	NR_WAB_5GFemto-Core

Source:	NEC (to be RAN3)

To:	RAN2

Cc:

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In Rel-19 5G femto study, RAN3 discussed mobility issues for 5G femto.

5G Femto is analogous to the LTE concept of HeNB, that is deployed to provide NR access at home or at enterprise premises, and large scale femto deployment is highly likely. Therefore, PCI confusion issue still exists and UE needs to be configured to perform CGI acquisition during handover preparation.

Since NR Femto can exchange its supported CAG ID and PCI of its CAG cells to neighbour node during Xn setup procedure and the neighbour node can store this information, it is possible to configure the UE the PCI of the CAG cells which are in the UE’s Allowed CAG ID List to UE in advance e.g. together with the other measurement configuration, for CGI acquisition. In this case, when UE determines it may be near a CAG member cell, UE performs SI acquisition using autonomous gaps to acquire CGI from the target CAG cell and then sends to gNB in a measurement report. This can allow the UE to perform measurements efficiently and can reduce the signalling overhead and the handover delay.

Therefore, RAN3 kindly asks RAN2 to take the above into account and update their specifications accordingly.

# 2	Actions

To RAN2:

ACTION: RAN3 kindly asks RAN2 to take the above into account and update their specifications accordingly.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#128		2025-05-19 - 2025-05-23		Malta, MT

RAN3#129		2025-08-25 - 2025-08-29		India, IN

# Annex LS to SA2

3GPP TSG-RAN WG3 Meeting #127-bis	R3-25xxxx

Wuhan, China, 7th – 11st April 2025

Title:	[draft] Reply LS on NR Femto node shared by PLMN and PNI-NPN

Response to:

Release:	Rel-19

Work Item:	5G_Femto, NR_WAB_5GFemto-Core, eNPN

Source:	NEC (to be RAN3)

To:	SA2

Cc:

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA2 for the LS on NR Femto node shared by PLMN and PNI-NPN.

RAN3 discussed the issue, and in order to increase the handover success rate and improve UE experience, RAN3 agreed the following:

When handover a UE to a PLMN and PNI-NPN shared cell, even if the membership check for the UE is failed due to not-a-member, AMF or target NR femto can accept the UE as a non-member and continue the handover preparation procedure with an acknowledgement message.

ACTION: RAN3 kindly asks SA2 to take the above into account.

# 2	Actions

To RAN2:

ACTION: RAN3 kindly asks SA2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#128		2025-05-19 - 2025-05-23		Malta, MT

RAN3#129		2025-08-25 - 2025-08-29		India, IN