3GPP TSG-RAN WG3 #127bis	R3-251685

Wuhan, China, April 7th – 11th, 2025

Agenda item:		14.2

Title:		(TP for TS 38.413) Discussion on NR NTN supporting MBS broadcast service

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the previous meetings, RAN3 made the following progress on NR NTN MBS broadcast service enhancement:

RAN3#126: The current MBS broadcast procedures can be reused for NR NTN without any enhancement. RAN3#127 Encode the Intended MBS service area as OCT STRING which refers to RAN2 spec.

The NR NTN support MBS broadcast will be discussed continuously in this document.

# 2. Discussion

2.1	NTN broadcast service for PWS

During the RAN3#123-bis meeting, the following problem was discussed "What broadcast service to be supported? Only MBS broadcast or MBS broadcast and PWS?" An agreement was reached that:

RAN3 starts work on MBS broadcast service.

The NTN broadcast service for PWS remains an option as it has not been precluded. Given the significant progress on NTN MBS broadcast service in Stage 2 and Stage 3, RAN3 can take a step forward to addressing NTN broadcast for PWS service.

The warning area information for PWS is described by:

- The Warning Area List IE included in the WRITE-REPLACE WARNING REQUEST and PWS CANCEL REQUEST messages.

- The Broadcast Cancelled Area List IE included in the PWS CANCEL RESPONSE message.

- The PWS RESTART INDICATION message.

Additionally, the format of the Warning Area List IE, Broadcast Cancelled Area List IE, and PWS RESTART INDICATION message is similar to the MBS Service Area Information IE. Consequently, the Intended Warning Area List IE could be introduced in the IEs or messages mentioned above, which should be similar to the MBS Intended Service Area List IE as agreed by RAN3 in the last meeting. The corresponding TP is provided in Annex A.

Proposal 1: RAN3 is suggested to initiate discussions on NTN broadcast service for PWS.

Proposal 2: The Warning Area List IE, Broadcast Cancelled Area List IE, and PWS RESTART INDICATION message should adopt the Intended Warning Area List IE, which is similar to the MBS Intended Service Area List IE.

# 3. Conclusion

Proposal 1: RAN3 is suggested to initiate discussions on NTN broadcast service for PWS.

Proposal 2: The Warning Area List IE, Broadcast Cancelled Area List IE, and PWS RESTART INDICATION message should adopt the Intended Warning Area List IE, which is similar to the MBS Intended Service Area List IE.

# References

# Annex A: TP for TS 38.413 (on top of R3-250893)

--------------------------------------------------------------Start of the change-----------------------------------------------------------

#### 9.2.8.5	PWS RESTART INDICATION

This message is sent by the NG-RAN node to inform the AMF that PWS information for some or all cells of the NG-RAN node are available for reloading from the CBC if needed.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| CHOICE Cell List for Restart | M |  |  |  | YES | reject |
| >E-UTRA |  |  |  |  |  |  |
| >>E-UTRA Cell List for Restart |  | 1..<maxnoofCellsinngeNB> |  |  | - |  |
| >>>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >NR |  |  |  |  |  |  |
| >>NR Cell List for Restart |  | 1..<maxnoofCellsingNB> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| TAI List for Restart |  | 1..<maxnoofTAIforRestart> |  |  | YES | reject |
| >TAI | M |  | 9.3.3.11 |  | - |  |
| Intended Warning Area List |  | 0..<maxnoofIntendedAreasforPWS> |  |  |  |  |
| >Intended Service Area Information | M |  | 9.3.1.x |  |  |  |
| Emergency Area ID List for Restart |  | 0..<maxnoofEAIforRestart> |  |  | YES | reject |
| >Emergency Area ID | M |  | 9.3.1.48 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinngeNB | Maximum no. of cells that can be served by an ng-eNB. Value is 256. |
| maxnoofCellsingNB | Maximum no. of cells that can be served by a gNB. Value is 16384. |
| maxnoofTAIforRestart | Maximum no. of TAIs subject for reloading warning message broadcast. Value is 2048. |
| maxnoofEAIforRestart | Maximum no. of Emergency Area IDs subject for reloading warning message broadcast. Value is 256. |
| maxnoofIntendedAreasforPWS | Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536. |

--------------------------------------------------------------Next change-----------------------------------------------------------

#### 9.3.1.37	Warning Area List

This IE indicates the areas where the warning message needs to be broadcast or cancelled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Warning Area | M |  |  |  |
| >E-UTRA Cell IDs |  |  |  |  |
| >>EUTRA CGI List for Warning |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >NR Cell IDs |  |  |  |  |
| >>NR CGI List for Warning |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  |
| >TAIs for Warning |  |  |  |  |
| >>TAI List for Warning |  | 1..<maxnoofTAIforWarning> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >GNSS description for Warning |  |  |  |  |
| >>Intended Warning Area List |  | 0..<maxnoofIntendedAreasforPWS> |  |  |  |
| >>>Intended Service Area Information | M |  | 9.3.1.x |  |  |
| >Emergency Area IDs |  |  |  |  |
| >>Emergency Area ID List |  | 1..<maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.3.1.48 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforWarning | Maximum no. of Cell ID subject for warning message broadcast. Value is 65535. |
| maxnoofTAIforWarning | Maximum no. of TAI subject for warning message broadcast. Value is 65535. |
| maxnoofEmergencyAreaID | Maximum no. of Emergency Area ID subject for warning message broadcast. Value is 65535. |
| maxnoofIntendedAreasforPWS | Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536. |

--------------------------------------------------------------Next change-----------------------------------------------------------

#### 9.3.1.44	Broadcast Cancelled Area List

This IE indicates the areas where broadcast was stopped successfully.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Broadcast Cancelled Area | M |  |  |  |
| >Cell ID Cancelled E-UTRA |  |  |  |  |
| >>Cancelled Cell List |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >>>Number of Broadcasts | M |  | 9.3.1.45 |  |
| >TAI Cancelled E-UTRA |  |  |  |  |
| >>TAI Cancelled |  | 1..<maxnoofTAIforWarning> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >>>Cancelled Cell in TAI List |  | 1..<maxnoofCellinTAI> |  |  |
| >>>>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >>>>Number of Broadcasts | M |  | 9.3.1.45 |  |
| >Emergency Area ID Cancelled E-UTRA |  |  |  |  |
| >>Emergency Area ID Cancelled |  | 1..<maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.3.1.48 |  |
| >>>Cancelled Cell in Emergency Area ID List |  | 1..<maxnoofCellinEAI> |  |  |
| >>>>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >>>>Number of Broadcasts | M |  | 9.3.1.45 |  |
| >Cell ID Cancelled NR |  |  |  |  |
| >>Cancelled Cell List |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>NR-CGI | M |  | 9.3.1.7 |  |
| >>>Number of Broadcasts | M |  | 9.3.1.45 |  |
| >TAI Cancelled NR |  |  |  |  |
| >>TAI Cancelled |  | 1..<maxnoofTAIforWarning> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >>>Cancelled Cell in TAI List |  | 1..<maxnoofCellinTAI> |  |  |
| >>>>NR-CGI | M |  | 9.3.1.7 |  |
| >>>>Number of Broadcasts | M |  | 9.3.1.45 |  |
| >GNSS description for Warning Cancelled NR |  |  |  |  |
| >>Cancelled Intended Warning Area List |  | 0..<maxnoofIntendedAreasforPWS> |  |  |  |
| >>>Intended Service Area Information | M |  | 9.3.1.x |  |  |
| >>>Cancelled Cell in TAI List |  | 1..<maxnoofCellinTAI> |  |  |  |
| >>>>NR-CGI | M |  | 9.3.1.7 |  |  |
| >>>>Number of Broadcasts | M |  | 9.3.1.45 |  |  |
| >Emergency Area ID Cancelled NR |  |  |  |  |
| >>Emergency Area ID Cancelled |  | 1..<maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.3.1.48 |  |
| >>>Cancelled Cell in Emergency Area ID List |  | 1..<maxnoofCellinEAI> |  |  |
| >>>>NR-CGI | M |  | 9.3.1.7 |  |
| >>>>Number of Broadcasts | M |  | 9.3.1.45 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforWarning | Maximum no. of Cell ID subject for warning message broadcast. Value is 65535. |
| maxnoofTAIforWarning | Maximum no. of TAI subject for warning message broadcast. Value is 65535. |
| maxnoofEmergencyAreaID | Maximum no. of Emergency Area ID subject for warning message broadcast. Value is 65535. |
| maxnoofCellinTAI | Maximum no. of Cell ID within a TAI. Value is 65535. |
| maxnoofCellinEAI | Maximum no. of Cell ID within an Emergency Area. Value is 65535. |
| maxnoofIntendedAreasforPWS | Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536. |

--------------------------------------------------------------Next change-----------------------------------------------------------

#### 9.3.1.x	Intended Service Area Information

This IE provides information on the Intended Service Area for MBS Broadcast Service and the PWS service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area Information | M |  | OCTET STRING | Encoded in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331 [18] |

--------------------------------------------------------------End of change-----------------------------------------------------------