3GPP TSG-RAN WG3 #127-bis	R3-251696

Wuhan, China, 7th – 11th April 2025

Agenda Item:	10.3.1

Source:	NEC

Title: 	Discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1	Introduction

A new WID on Data collection for SON/MDT in NR standalone and MR-DC Phase 4 has been approved at the RAN#102 meeting [1]. In the WID, “Support of SON/MDT enhancements for Intra-NTN mobility” is included.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

In this contribution, we describe our view for SON/MDT for NTN.

2	Discussion

2.1 MRO for time-based / location-based CHO

Time-based CHO

In the last meeting, the RLF report for single time-based trigger condition was discussed and following agreement was made.[2]

[Agreement] Existing timers can be reused to identify when RLF happens with 100ms accuracy. FFS whether more accuracy is needed. [Agreement] No need to identify whether RLF happens before or after T1 for time-based CHO.

Now let’s look at current time-based condition. CondeventT1 is specified in TS38.331[3] as time-based CHO.

condEventT1-r17 SEQUENCE { t1-Threshold-r17 INTEGER (0..549755813887), duration-r17 INTEGER (1..6000) }, t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

The T1-Threshold is determined by absolute time (UTC) and is still set with a high granularity (i.e., 10 ms). Therefore, T1 is not a timer dependent on other events in the UE and has the flexibility to be set independently of the time at which T1 is configured. Duration, on the other hand, is defined as the time elapsed from T1.

Observation 1: The T1-Threshold, which is the trigger for time-based CHO, is determined by absolute time and has a high granularity (i.e., 10 ms), making it independent of the time at which the CHO is configured and providing high operational flexibility.

Next, in current spec [3], following time information has been included. These timers can be candidates to reuse for this purpose.

timeConnFailure This field is used to indicate the time elapsed since the last HO execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link or handover) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link or handover) failure.

Here, we explain our view on how the following two cases agreed in RAN3#125 would be calculated if existing timers were reused.

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. - Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For Case1: Too Late CHO Execution

Case 1-1 refers to the case where the RLF occurs before the T1-Threshold, while Case 1-2 refers to the case where the RLF occurs after the T1-Threshold.

For Case2: Too Early CHO Execution

Case 2-1 refers to cases where a hand-over failure occurs due to CHO execution, Case 2-2 refers to the case where the RLF occurs before the T1-Threshold + Duration, and Case 2-3 refers to the case where the RLF occurs after the T1-Threshold + Duration.

In both case1 and 2, the time when the RLF occurred can be determined by matching the timeSinceCHO-Reconfig or timeSinceFailure with the time of the respective network side event. However, it should be noted that the timeSinceCHO-Reconfig has a limited value range, so the period between the reconfiguration and T1-Threshold must be limited to 102.3 s to be able to determine the time. In addition, timeSinceFailure can only be calculated every 1 sec due to the rough granularity of the timer. Therefore, these are not sufficient compared to the value range of T1-Threshold in condEventT1, and the reuse of existing timers limits the flexibility of the SON function.

Observation 2: Existing timers do not have a sufficient range compared to T1-threshold which is the trigger for time-based CHO.

Location-Based CHO

In the last meeting, the RLF report for location-based trigger condition was discussed and following agreements were made.[2]

[Agreement] Revert the WA: There is no need to report any other info in the RLF report for the location based CHO. For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged. - Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred). FFS on option 1 or option 2.

We understand that for earth moving scenario the relative velocity of the satellite is 7.56 km/s, and it causes only rough estimation from timeSinceCHO-Reconfig than the granularity of UE measured distance which mentioned in R3-250300.

As more natural straightforward for above problem, we support to introduce the absolute UTC time of when RLF/HOF happens in the RLF report (option 1 in last meeting) as finer granularity time report. And we prefer to introduce new IE which can be used for multiple purpose, not only for individual purpose.

Observation 3:  Absolute time (UTC time) has finer granularity and larger range. It can be useful for multiple objectives.

Proposal 1: 	RLF report should include Absolute time (based on UTC).

3	Summary

In this discussion paper, SON for NTN enhancements were discussed and the following observations and proposals were made:

Observation 1: 	The T1-Threshold, which is the trigger for time based CHOs, is determined by absolute time and has a high granularity of 10 ms, making it independent of the time at which the CHO is configured and providing high operational flexibility.

Observation 2: 	Existing timers do not have a sufficient duration or granularity compared to T1-threshold which is the trigger for time-based CHO.

Observation 3: 	Absolute time (UTC time) has finer granularity and larger range. It can be useful for multiple objectives.

Proposal 1: 	RLF report should include Absolute time (based on UTC).

4	References

- RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, RAN#102, CMCC

- Draft meeting report RAN3#127, Feb28

- TS38.331, Radio Resource Control (RRC) protocol specification, v18.3.0