3GPP TSG-RAN WG3 Meeting #127-bis                           	  R3-251712

Wuhan, CN, 7th – 11th April, 2025

Source:	CATT, Thales

Title:	Updated work plan for NR_NTN_Ph3

Agenda Item:	14.1

WID/SID:	NR_NTN_Ph3-Core

Document for:	Information

# 1.	Introduction

In this contribution, an updated work plan for the Rel-19 NR-NTN-ph3 work item is proposed on top of previous work plan [1].

# 2.	WID Objective

- (NR_NTN_Ph3-Core) [2]

The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions:

- GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- UEs with GNSS (Global Navigation Satellite Systems) capabilities

- In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO

- Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant

Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion).

The objectives of the work item are the following:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - Link level enhancements are to be specified for the following channels:

    - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

    - PDSCH with Msg4 via PDSCH repetition

    - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

  - System-level enhancements are to be specified for the following:

    - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

      - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used

  - Specify necessary signalling

  - Update RF requirements accordingly, if needed

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

- Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

- For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4]

- For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1]

  - Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

    - Semi-statically configured DL reception collides with semi-statically configured UL transmission

    - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

  - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution.

- Notes for this objective:

  - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 3.	Work Plan

The work plan for RAN3 is provided in Table 1:

Table 1: RAN3 work plan

| Meeting # | TU | Work plan |
| --- | --- | --- |
| RAN3#123bis | 0.5 | - Start the discussion on support of MBS broadcasting: - Potential RAN3 impact on how to provide the target service area over NGAP. - Start the discussion on support of Regenerative payload. - NG-RAN architecture aspects on support of full gNB on board. - Identify, if any NGAP impact to support regenerative payload, e.g. NG maintenance in case of feeder link switch. - Identify, if any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. |
| RAN3#124 | 0.5 | - Further discuss the support of MBS broadcasting: - Potential RAN3 impact on how to provide the target service area over NGAP, taking RAN2 progress into account. - Further discuss the support of Regenerative payload: - Discuss and decide the NG-RAN architecture on support of full gNB on board, endorse the stage 2 BL CR. - Discuss and decide, if any NGAP impact/issue to support regenerative payload, e.g. NG maintenance in case of feeder link switch. Taking the LS from SA2 into account. - Discuss and decide, if any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. |
| RAN3#125 | 0.5 | - Further discuss the support of MBS broadcasting: - The necessary signalling design on how to provide the target service area over NGAP, taking RAN2 progress into account. - Further discuss the support of Regenerative payload: - Further stage 2 work to support full gNB on board, if any. - Discuss and decide the potential enhancements, if any, to NGAP to support regenerative payload. - Discuss and decide the necessary enhancements, if any, to support intra and inter gNB mobility. |
| RAN3#125bis | 0.5 | - Further discuss the support of MBS broadcasting: - The necessary signalling design on how to provide the target service area over NGAP, taking RAN2 agreements into account. - Further discuss the support of Regenerative payload: - Further stage 2 work to support full gNB on board, if any. - Specify the necessary enhancements to NGAP, if any. - Specify the necessary enhancements for intra and inter-gNB mobility, if any. |
| RAN3#126 | 0.5 | - Further discuss the support of MBS broadcasting: - The necessary signalling design on how to provide the target service area over NGAP. - Further discuss the support of Regenerative payload: - The stage 2 work to support full gNB on board. - Specify the necessary enhancements to NGAP to support regenerative payload. - Specify the necessary enhancements for intra and inter-gNB mobility. |
| RAN3#127 | 0.5 | - Further discuss the support of MBS broadcasting: - The necessary signalling design on how to provide the target service area over NGAP. - Further discuss the support of Regenerative payload: - The stage2 work to support full gNB on board. - Specify the necessary enhancements to NGAP to support regenerative payload, if any. - Specify the necessary enhancements for intra and inter-gNB mobility, if any. |
| RAN3#127bis | 0.5 | - Further refine the stage 2/3 BL CR on support of MBS broadcast, if needed. - E.g. Clarification on usage of any combination of the MBS service area (TAI/Cell/Intended Service Area). - Discuss the pending issues on support of Regenerative payload, including: - Whether to support any enhancement to support RRC_INACTIVE. - Whether to support NG Suspend/Resume. - Other issue(s), if needed. |
| RAN3#128 | 0.5 | - No new issues are expected to be discussed. - Conclude on the remaining aspects, if possible. - Further update the stage 2 and stage 3 BL CRs, if needed. |
| RAN3#129 | 0.5 | - Finalize the RAN3 CR/draft CR work: - On provision of the intended service area over NGAP. - The stage 2 and stage 3 signalling enhancements on support of Regenerative payload with full gNB on board. |

# 4.	References

[1] 	R3-243671	Work plan for Rel-19 NR_NTN_Ph3	CATT, Thales

[2]	RP-243300	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3