3GPP TSG-RAN WG3 #127bis                                          R3-251717

Wuhan, CN, 7th – 11th Apr, 2025

Source:	CATT

Title:	Discussion on A-IoT reader selection

Agenda Item:	16.2

Document for:	Decision

Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the Reader selection solution for AIoT Topology 1, corresponding agreements and working assumptions are listed below:

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

AIOTF may be aware of the location of reader via OAM configuration.

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

In this contribution, we will further discuss the Reader selection and signaling impact on support of Ambient IoT. Base on the discussion, we will provide the proposals accordingly.

Discussion

In last RAN3 meeting, A-IoT reader selection in AIOTF and AIoT RAN has been discussed, and there are some open issues need further discussion.

On providing reader related information to AIOTF, there are two solutions:

- Signaling based solution.

- OAM based solution.

OAM based solution has been agreed in last meeting as below:

AIOTF may be aware of the location of reader via OAM configuration.

As for signalling based solution, the FFS is captured below:

FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

For signalling based solution, reader related information is provided from A-IoT RAN to A-IoT CN via NG interface message. Considering the reader(s) in Topology 1 are stationary, OAM based solution is slightly preferred for simplicity. Also, for the location of a reader, or the service area of a reader, it is hard to be exchanged over NGAP interface for all available scenarios. For GNSS area, it cannot to indicate reader deployed vertically. For floor or warehouse, it could only be used in certain scenarios.

Proposal 1: OAM based solution is sufficient on provision of Reader related information to AIoTF, e.g. the Reader ID, Reader Location, Reader service area, etc. Signalling based solution is not needed.

AIOTF selects A-IoT RAN and optionally, a list of readers based on area information from AF. The following open issue is about the selection of A-IoT RAN in AIOTF.

FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

If AIOTF could know the A-IoT RAN related information, the selection of RAN node by AIOTF can be supported. On providing A-IoT RAN related information to AIOTF, there are also two solutions:

- Signaling based solution.

- OAM based solution.

For the same reason as providing reader related information to AIOTF, OAM based solution is also preferred and it is not needed to define A-IoT area To support the selection of A-IoT RAN within AIOTF.

Proposal 2: On provision of A-IoT RAN related information to AIOTF for RAN node selection, OAM based solution is sufficient, not necessary to define and transfer A-IoT RAN service area to AIoTF via signalling.

On reader selection in A-IoT RAN，the FFS is captured below:

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

Similarly, in the inventory request, we do not see any need to define new A-IoT area and transfer the info to AIoT RAN. In the current spec, 5GC usually use TAI to roughly locate a UE for routing, regulation or other purposes. We can assume 5GC has the basic knowledge on the geographical area of each Tracking Area.

Therefore, in the Inventory Request, AIoTF may use TAI list as a kind of inventory area, AIoT RAN selects the proper Readers to perform Inventory service in the AIoT Radio interface.

Proposal 3: AIoTF may use TAI list in Inventory Request as the service area/inventory area.

Then the FFS is about the action of A-IoT RAN when receiving both the area and the reader list in Inventory Request. In our understanding, it is not reasonable to indicate both area and the reader list in Inventory Request. If AIOTF could take the responsible for reader list selection, it is not needed also providing area to let A-IoT RAN perform reader selection again which makes the reader list selection more complicated artificially. For simplicity, we suggest a choice structure may be introduced in Inventory Request message which includes TA list and reader list.

Proposal 4: A choice structure including TAI list and Reader list may be included in Inventory Request message as AIoT service area or inventory area.

Conclusions

In this contribution, we mainly discussed the issue on UE reader selection. Based on the discussion above, the following proposals are provided:

Proposal 1: OAM based solution is sufficient on provision of Reader related information to AIoTF, e.g. the Reader ID, Reader Location, Reader service area, etc. Signalling based solution is not needed.

Proposal 2: On provision of A-IoT RAN related information to AIOTF for RAN node selection, OAM based solution is sufficient, not necessary to define and transfer A-IoT RAN service area to AIoTF via signalling.

Proposal 3: AIoTF may use TAI list in Inventory Request as the service area/inventory area.

Proposal 4: A choice structure including TAI list and Reader list may be included in Inventory Request message as AIoT service area or inventory area.

Reference

- TR 38.769 TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR