3GPP TSG-RAN WG3 Meeting #127bis														R3-251729

Wuhan, China, 7th – 11th April 2025

Agenda item:	14.2

Source:	Nokia, Nokia Shanghai Bell, Qualcomm

Title:	(TP to BL CR for TS 38.300 and TS 38.413) Update on the support of MBS Broadcast Service

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 meeting agreed Stage-2/3 TP to align with the RAN2 IE name. The remaining issue is whether need specification text for identify relevant cell, area overlap, etc. This contribution discusses the remaining issues and propose a way forward.

# 2	Discussion on relevant Uu cells

How to determine the relevant Uu cells

NTN cells are in green, signaled polygon is in blue. It is possible that the MBMS Service Area Information IE only includes MBS Intended Service Area List IE. In last meeting, it was questioned whether the gNB need to decode the Intended Service Area Information IE, in order to determine the Uu cells to start the MBS broadcast. This is up to the gNB’s implementation.

The MBMS Service Area Information IE is defined as a SEQUENCE structure, so it allows any combination of Mapped Cell ID list, TAI List and Intended Service Area List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Intended Service Area List |  | 0..<maxnoofIntendedAreasforMBS> |  |  |
| >Intended Service Area Information | M |  | 9.3.1.x |  |

There is no 1:1 mapping between the Mapped Cell ID (or TAI, or Intended Service Area Information) with the NTN Uu cell. For example,

- The coverage of a NTN Uu cell may corresponds to multiple Mapped Cell ID, or multiple TAI, or multiple Intended Service Area.

- The coverage of a Mapped Cell ID, or TAI, or Intended Service Area may be split in more than one Uu cell.

So the gNB need to determine the relevant Uu cell based on the received MBS Service Area Information IE, including Mapped Cell ID list, or TAI list, or Intended Service Area List.

Proposal 1-1: the gNB determine the Uu cell based on the received Mapped Cell ID list, or TAI list, or Intended Service Area List.

This may require a small update to Stage-2 TP, for example,

The AMF may provide the gNB with the service area information for MBS broadcast service. This information may be signaled as cell ID list, TAI list, and/or MBS intended service area list, as defined in TS 38.413 [26]. Based on this received information, the gNB determines the relevant Uu cell(s), and the gNB provides the intended service area information to the UEs in the relevant cell(s) as defined in TS 38.331 [12].

Proposal 1-2: Update Stage-2 BL CR, as described in Annex A – TP for TS 38.300 BL CR

In current Broadcast Session Modification procedure, the gNB behavior is defined below:

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

The “shall update the MBS service area” is correct for TN. If the MBS Service Area IE is received, it does cause the update of the MBS service area. However, it may not be true in NTN. In NTN, it is possible that the Broadcast Session Modification procedure may be initiated to only update the intended service area information over Uu interface, without affecting the MBS service area/Uu cell(s). So the Stage-3 need to be updated, e.g. to add “if needed”

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area if needed, and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

Proposal 1-3: Update Stage-3 BL CR, as described in Annex B – TP for TS 38.413 BL CR

# 3	Discussion on service area overlap

In last meeting, it was proposed to discuss if there is need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap.

Before we discuss the NTN case, it is better to first check the TN case. The MBMS Service Area Information IE is defined as a SEQUENCE structure. In TN, this IE allows any combination of Cell ID list and TAI List. For example,

- Option 1: The IE only includes a list of NCGIs.

- Option 2: The IE only includes a list of TAIs.

- Option 3: The IE only includes a list of NCGIs and a list of TAIs.

In theory, the overlap issue can also happen in TN. For example, in option 3, a TAI may be used by 10 Uu cells, but the list of NCGIs is only for 4 Uu cells (i.e. the rest 6 Uu cells IDs is not included in this IE). But we think this scenario should not happen. In other words, the whole MBS service area is a combination of areas identified by the list of cell IDs and the TAI list.  In above example, the whole MBS service area is the area identified by the TAI (i.e. the coverage of the 4 Uu cells is a subset of the coverage of the TAI). RAN should not try to determine whether there is an overlapping.

Observation 1: In case MBMS Service Area Information IE includes the list of NCGIs and list of TAIs, the whole MBS service area is a combination of the areas identified by the list of cells, and by the list of TAIs.

In case the target MBS service area is a small or overlap area, it is the responsibility of CN to determine the right format for MBS Service Area Information IE, i.e. only include list of NCGIs, or only include list if TAIs.

Observation 2: it is the responsibility of CN to determine the format of MBS service area, e.g. only using list of NCGIs, or only list of TAIs, or both. There is no need to restrict the indicated MBS service areas no overlapping or partially overlapping.

It is same for NTN. In NTN, the MBS Service Area Information IE supports following options:

- Option 1: The IE only includes a list of NCGIs.

- Option 2: The IE only includes a list of TAIs.

- Option 3: The IE only includes a list of ISAIs.

- Option 4: The IE includes a list of NCGIs and a list of TAIs.

- Option 5: The IE includes a list of NCGIs and a list of ISAIs.

- Option 6: The IE includes a list of TAIs and a list of ISAIs.

- Option 7: The IE includes a list of NCGIs, a list of TAIs and a list of ISAIs.

Same as TN, the whole MBS service area in NTN is all Uu cells identified by the list of NCGIs, or list of TAIs, or list of Intended Service Areas. Similarly, the Intended Service Areas Information sent over Uu is a combination of the areas identified by the list of NCGIs, or list of TAIs, or list of Intended Service Areas. There is no need to restrict the indicated MBS service area overlapping or partial overlapping.

Proposal 2: No need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap.

# 3	Conclusion

In this contribution, we discussed the further update to Stage-2/3 BL CR. Our proposals are:

Proposal 1-1: the gNB determine the Uu cell based on the received Mapped Cell ID list, or TAI list, or Intended Service Area List.

Proposal 1-2: Update Stage-2 BL CR, as described in Annex A – TP for TS 38.300 BL CR

Proposal 1-3: Update Stage-3 BL CR, as described in Annex B – TP for TS 38.413 BL CR

Proposal 2: No need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap.

TP for TS 38.300 can be found in Annex A – TP for TS 38.300 BL CR

TP for TS 38.413 can be found in Annex B – TP for TS 38.413 BL CR

# References

- R3-251561	(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

- R3-251562	(BL CR to 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN

# Annex A – TP for TS 38.300 BL CR

-----------------Start of Change-------------------

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

The AMF may provide the gNB with the service area information for MBS broadcast service. This information may be signaled as cell ID list, TAI list, and/or MBS intended service area list, as defined in TS 38.413 [26]. Based on this received information, the gNB determines the relevant Uu cell(s), and the gNB provides the intended service area information to the UEs in the relevant cell(s) as defined in TS 38.331 [12].

-----------------End of the Change-------------------

# Annex B – TP for TS 38.413 BL CR

-----------------Start of Change-------------------

### 8.17.2	Broadcast Session Modification

#### 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area if needed, and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

-----------------End of the Change-------------------