3GPP TSG-RAN WG3 Meeting #127bis														R3-251730

Wuhan, China, 7th – 11th April 2025

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.413) Discussion on the support of Regenerative payload

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 meeting agreed the Stage-2/3 TP for NG Removal procedure. This contribution analyses whether need the NG Suspend/Resume procedure. This contribution also discusses the FLSO, and the support for RRC INACTIVE UE.

# 2	Management of NG interface

Last RAN3 meeting agreed Stage-2/3 TPs for NG Removal procedure. There is a proposal for NG Suspend/Resume procedure.

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

We think there is no need to introduce NG Suspend/Resume for following reasons:

- The saved NG context is very limited.

Unlike UE suspend that a lot of UE context is stored and resumed for further resume, the NG interface does not have much context. The main saved context for NG interface can be the supported TAIs. However, last RAN3 meeting agreed the AMF can be configured with Supported TAIs of gNB. So there is no saving for the NG Suspend/Resume.

- No benefit for maintaining the TNL connection

In last RAN3 meeting, there was a comment that this can maintain the TNL connection (e.g. maintain the SCTP connection). First, it is not appropriate to use RNL procedure to maintain a TNL connection. Second, the low layer transport connection anyway need to be reestablished. The gNB may get a different TNL address when it revisit the geo-area. So the maintained TNL connection using old TNL address will be useless.

- Heavy cost to the satellite and AMF.

A NTN system based on NGSO may have thousands of satellites or tens of thousands of satellites. For example, Starlink has launched ~5400 satellites, and plan to expand the number of satellites to 12000. Due to earth rotation, every satellite in a NGSO system can visit a specific geographical area but at different time. Similar to terrestrial network, an AMF in NTN serving UEs in specific areas. For example, an AMF deployed in USA may only serve the UEs that are located in the USA. For every satellite to serve USA, it needs to have setup NG with the AMF in the USA. Currently, there is no method to remove an established NG interface. This means an AMF need to maintain a very large number of NG interfaces (including IPSec, TNL connection), e.g. NG interface with every satellite in NGSO system.

On the other hand, at any specific timing duration (e.g. 1-hour), only a limited number of satellites pass USA and need to setup NG (or have “active” NG) with the AMF in USA.

It is very inefficient for AMF to manage a very large number of NG interfaces, while only a very small number of NG interfaces are “active” at any timing duration.

Observation 1-1: considering the potential very large number of satellites in NGSO, it is inefficient for an AMF to manage the NG with all satellites in a NGSO system.

It is similar situation to the gNB. There can be many (tens of AMFs or hundreds of AMFs) around the world, a NGSO satellite need to setup NG with AMFs around the world. At any specific timing duration (e.g. 1-hour), the gNB only need to have one or a few “active” NG interfaces with the AMFs in that specific geographical area. It is also not efficient for a NGSO satellite to maintain NG with an AMF when it does not serve the AMF’s serving area for a long duration (e.g. 12-hour).

Observation 1-2: considering the large number of AMFs around the world, it is inefficient for a gNB to manage the NG with all AMFs around the world.

In a summary, there is no clear benefit to introduce NG suspend/resume procedure.

To reduce the complexity in both gNB and AMF, there is no need to keep the NG when the NG is “not active” for a long period of time. Up to the implementation/configuration, the gNB may decide to remove the NG when it moves out of the serving area of the AMF. By removing the NG, both gNB and AMF does not need to maintain the NGAP application level data, and no need to maintain the NG TNL connection (i.e. IPSec tunnel, SCTP association)

Based on above analysis, we propose to not introduce NG Suspend/Resume, and conclude the discussion on NG management.

Proposal 1: No need to introduce NG Suspend/Resume, and conclude the discussion on NG management.

# 3	Hard Feeder Link Switch Over

In case Hard FLSO is performed, the gNB only connect with the new NTN GW after disconnect from the old NTN GW. This actually includes multiple steps, e.g. disconnect from old NTN GW, adjust antenna to new NTN GW, setup layer 1/2 connection, setup IPSec tunnel, setup SCTP, initiate NGAP RAN Configuration Update procedure via new SCTP, etc. This takes time. During this period, since 5GC does not know the hard FLSO until gNB informs 5GC, the 5GC may continue sending the DL NG-C/U packet to the old NTN GW. The old NTN GW has no way to deliver it to the gNB or forward it to the new NTN GW. This causes packet loss.

Please note the issue does not exist in Transparent architecture. In transparent architecture, the gNB is on the ground, and is aware of the FLSO. The DL data is always sent to the gNB irrespective of the FLSO. For a UE within the gNB’s coverage, the FLSO only affect which NTN GW to be used by the gNB to send the DL data to the satellite. When a hard FLSO is to be performed, the gNB just stop the DL transmission and buffer the DL data. After the hard FLSO is completed, the gNB forward the buffered DL data to the satellite via the new NTN GW. So there is no packet loss during the hard FLSO in Transparent architecture.

Observation 2-1: without knowing the hard FLSO, the 5GC (AMF/UPF) continue sending the DL NG-C/U to the gNB via old NTN GW, and will be discarded at the old NTN GW.

It is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U. This can be supported by the RAN Configuration Update procedure or other NGAP procedure, which can be further discussed.

Observation 2-2: it is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U.

Proposal 2-1: the gNB can initiate RAN Configuration Update procedure to inform the CN to tentatively suspend DL NG transmission before hard FLSO.

There are different views on whether the gNB’s TNL address is changed after the FLSO. In case the TNL address is not changed, the gNB need to inform the CN to resume the DL NG transmission. This can be performed by gNB initiating a RAN Configuration Update procedure to inform 5GC.

Proposal 2-2: the gNB can initiate RAN Configuration Update procedure to inform the CN to resume DL NG transmission after hard FLSO.

Stage-2 TP can be found in ([5]). Stage-3 TP can be found in ([6]).

In case the gNB’s TNL address is changed, after the hard FLSO, gNB need to inform 5GC (AMF/UPF) for the change of its TNL address, so the AMF/UPF can use the new TNL address to send the DL NG-C/U. To change the DL TNL address for NG-U, the gNB can initiate the PDU Session Resource Modify Indication procedure per UE. Accordingly, the AMF need to request SMF then request UPF to modify the DL TNL address of DL F-TEID for every affected UE.

Considering the large coverage area of the NTN cell, the modification procedure of DL TNL address has to be performed for potentially many UEs.  This takes time, and can cause interruption, and signaling storm to gNB/AMF/SMF/UPF.

Observation 2-3: existing Per-UE PDU Session Resource Modify Indication procedure may cause interruption, and signaling storm, e.g. when many UEs are served by the gNB/satellite.

We propose RAN3 to study possible solutions to address this issue. Since it is same gNB before and after the FLSO, the TEID part of the DL F-TEID can remain unchanged. The UPF only need to switch the TNL address of the gNB (i.e. from the old TNL address anchored in old NTN GW to the new TNL address anchored in the new NTN GW). One potential solution is to use a non-UE-associated NGAP procedure to request CN to switch the TNL address of the gNB (i.e. from the old TNL address anchored in old NTN GW to the new TNL address anchored in the new NTN GW). By using this non-UE-associated procedure, the update the DL F-TEID for all UEs can be done in one procedure. This saves time and avoid signaling storm. It is FFS whether this procedure can be based on existing procedure or a new procedure.

It is worthy to note that there is similar issue was discussed and concluded for mobile IAB in Rel-18. After the mobile IAB performed partial migration and received a new TNL address, the mobile IAB need to inform the gNB-CU-UP to use the new TNL address for DL F1-U. RAN3 agreed a new non-UE associated F1AP IAB UP Configuration Update procedure to update the UL/DL TNL address for F1-U tunnels of all connected UEs, instead of using UE-associated procedure to update the UL/DL TNL address for each connected UE. Similar E1AP procedure (i.e. IAB UP TNL Address Update procedure) was also introduced.

Here are the related text from F1AP and E1AP:

F1AP:

- If the DL UP TNL Address to Update List IE is included in the IAB UP CONFIGURATION UPDATE RESPONSE message, the gNB-CU shall replace the old TNL address with the new TNL address for all the maintained DL F1-U GTP tunnels corresponding to the old TNL address.

E1AP:

- Upon reception of the IAB UP TNL ADDRESS UPDATE message, if the DL UP TNL Address to Update List IE is included therein, the gNB-CU-UP shall replace the old TNL Address(es) by the new TNL Address(es) for all the maintained DL F1-U GTP tunnels corresponding to the old TNL Address(es).

Observation 2-4: using a non-UE-associated procedure to modify the transport layer address for all GTP-U tunnels of connected UEs is already introduced in F1AP and E1AP for mobile IAB.

We propose to use similar mechanism in FLSO, to reduce the interruption and avoid signaling storm via the existing per-UE associated procedure. This may be performed by the gNB initiating NGAP RAN Configuration Update procedure with new TNL address, so the CN can update the TNL address of DL F-TEID for all UEs connected with the gNB.

An example call flow is shown as below.

Figure 1: Hard FLSO without changing 5GC

Step 1: before the hard FLSO, the gNB informs the 5GC about the hard FLSO, so the 5GC can suspend DL NG-C/U transmission. It is possible to enhance the RAN Configuration Update procedure.

Step 2: The gNB disconnect from old NTN Gateway, and connect with new NTN Gateway. The gNB receives a new TNL address anchored in the new NTN Gateway.

Step 3: The gNB setup the new TNL by using the new TNL address.

Step 4 and 5: The gNB send a RAN CONFIGURATION UPDATE message over the new TNL. The AMF can know the new TNL association is available in the gNB.

Step 6: The gNB request the 5GC (e.g. AMF, then AMF request SMF, then UPF) to use the new TNL address in DL-F-TEID for all connected UEs.  It is possible to enhance the RAN Configuration Update procedure performed in Step 4 for this purpose.

Step 7 and 8: After the TNL address of the DL F-TEID is updated, the gNB can initiate a RAN Configuration Update procedure to remove the old TNL address.

Proposal 2-3: In case gNB’s TNL address is changed due to FLSO, RAN3 study the solution to reduce interruption and avoid signaling storm for hard FLSO,  and consider using a non-UE associated NGAP procedure (e.g. RAN Configuration Update procedure) to update the DL TNL address for NG-U tunnels of all connected UEs

An example for enhancement to RAN Configuration Update procedure is shown in Annex A – TP for TS 38.413

# 4	INACTIVE UE

In Regenerative architecture, a UE may transition to RRC INACTIVE via Satellite1/gNB1, then resume in another Satellite4/gNB4. The Xn may only exist between the neighboring gNBs/Satellites. There may be no Xn between gNB1 and gNB4. This can cause the failure of Retrieve UE context.

In addition, for MT, the AMF sends the DL data/signaling to last serving gNB. The last serving gNB initiates the XnAP RAN Paging procedure. But in case no Xn, the paging procedure will fail and cause failure to the MT data/signaling.

Observation: In case no Xn between the serving gNB and last serving gNB, both MO and MT will fail for RRC INACTIVE UE.

Following are some options that RAN3 can consider:

- Option 1: before Xn is about to be disconnected, relocate UE context to next satellite.

In this option, the UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

Here is an example based on above solution:

- UE transition to RRC INACTIVE via gNB1. gNB1 only have ISL/Xn with next satellite/gNB (i.e. gNB2) of same orbit plane.

- gNB2 arrives. Before gNB2 move out, gNB1 initiate XnAP RAN paging. UE resume in gNB2. UE context is relocated to gNB2.  gNB2 have ISl/Xn with next satellite/gNB (i.e. gNB3)

- gNB3 arrives. Before gNB3 move out, gNB2 initiate XnAP RAN paging. UE resume in gNB3. UE context is relocated to gNB3.  …

This solution requires the UE need to be waken up in every next satellite, in order to relocate the UE context to the next satellite, in case ISL/Xn only exists between the neighboring satellites/gNBs. This may be performed very frequent, considering a LEO satellite may only serve a specific geo-area for a few minutes. Current TS 38.331 defines

PeriodicRNAU-TimerValue ::= ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

The minimal periodic RNA Update timer is 5-min. This solution consumes more battery.

- Option 2: Pro-actively move UE context to next satellite: Since the satellite’s trajectory is predictable. It can be predicted on which satellite will serve the UE’s geographical area in the future. One possible solution is to proactively push the UE context with updated AS security context to the next satellite via Xn interface at the time of satellite switch.

- Option 3: Relocate the UE context to the AMF on the ground. When the UE resumes, the retrieve UE context procedure is initiated towards the AMF on the ground, without relying on the Xn interface. Also, for a mobile terminated data/signaling, the AMF can initiate RNA-based paging procedure, then the serving gNB retrieve UE context from AMF.

- Option 4: introduces NG-based UE context retrieve procedure. The UE context remains in last serving gNB. When UE resumes, the current serving gNB initiate NG-based UE context retrieve procedure to retrieve the UE context from last serving gNB. For MT data/signaling, the AMF can initiate RNA-based paging procedure.

There are just some possible options. Other solutions are not precluded. It is suggested that RAN3 perform a further study.

Proposal 3: RAN3 study how to support RRC INACTIVE UE in Regenerative architecture.

# 5	Conclusion

In this contribution, we discussed the issues to support regenerative architecture. Our proposals are:

Proposal 1: No need to introduce NG Suspend/Resume, and conclude the discussion on NG management.

For hard FLSO:

Observation 2-1: without knowing the hard FLSO, the 5GC (AMF/UPF) continue sending the DL NG-C/U to the gNB via old NTN GW, and will be discarded at the old NTN GW.

Observation 2-2: it is beneficial to inform the 5GC before the hard FLSO, so the 5GC can suspend the DL NG-C/U.

Proposal 2-1: the gNB can initiate RAN Configuration Update procedure to inform the CN to tentatively suspend DL NG transmission before hard FLSO.

Proposal 2-2: the gNB can initiate RAN Configuration Update procedure to inform the CN to resume DL NG transmission after hard FLSO.

Stage-2 TP can be found in ([5]). Stage-3 TP can be found in ([6]).

Observation 2-3: existing Per-UE PDU Session Resource Modify Indication procedure may cause interruption, and signaling storm, e.g. when many UEs are served by the gNB/satellite.

Observation 2-4: using a non-UE-associated procedure to modify the transport layer address for all GTP-U tunnels of connected UEs is already introduced in F1AP and E1AP for mobile IAB.

Proposal 2-3: In case gNB’s TNL address is changed due to FLSO, RAN3 study the solution to reduce interruption and avoid signaling storm for hard FLSO,  and consider using a non-UE associated NGAP procedure (e.g. RAN Configuration Update procedure) to update the DL TNL address for NG-U tunnels of all connected UEs

An example for enhancement to RAN Configuration Update procedure is shown in Annex A – TP for TS 38.413

Proposal 3: RAN3 study how to support RRC INACTIVE UE in Regenerative architecture.

# References

- R3-247033	(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

- R3-247035	(BL CR to 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN

- R3-247034	(BL CR to 38.410) Introduce NG Removal procedure

- R3-245179, (TP for TS 38.413) Introduce NG Removal procedure

- R3-251784	(TP to BL CR for TS 38.300) Enhancement to support hard FLSO

- R3-251731	(TP to BL CR for TS 38.413) Enhancement to support hard FLSO

# Annex A – TP for TS 38.413

-----------------Start of the Changes-------------------

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the DL UP TNL Address to Update List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, replace the old TNL address with the new TNL address for all the maintained DL NG-U GTP tunnels corresponding to the old TNL address.

-----------------Next Changes-------------------

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| DL UP TNL Address to Update List |  | 0..1 |  |  | YES | ignore |
| >DL UP TNL Address to Update List Item IEs |  | 1.. < maxnoofUPTNLAddresses> |  |  | EACH | ignore |
| >>Old TNL Address | M |  | Transport Layer Address 9.3.2.4 | The old DL UP Transport Layer Address of gNB used for DL NG-U GTP Tunnel before the configuration update. |  |  |
| >>New TNL Address | M |  | Transport Layer Address 9.3.2.4 | The corresponding new DL UP Transport Layer Address that replaces the old one. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofUPTNLAddresses | Maximum no. of TNL addresses for NG-U. Value is FFS. |