3GPP TSG-RAN WG3 Meeting #127-bis	R3-251741

Wuhan, China, April 7th – 11st, 2025

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	(TP for NR_Mob_Ph4 TS 38.473) Discussions on key distribution and NCC delivery for Inter-CU LTM

Document for:	Other

# Introduction

Regarding the Inter-CU LTM security, SA3 reached the following conclusions [1]:

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

There are two key aspects dependent on RAN3:

- CU to DU transfer of NCC (for delivery to the UE via MAC CE)

- Key distribution from the source (or current serving) CU to the target (or another candidate) CU that has been left for RAN3 to decide.

In this contribution, we provide our views and analysis on the above two key aspects.

# Discussion

We first discuss key distribution, then NCC delivery.

## Supporting key distribution

At RAN3-126 (Orlando), [2] discussed how the new serving CU (after path switch) can deliver the newly derived master keys to candidate cells lying in other CU(s) and raised two options:

At the network side, when the UE switches from the source gNB to the target gNB for the first time cell switch, upon path switch completion, the target gNB (gNB2 in Figure 2) receives the new (NH, NCC) pair from the AMF. There are two options for the gNB2 to deliver the new keys to the candidate/target gNBs (gNB3 in Figure 2) for next LTM cell switch: - Option 1: The new serving gNB (gNB2) sends the new keys to all candidate gNBs before next target cell is determined. - Option 2: The new serving gNB (gNB2) sends the new keys to the next target gNB (gNB3) once after next LTM cell switch is executed. The two options are illustrated in the Figure 2 below. Figure 2. Procedures to deliver new keys from the source gNB to the target gNB

Regarding this key distribution, at RAN3-124 (Fukuoka), we provided a comprehensive summary of the  security master key change process in 5G and, based on our analysis, suggested that Option 1 is the right solution approach for Rel-19 Inter-CU LTM [3]:

The right solution approach from NW signalling point of view With respect to the security key handling, the following summarizes the security master key (KNG-RAN) change process in 5G systems [5][6] during handover until Rel-18: - 5G NAS and AS security contexts (e.g. KAMF, KNG-RAN, NH (Next Hop parameter) and NCC (Next Hop Chaining Counter), etc.) described in [5] are per-UE basis. In other words, those KAMF or KNG-RAN (as their name suggests) does not imply that they are some node-specific security key that can be used for multiple UEs. - The security key handling during Xn handover has been that it is the source NG-RAN node who computes a new KNG-RAN* using the PCI and frequency ARFCN-DL/EARFCN-DL of the cell in the target NG-RAN node that the UE will be handover to, either by using its current key (i.e. KNG-RAN) in case of the horizontal key derivation or by using the fresh (i.e. unused) NH (Next Hop parameter) in case of vertical key derivation. - The source NG-RAN node forwards the derived {KNG-RAN*, NCC} to the target, to be used when the UE successfully accesses the target cell. - If a fresh NH is available for a UE, the source NG-RAN node shall use the vertical key derivation for HO. The fresh NH (and associated NCC value) used for vertical key derivation is computed by AMF (by increasing its locally kept NCC value by one) and provided to the target NG-RAN node during path switch procedure after HO is completed. The received {NH, NCC} pair shall be stored for further handovers of the UE and replace the existing pair if any. - A UE does not receive a security key over the air. Instead, a UE computes the next master key to use at the target cell. That is, the UE derives a new KNG-RAN* (or may retain the same KNG-RAN based on indication from NW in case of intra-CU handover) depending on NCC value received in the HO command from the target NG-RAN node (whose value is the same to what the target received from the source). If the received NCC value is the same to the one already stored, the UE performs horizontal key derivation, same as the source NG-RAN node. Otherwise, the UE first synchronizes the locally kept NH parameter iteratively until the NCC values match and then performs the vertical key derivation (same as the source NG-RAN node). - In Rel-16/17, conditional mobility was specified where multiple candidate cells (that may belong to one or more candidate target node(s)) are prepared/pre-configured to the UE for which one of them is selected for access later. For this, the legacy security key mechanism was re-utilized. That is, during the preparation phase, HO signalling is exchanged between the source and a candidate target node individually per each candidate cell basis, where the source derives a new KNG-RAN* for each candidate cell (except ones belonging to the source for which the source may decide to re-use the same master key and indicate the UE to retain the same key), which is provided to each candidate target node accordingly. Their associated NCC value is also sent to each candidate target node and forwarded to the UE, same as legacy. - Moreover, conditional reconfigurations were specified to be released in the UE and NW (except some special failure case which is rare) once successfully executed and one of candidate cells is selected for access, which means that actual serving cell change (and master key change, if applicable) happens only once. Compared to the legacy mobility and conditional mobility mechanisms, Rel-19 Inter-CU LTM mobility shall support the initial and subsequent serving PCell change based on a single mobility configuration (i.e. no RRC reconfiguration between PCell switches). This requires supporting consecutive updates of security master key from one node to another node, and so on, as the UE moves between nodes. Before handover, the UE applies the target cell configuration, and if it contains the security master key update indication, the UE derives the new master key. This means that the UE master key is derived (if configured to do so) before executing handover, so that the UE can be ready to communicate through the target cell safely and immediately after successfully accessed. From this sense, NW should also be ready to use the right key at the new serving gNB (target) immediately whenever serving PCell changes. This is also in-line with the previous release handlings. In the legacy HO, NW decided HO for a specific target cell (only one target gNB to consider) and it is executed immediately once configured to the UE. As a result, it was the source (current serving gNB) who derived the next key to use (using the target cell information) and the source sent it to the target during HO preparation. In case of conditional mobility for which is not executed immediately after configured to the UE, the source doesn’t know which target cell (among candidates) will be accessed by the UE. However, the legacy mechanism was re-utilized in a sense that the source (current serving gNB) derives a new key for each candidate target cell in advance and configures them to the potential target gNB(s), before it configures the UE. In the conditional mobilities, any potential target node was made ready to use the right key regardless of which cell the UE accesses to. Since LTM aims the same principle of preparing/pre-configuring one or more candidate target cell(s), a solution for security maser key update process of Rel-19 Inter-CU LTM is better to embrace the same principle. Proposal 3: Following legacy and previous handover principles, for Inter-CU LTM, a potential target gNB shall be updated with the next master key (derived from the current serving gNB) for the next Inter-CU LTM cell switch, before PCell switch is commanded to the UE by the current serving gNB.

From our analysis, we observe that Option 2 (providing newly derived key only to the target CU after PCell switch has been commanded) is not aligned with our legacy and previous handover principles, and could degrade performance as the target CU may not be ready to communicate with the UE using the new key. Option 2 also complicates the LTM failure re-attempt handling, where the Rel-18 principle is likely to be re-played in Rel-19. If the UE fails to execute the commanded Inter-CU PCell switch and re-attempts with another candidate cell belonging to a different CU, then that CU is unable to communicate with the UE as it never received the new key from the previous serving CU.

Proposal 1: For Inter-CU LTM key distribution, adopt Option 1 (the new serving CU sends the newly derived keys to all candidate CU(s) before next target cell is determined), which aligns with the legacy/previous handover principles and avoid potential side effects.

In stage-3 level, the LTM Configuration Update procedure is the right choice, as the purpose of delivering the newly derived keys to all candidate CU(s) is to support subsequent LTM, and this procedure was already agreed in RAN3-127 (Athens) to use to sync up configurations for subsequent LTM:

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

Moreover, the new serving CU may need to establish a direct UE-association with other C-CU(s), which was also agreed in RAN3-126 (Orlando) to be handled via this same procedure after an Inter-CU LTM switch:

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Proposal 2: Re-use the LTM Configuration Update procedure to distribute the newly derived keys to all the candidate CU(s) for subsequent LTM.

This requires the LTM Configuration Update procedure should NOT be designed on a per-cell basis, since a new key is derived for each candidate cell, and the procedure may need to deliver multiple keys for multiple candidate cells belonging to a single CU. However, the current XnAP BL CR [4] still captured the stage-3 structure as per-cell basis:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | ignore |
| Data Forwarding Info | FFS if List is needed |  | 9.2.1.16 |  | YES | ignore |

#### 9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Given this procedure is now being specified from scratch, there is no reason to stick to a per-cell handling. Syncing up configurations may require updating for multiple candidate cells of a single CU simultaneously by one-shot. One example is the post-LTM-preparation step, where this procedure is utulized to finalize LTM configurations to the UE with each C-CU, based on the common information collected from all the admitted candidate cells. Any update or change to this common information (e.g. modification to a set of candidate cells) may require a holistic update of LTM configurations generated by each CU.

Therefore, the stage-3 message design for LTM Configuration Update is better to support updates related to multiple candidate cells by one shot, including the delivery of newly derived keys for each of them belonging to the recipient CU.

Proposal 3: Correct the stage-3 message design of the LTM Configuration Update procedure to support updates for multiple candidate cells simultaneously, including the delivery of newly derived keys for each candidate cell belonging to the recipient CU.

In [4], the LTM Configuration Update procedure is currently written as being initiated by any CU. However, this should be corrected to ensure that it is triggered by the “source”. This correction aligns with the current 38.300 BL CR [5], for which it is initiated by the serving CU (i.e. S-CU) during LTM preparation. This also aligns with that a CU currently serving the UE is responsible of controlling configuration updates to potential target CUs and handling key distribution in a consistent and successive manner throughout LTM operations.

Proposal 4: Correct the LTM Configuration Update procedure to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during the preparation phase, and also expected to be initiated from the CU who is currently serving the UE (i.e. “source”) during subsequent LTM for syncing up configurations and delivering newly derived keys.

## Supporting NCC delivery to the UE via MAC CE

Transferring a NCC value from CU to DU doesn’t occur during the initial preparation phase, as the NCC value required for initial execution is already included in the initial RRC reconfiguration configuring LTM to the UE. The UE is already configured with the correct NCC value to use for initial execution.

After the first Inter-CU LTM execution, the new serving CU receives a new {NH, NCC} pair from AMF during path switch. The serving CU must then transfer the new NCC value to the serving DU, so that DU can deliver it to the UE via MAC CE for subsequent LTM (as concluded by SA3). At this stage, the UE context has already been established in the serving DU, and the NCC value needs to be stored within this UE context until it is delivered to the UE. Given these, we think that UE CTXT MOD REQ message (although class-1) can be leveraged to transfer the NCC value to the serving DU. There seems no ground breaking reason to seek for class-2 delivery or come up with a completely new procedure for this purpose.

Moreover, no scenario is observed requiring NCC transfer during UE Context Setup, since the NCC transfer to DU is required only after Inter-CU LTM is executed, from the new serving CU to the new serving DU after path switch, i.e. when the UE context has been already established in the DU.

Proposal 5: Enhance UE CTXT MOD REQ > LTM Information Modify IE to carry the NCC value to DU.

Proposal 6: No scenario is observed to transfer NCC to DU during UE Context Setup.

In response to RAN2’s request to avoid RRC reconfiguration between cell switches during subsequent Inter-CU LTM, SA3 reached a compromise allowing NCC to be transmitted to the UE in plaintext via MAC CE, despite security concerns due to its unprotected nature and potential risks and handover failures. SA3 also noted that the security protection for MAC CE will not be considered in this release, although future study may be taken.

Given this situation, we believe that RAN3 should not make DU blindly deliver the NCC value to the UE as soon as it is received from CU. To minimize the exposure of unprotected information over the air, the new NCC value from AMF should be delivered to the UE only when necessary. It may be needed only for Inter-CU PCell switching, which may not occur immediately and depends on L1 measurement results from the UE. Therefore, it will be safer for DU to deliver the NCC value only when strictly necessary, i.e. when the serving DU triggers an “Inter-CU” PCell switching.

To support this, DU needs to know whether the selected target cell requires NCC delivery for updating the UE’s master key. After path switch, if the serving CU decides to use a new {NH, NCC} pair and performs vertical key derivation for candidate cells under other CU(s), then the new NCC value must be delivered to the UE when it is commanded to switch to a PCell under a different CU. The DU needs to be aware of whether the selected target cell involves a CU change requiring the UE to obtain new NCC value.

Note that RAN2 has already agreed to introduce a new Rel-19 ID related to security changes, to control the UE’s master key update process during Inter-CU LTM [6]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

cltm-ExecutionConditions-r19                   SetupRelease {CLTM-ExecutionConditions-r19}           OPTIONAL     -- Need M

Editor’s Note: FFS whether the 8 is the number of LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations

Editor’s Note: How the execution conditions for CLTM are configured is pending the progresses on the event-triggered L1 measurements objective.

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

CLTM-ExecutionConditions-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF ExecutionConditions-r19

ExecutionConditions-r19 ::= SEQUENCE {

ltm-CandidateId-r19                            LTM-CandidateId-r18,

executionCondition-r19                         CHOICE {

l1-Conditions-r19                              LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                              SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NoSecurityChangeID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

#### –	LTM-Config

The IE LTM-Config is used to provide LTM configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

Editor’s Note: FFS is the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG in case it containes both MCG and SCG configurations.

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19       INTEGER (1..maxNrofLTM-Configs-plus1-r18)                           OPTIONAL,   -- Need N

ltm-SK-Counters-r19                         SetupRelease {SK-CounterList-r19}                                   OPTIONAL,   -- Need M

cltm-ServingCellExecutionConditions-r19     SetupRelease {CLTM-ExecutionConditions-r19}                         OPTIONAL    -- Need M

]]

}

SK-CounterList-r19 ::=   SEQUENCE {

sk-CounterList-r19              SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

Editor’s Note: FFS whether the list of sk-counters is part of the LTM-Config or is provided outside (same level as ltm-Config in the RRCReconfiguration message)

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

ltm-SK-Counter This field can only be included within the field ltm-ConfigSCG.

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

The IE VarLTM-ServingCellNoSecurityChange is used to store the serving cell ID based on which the UE determines whether a security change is needed or not upon an LTM cell switch procedure and the list of sk-counter value that can be used when a security key change is performed at the SCG.

VarLTM-ServingCellNoSecurityChange UE variable

-- ASN1START

-- TAG-VARLTM-SEVINGCELLNOSECURITYCHANGE-START

VarLTM-ServingCellNoSecurityChangeID-r19 ::=     SEQUENCE {

ltm-ServingCellNoSecurityChangeID-r19           INTEGER (1..maxNrofLTM-Configs-plus1-r18)                         OPTIONAL,

ltm-SK-Counters-r19                             SetupRelease {SK-CounterList-r19)                                 OPTIONAL

}

-- TAG-VARLTM-SEVINGCELLNOSECURITYCHANGE-STOP

-- ASN1STOP

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

This is similar to the L2 reset configurations in Rel-18 LTM, where NW assigns some IDs grouping candidate cells and configures them to the UE during LTM preparation. When the UE receives a PCell switch command, it compares the ID of the current serving cell with that of the target cell to determine whether to update the master key (if the IDs are equal, no update; if different, update).

To ensure that PCell switches across CUs always trigger a master key update, NW must assign distinct IDs to the groups of candidate cells under different CUs. These same IDs configured to the UE can also be provided to the DU. Following the UE's logic, when the serving DU initiating a PCell switch based on L1 measurements, the DU can compare the IDs of the current serving cell and the selected target cell, and determine whether to deliver the received NCC value to the UE (e.g. if equal, no need to deliver via CSC MAC CE; if different, deliver via CSC MAC CE).

Proposal 7: Given that NCC is delivered in plaintext via MAC CE (unprotected), DU shall deliver the NCC value (if received from CU) to the UE only when strictly necessary.

Proposal 8: Configure the Rel-19 security-related IDs (introduced in RAN2’s RRC running CR) to the DU, so that DU can compare the IDs of the current serving cell and the selected target cell during Inter-CU PCell switching, and determine whether NCC delivery (if received from CU) is required.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Supporting key distribution

Proposal 1: For Inter-CU LTM key distribution, adopt Option 1 (the new serving CU sends the newly derived keys to all candidate CU(s) before next target cell is determined), which aligns with the legacy/previous handover principles and avoid potential side effects.

Proposal 2: Re-use the LTM Configuration Update procedure to distribute the newly derived keys to all the candidate CU(s) for subsequent LTM.

Proposal 3: Correct the stage-3 message design of the LTM Configuration Update procedure to support updates for multiple candidate cells simultaneously, including the delivery of newly derived keys for each candidate cell belonging to the recipient CU.

Proposal 4: Correct the LTM Configuration Update procedure to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during the preparation phase, and also expected to be initiated from the CU who is currently serving the UE (i.e. “source”) during subsequent LTM for syncing up configurations and delivering newly derived keys.

## Supporting NCC delivery to the UE via MAC CE

Proposal 5: Enhance UE CTXT MOD REQ > LTM Information Modify IE to carry the NCC value to DU.

Proposal 6: No scenario is observed to transfer NCC to DU during UE Context Setup.

Proposal 7: Given that NCC is delivered in plaintext via MAC CE (unprotected), DU shall deliver the NCC value (if received from CU) to the UE only when strictly necessary.

Proposal 8: Configure the Rel-19 security-related IDs (introduced in RAN2’s RRC running CR) to the DU, so that DU can compare the IDs of the current serving cell and the selected target cell during Inter-CU PCell switching, and determine whether NCC delivery (if received from CU) is required.

The F1AP TP for supporting NCC delivery to the UE via MAC CE can be found in Section 5.

# Reference

[1] R3-251525/S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3

[2] R3-247261, “(TP for LTM BLCR for TS38.473): Further discussion on inter-CU LTM procedure”, Huawei

[3] R3-243772, “Response to R3-243114”, LG Electronics Inc.

[4] R3-251558, “(BL CR to 38.423) Xn support for inter-CU LTM”, Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno

[5] R3-251556, “(BL CR to 38.300) Support for Inter-CU LTM”, Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

[6] R2-2502534, “Running RRC CR for inter-CU and conditional LTM”, Ericsson

[7] R3-251559, “(BL CR to 38.473) Inter-CU LTM”, Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo

# TP for F1AP BL CR [7]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation) in the UE CONTEXT MODIFICATION RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the Early Sync Candidate Cell Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Candidate Cell Information List IE for some candidate cell, the gNB-DU shall, if supported, take them into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the Early Sync Serving Cell Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Serving Cell Information IE, the gNB-DU shall, if supported, take it into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the LTM CFRA Resource Config List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [16].

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

If the LTM Security Information IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for supporting the UE’s AS security continuation during an inter-CU LTM cell switch and act as specified in TS 38.401 [4].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Direct Path Addition IE is contained in the Path Addition Information IE which is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns the direct path addition for the included SpCell ID IE as specified in TS 38.401 [4] and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using PC5 link and use it as specified in TS 38.401 [4]. If the N3C Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using N3C and use it as specified in TS 38.401 [4].

If the S-NSSAI IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store the corresponding information and replace any existing information.

If the S-CPAC Lower Layer Reference Config Request IE set to "true" is contained in the Conditional Intra-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the S-CPAC Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message for the gNB-CU to generate the S-CPAC reference configuration.

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the S-CPAC Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the S-CPAC candidate configuration is a complete candidate configuration.

If the musim-CandidateBandList IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for temporary capability restriction.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the gNB-CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], , and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >LTM Security Information | O |  | 9.3.1.XX |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.1.XX	LTM Security Information

This IE contains the security related information for LTM candidate cell(s) to support the UE in generating the key material for AS security during an inter-CU LTM cell switch.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [Y] |
| LTM Security Change Configuration List |  | 0..1 |  |  |
| >LTM Security Change Configuration Item |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>LTM Security Change Configuration | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. Details FFS pending RAN2. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |