3GPP TSG-RAN WG3 #127bis                                                 R3-251775

Wuhan, China, 7th – 11th Apr, 2025

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 37.340, 38.423 and 38.300) MRO for CHO with Candidate SCG(s) and S-CPAC

Document for:	other

Introduction

In last RAN3 meeting, the following agreements are captured:

MRO for CHO with Candidate SCG(s)

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

MRO for S-CPAC：

The forwarding mechanism in BLCR for TS37.340 need to be refined.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - MRO for CHO with candidate SCGs

    - Analysis for MCG failure case

In last RAN3 meeting, the case 1a/2a/3a/9a was discussed and Too Late CHO with candidate SCG failure type was included in handover report message. Here we continue discussing other failure types in CHO with candidate SCG and the FFS is captured below:

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

For case 7, the description is captured below:

| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| --- | --- | --- |

For case 7a, only CHO failure is detected, so only source MN is required to perform optimization. In this case, handover report message is not needed to candidate MN for CPC optimization.

Proposal 1: It is proposed to not send handover report message from source MN to candidate MN for case 7a.

For too late CHO with candidate SCG case, following issue was discussed in last meeting:

If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the candidate cell in the NG-RAN node 2.

FFS on the new semantics above.

In candidate MN, the CPC optimization should be performed based on the PCell.

The following IE in HANDOVER REQUEST ACKNOWLEDGE message could describe the reason why optimization on CPC should be performed based on PCell.

| Conditional Handover Information Acknowledge | O |  |  |  |
| --- | --- | --- | --- | --- |
| >Requested Target Cell ID | M | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | PCell |
| >Maximum Number of CHO Preparations | O | 9.2.3.101 |  |  |
| >CHO-CPAC Information | O | 9.2.3.202 |  | Optimization on CPC is based on PCell. |

So, the PCell should be included in handover report message.

While Candidate cell list may include multiply PCells, the target cell CGI cannot be reused. The source MN can select the most suitable PCell and inform to candidate MN

Proposal 2: It is proposed to include a suitable PCell in handover report message and legacy Target cell CGI IE could be reused.

    - Analysis for SCG failure case

In last RAN3 meeting, a new message has been introduced between MNs as below:

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

In candidate MN, the CPC optimization should be performed based on the PCell. Therefore, the suitable PCell can be sent to candidate MN.

Proposal 3: source MN sends suitable PCell to candidate MN in the new message.

During the Preparation of CHO with candidate SCG, the source MN first decides the candidateCellInfoList and provides it in HandoverPreparationInformation message to candidate MN as below:

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=      SEQUENCE {

criticalExtensions                      CHOICE {

c1                                      CHOICE{

handoverPreparationInformation          HandoverPreparationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverPreparationInformation-IEs ::=  SEQUENCE {

ue-CapabilityRAT-List                   UE-CapabilityRAT-ContainerList,

sourceConfig                            AS-Config                                       OPTIONAL, -- Cond HO

rrm-Config                              RRM-Config                                      OPTIONAL,

as-Context                              AS-Context                                      OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                     OPTIONAL

}

……

RRM-Config ::=              SEQUENCE {

ue-InactiveTime             ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}                            OPTIONAL,

candidateCellInfoList       MeasResultList2NR                                           OPTIONAL,

...,

[[

candidateCellInfoListSN-EUTRA      MeasResultServFreqListEUTRA-SCG                      OPTIONAL

]]

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

RRM-Config field descriptions

candidateCellInfoList A list of the best cells on each frequency for which measurement information was available

After receiving the HandoverPreparationInformation message, candidate MN forward candidateCellInfoList IE to candidate SN via SN addition request message which performs the final PSCell selection.

Observation 1: In CHO with candidate SCG, source MN first select candidateCellInfoList, and then candidate SN select the final candidate PSCell list.

There may be a case that the next suitable PSCell is not in the list of candidateCellInfoList provided by source MN due to un-optimized measurement configuration in source MN. For example, source MN not configure the right frequency, or measurement configure (ReportConfig) is too strict, etc.

Proposal 4: There may be a case that the next suitable PSCell is not in the list of candidateCellInfoList provided by source MN and source MN may optimize configuration to include the next suitable PSCell.

After receiving SCGFailureInformation from UE, source MN informs candidate MN to perform MRO on CPC. On the information in the new message, the following FFS is captured:

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

As for the next suitable PSCell selection after SCG failure, it is also selected based on the measurement result in SCGFailureInformation. Logically either source MN or candidate SN could use the measurement result in SCGFailureInformation to select PSCell. But considering the measurement is configured by source MN, it is better for source MN to use the measurement result.

Also, after receives SCGFailureInformation from UE, it is relatively simple for source MN first selecting the next suitable PSCell and if needed (the next suitable PSCell is not in the list of candidate cell provided by source MN), source MN may make optimization. Otherwise, source MN provides the next suitable PSCell  and the SCG failure information to candidate MN.

Proposal 5: It is proposed to send the next suitable PSCell from source MN to candidate MN.

CPC failure type is detected based on the next suitable PSCell and MRO configuration, for example, Tstore_UE_cntxt. Since source MN and candidate MN may have different configuration, if candidate MN detect failure type e.g., too late, different failure type may be detected in source MN e.g., to wrong cell. So, CPC failure type needs to be provided from source MN to candidate MN.

Proposal 6: It is proposed for source MN to send CPC failure type to candidate MN.

    - SCG failure occurs shortly after successful CHO with candidate SCGs

When SCG failure occurs shortly after successful CHO with candidate SCGs, the target MN would receive SCG failure information message from UE.

The related FFS is captured below:

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

Check whether respective MN is only a candidate?

For CHO with candidate SCGs, after receiving SCG failure information message, the target MN would perform MRO analysis. If the failure is caused by othere MNs,a new message should be sent to source MN.

After receiving message from target MN, source MN may also inform other candidate MN to perform optimization on candidate PSCell list selection. Note that the candidate MN may be not the same MN as the original target MN. So, the MRO procedure may be target MN->source MN->candidate MN.

Proposal 7: For SCG failure occurs shortly after successful CHO with candidate SCGs, the MRO procedure may be target MN->source MN->candidate MN. Note that the candidate MN may be not the same MN as target MN. The respective also can be source MN.

    - Stage2 and stage 3 update

According to above analysis, the stage 2 descriptions on TS38.300 and stage 3 TS38.423 are updated in annex.

Proposal 8: It is proposed to discuss TP for TS38.300 /TS38.423 in annex.

  - MRO for Subsequent CPAC

Forwarding mechanism

In last RAN3 meeting, forwarding mechanism has been discussed and the stage2 description still needs further check.

Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

There are some differences due to the following principles applied to subsequent CPAC captured in TS37.340.

The related nodes to perform execution conditions optimization and candidate PSCell list selection are listed in following table.

|  | Node for execution conditions optimization | Node for candidate PSCell list selection |
| --- | --- | --- |
| Initial S-CPAC initiated by MN | MN(initiating node) | MN, target SN |
| Following S-CPAC initiated by MN | Source SN | MN, target SN |
| Initial S-CPAC initiated by SN | Initiating SN | Initiating SN, target SN |
| Following S-CPAC initiated by SN | Source SN | Initiating SN, target SN |

The forwarding mechanism captured in TS37.340 in last meeting is as below:

For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, the MN performs the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation.

According to above table, if the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, the other optimization may include:

- wrong candidate PSCell selection in candidate or target SN;

- wrong execution condition in source SN;

The following modification includes optimization in candidate or target SN and source SN.

There may be two solutions for MN sending SCG Failure Information Report message to the candidate or target SN and source MN.

Solution 1: MN sends SCG Failure Information Report message to the candidate or target SN, and at the same time MN sends SCG Failure Information Report message to source SN.

Solution 2: MN sends SCG Failure Information Report message to the candidate or target SN, if the suitable PSCell is not one of the candidate PSCells selected by the candidate or target SN, the candidate or target SN performs the final MRO related optimisation. Otherwise, the candidate or target SN sends SCG FAILURE TRANSFER message to MN, then MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

We prefer solution 2 to solution 1 in order for avoiding MN always send SCG failure transfer message to source SN even it does not know whether source SN need to be optimized.

Observation 2: the current wording implicitly indication that MN has to always send SCG failure transfer message to source SN even it does not know whether source SN need to be optimized.

So, the following text is proposed:

For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, the MN performs the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to the candidate or target SN, if the suitable PSCell is not one of the candidate PSCells selected by the candidate or target SN, the candidate or target SN performs the final MRO related optimisation. Otherwise, the candidate or target SN sends SCG FAILURE TRANSFER message to MN, then MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

The forwarding mechanism captured in TS37.340 in last meeting is as below:

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis. - For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation. - For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN performs the final MRO related optimisation. Otherwise, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN.

According to above table, if the suitable PSCell is not one of the candidate PSCells provided by the initiating SN at CPAC preparation, the other optimization may include:

- wrong candidate PSCell selection in candidate or target SN;

- wrong execution condition in source SN;

The following modification includes optimization in candidate or target SN and source SN.

- For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN performs the final MRO related optimisation. Otherwise, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN, if the suitable PSCell is not one of the candidate PSCells selected by the candidate or target SN, the candidate or target SN performs the final MRO related optimisation. Otherwise, the candidate or target SN sends SCG FAILURE TRANSFER message to MN, then MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

Proposal 9: It is proposed for RAN3 to discuss above forwarding mechanism in TP for TS37.340.

UHI

There is an issue raised in RAN3 meeting as below:

Support UHI and SCG UE History Information transmission during S-CPAC procedure to avoid the ping-pong or unnecessary PSCell change

To solve the issue on ping-pong or unnecessary PSCell change, there are mainly two steps:

The first step is how to record UHI and detect ping-pong occurrence.

The second step is to optimize configuration to solve the issue.

For the first step, UHI could be updated by MN during S-CPAC procedure. The related text in TS37.340 is as below:

The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

Since MN is aware of the each PSCell selected during S-CPAC procedure according to above text, MN could correlate PCell and PSCell, i.e. update the correlated UHI which includes PCell and PSCell information.

After the analysis of UHI, MN can detect ping-pong issue occurs during S-CPAC procedure. To solve the issue, i.e. the second step, there may be the following solutions:

Solution 1: MN sends UHI to every candidate SN each time UHI is updated.

Solution 2: MN sends UHI to every candidate SN when ping-pong issue is detected by MN.

Solution 3: MN sends UHI to the candidate SN which ping-pong issue is related to when detecting ping-pong issue by MN.

Solution 4: it is not needed to solve ping-pong issue immediately, i.e. the next time configuring S-CPAC, ping-pong issue will be solved according to the previous collected UHI.

Considering above solutions, we think not every candidate SN should be informed because they may be not involved in ping-pong issue, and it is also not needed to inform SN each time UHI is updated because ping-pong issue may not exist. So, solution 1 and 2 is not preferred.

For solution 3, after detecting ping-pong issue, MN informs the related SN about the updated UHI. The candidate cell add/release may be optimized at once which may lead to many interactions between every candidate cells because the change on each candidate cell should be informed to every other candidate cells in S-CPAC procedure.

For solution 4, considering the complexity to perform the modification of S-CPAC configuration, UHI can be used for the next S-CPAC, i.e. not perform optimization for current S-CPAC. Moreover, most of the times MRO is performed based on the statistics, not one time failure. Therefore, it is not needed to perform MRO in a hurry or each time Ping-Pong issue occurs. Comparing with solution 3, solution 4 also has no specification impact. So, solution 4 is preferred.

Proposal 10: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

Information in SCGfailureInformation message

Network could save UE context in MN which may include the initiating node information. But MN may only know the initiating SN node ID and may not know the PSCell ID in the initiating node which is optional present in S-NODE CHANGE REQUIRED message. For the initiating node, it may be not selected as candidate cell and cannot save UE context when UE is not connected to this node. So, it is hard for network to save PSCell information in initiating node in case of SN initiating S-CPAC case.

As we discussed before, the initiating node, specifically, initiating PSCell may be required to perform MRO e.g., candidate PSCell list selection. Therefore, SCGfailureInformation may be enhanced to include source initiating PSCell ID.

Proposal 11: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Conclusions

MRO for CHO with candidate SCGs:

Proposal 1: It is proposed to not send handover report message from source MN to candidate MN for case 7a.

Proposal 2: It is proposed to include a suitable PCell in handover report message and legacy Target cell CGI IE could be reused.

Proposal 3: source MN sends suitable PCell to candidate MN in the new message.

Observation 1: In CHO with candidate SCG, source MN first select candidateCellInfoList, and then candidate SN select the final candidate PSCell list.

Proposal 4: There may be a case that the next suitable PSCell is not in the list of candidateCellInfoList provided by source MN and source MN may optimize configuration to include the next suitable PSCell.

Proposal 5: It is proposed to send the next suitable PSCell from source MN to candidate MN.

Proposal 6: It is proposed for source MN to send CPC failure type to candidate MN.

Proposal 7: For SCG failure occurs shortly after successful CHO with candidate SCGs, the MRO procedure may be target MN->source MN->candidate MN. Note that the candidate MN may be not the same MN as target MN. The respective also can be source MN.

Proposal 8: It is proposed to discuss TP for TS38.300 /TS38.423 in annex.

MRO for Subsequent CPAC:

Observation 2: the current wording implicitly indication that MN has to always send SCG failure transfer message to source SN even it does not know whether source SN need to be optimized.

Proposal 9: It is proposed for RAN3 to discuss above forwarding mechanism in TP for TS37.340.

Proposal 10: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

- Proposal 11: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Reference

[1]

TP on S-CPAC for TS 37.340

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure:

-	Too Late CPC Execution: if CPC/S-CPAC is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

For CPA or MN initiated CPC or initial S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, the MN performs the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to the candidate or target SN, if the suitable PSCell is not one of the candidate PSCells selected by the candidate or target SN, the candidate or target SN performs the final MRO related optimisation. Otherwise, the candidate or target SN sends SCG FAILURE TRANSFER message to MN, then MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis.

- For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.

- For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is not one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN performs the final MRO related optimisation. Otherwise, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN, if the suitable PSCell is not one of the candidate PSCells selected by the candidate or target SN, the candidate or target SN performs the final MRO related optimisation. Otherwise, the candidate or target SN sends SCG FAILURE TRANSFER message to MN, then MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

- TP for TS38.423 on CHO with candidate SCG(s)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

- 8.4.8	Handover Report

- 8.4.8.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

- 8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "Too Late CHO with candidate SCG", then NG-RAN node2 shall deduce that a too late CHO with candidate SCG from a cell of NG-RAN node1 to a cell of NG-RAN node2 occurred. NG-RAN node1 indicates to NG-RAN node2 the target PCell as the next suitable PCell. The detection of Too Late CHO with candidate SCG event is made according to TS 38.300 [9].

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

- 8.4.X	SCG Failure Indication

- 8.4.X.1	General

The purpose of the SCG Failure Indication procedure is to transfer SCG failure information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

- 8.4.X.2	Successful Operation

Figure 8.4.8.2-1: SCG Failure Indication, successful operation

NG-RAN node1 initiates the procedure by sending the SCG Failure Indication message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "Too Late CPC", “Too Early CPC”,”CPC Execution to Wrong Cell”,NG-RAN node1 indicates to NG-RAN node2 the target PCell and PSCell, NG-RAN node2 makes MRO optimization according to TS 38.300 [9]. The detection of Too Late CPC event, Too Early CPC enent and CPC Execution to Wrong Cell are made according to TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …, Too Late CHO with candidate SCG) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the next suitable PCell in the NG-RAN node 2. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |
| Target PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | The next suitable PSCell | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (Too Late CPC, Too Early CPC, CPC Execution to Wrong Cell…) |  | YES | ignore |
| Target PCell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | The suitable PCell | YES | ignore |
| Target PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | The next suitable PSCell | YES | ignore |

Editor’s note: FFS is this message will contain only one SCGFailureInformation message or a list of SCGFailureInformation messages.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 38.300

- 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover in the definition above means Too Late CHO with candidate SCG(s).

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of failure when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a CHO with candidate SCG(s) failure type, the next suitable PCell and PSCell via Handover Report over Xn to the candidate NG-RAN node(s) controlling the candidate PSCell(s) whose mobility configuration caused the failure.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

<<<<<<<<<<<<<<<<<<<< Next OF CHANGES >>>>>>>>>>>>>>>>>>>>

- 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different M-NG-RAN node, the serving M-NG-RAN node transfers the SCGFailureInformation to that M-NG-RAN node.