3GPP RAN WG3 Meeting #127-bis	R3-251828

# Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	(TP for 37.480 and 38.470)AI/ML in split gNBs

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document is mainly a revised one based on R3-250597. The only difference is:

- Reorganise the discussion on UE performance.

- Discussion

  - Exchanging UE performance

The IE “UE Performance” is a group of per-UE metric, and may contain three types of measurement results: Average UE throughput, Average Packet Delay, and Average Packet Loss DL. In DC scenario the data for a UE may be split into more than one node, so the MN has to collect both the UE performance measured in the MN itself and the one measured in the SN in order to generate the final “UE Performance” provided toward e.g. the source node.

The most typical case is on packet delay. Two options were raised:

- CP-centric solution: Both the gNB-DU and the gNB-CU-UP report the delay measurement result directly toward the gNB-CU-CP.

- UP-centric solution: The gNB-DU reports the delay measurement toward the gNB-CU-UP, and gNB-CU-UP reports both parts toward the gNB-CU-CP (either separately or combined).

Last meeting there was two agreements leaning toward the UP-centric solution:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

But we do not consider them suitable.

First, both options require a F1AP class-1 message for data collection initiation:

- In AI/ML the periodicity or time window of measurement should be determined by the requesting node, so as to provide proper training input (e.g. as an input of “reward”).

- In AI/ML it is assumed that every node may reject data collection requests due to various reasons, and upon such reject further action may be taken, e.g. issuing a reject further back (gNB-DU rejects gNB-CU and then gNB-CU rejects the originating gNB), altering the parameter of request or cancelling requests to other nodes which are deemed to be useless.

Figure 1: F1-CP interaction with gNB-DU is anyway needed for the delay metrics.

The most complex part of F1AP impact is on the initiation. Therefore, the CP-centric solution is not significantly more complex on F1AP than the UP centric solution.

Observation 1: The gNB-CP-centric solution is not significantly more complex on F1AP than the gNB-UP centric solution, because both solutions requires enhancement to support data collection initiation.

Second, it is strange to enhance the TS 38.425 DDDS frame for delivering delay results, since there is already delay result IEs in the TS 38.425 Assistance Information frame. And using the TS 38.425 Assistance Information frame to deliver the averaged downlink delay result provide no accurate information on for what packets the downlink delay result is averaged, therefore it is also impossible to align the measurement windows—and such alignment is impossible for the uplink side as well since some part of it is measured by the UE.

Observation 2: It is strange to use the TS 38.425 DDDS frame for delivering delay results.

Observation 3: Using the TS 38.425 Assistance Information frame to deliver the averaged downlink delay result (as in the legacy specification) making it impossible to align the averaging windows (to precisely the same section of continuous packets) for the DL side, totally same as the CP-centric solution

Observation 4: It is anyway impossible to align the averaging windows for UL delay since some part of the UL delay is measured by the UE.

Third, the motivation itself is vague of aligning averaging windows.

It typically costs ~10ms to deliver an XnAP message, but we simply ignored it when designing the XnAP Data Collection messages, i.e. these messages do not use any time stamps but use time lengths relative to the time of sending/receiving the request message. This is because AI/ML mechanics in RAN3 are typically based on predictions for future duration with length in seconds, making such ~10ms difference meaningless.

The averaging windows misalignment is also typically in tens of milliseconds, therefore it is meaningless for AI/ML purpose as well.

Observation 5: It is meaningless for AI/ML purpose to align strictly the averaging windows for delay measurement.

Fourth, the CP-centric solution follows the principle that the measurement result is reported on the same interface where the initiating message is delivered, and the UP-centric solution violates.

Observation 6: The CP-centric solution follows the principle that the measurement result is reported on the same interface where the initiating message is delivered, and the UP-centric solution violates.

Proposal 1: Revise the agreements captured last meeting and adopt the CP-centric solution for average packet delay.

According to Section 4.2.1.2.1 of TS 38.314, the downlink average packet delay is defined as the sum of 4 items:

- Average delay DL air-interface in Section 6.3.1.1.1 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.1.1 of TS 28.552);

- Average delay in RLC sublayer, measured by the gNB-DU as defined in Section 6.3.1.1.2 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.3.3 of TS 28.552);

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average delay DL in CU-UP, measured by the gNB-CU-UP as defined in Section 6.3.1.1.4 of TS 28.558.

The uplink average packet delay is defined as the sum of 5 items:

- UL PDCP packet average delay, measured by the UE as defined in Section 4.3.1.1 of TS 38.314 (it is wrongly defined as measured by the gNB-CU-UP in TS 28.558);

- Average over-the-air interface packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.2 of TS 38.314;

- Average RLC packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.3 of TS 38.314;

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average PDCP re-ordering delay, measured by the gNB-CU-UP as defined in Section 4.2.1.2.4 of TS 38.314.

Nevertheless, it is not necessary to report every metric separately. The delay measured in gNB-DU can be combined into one single metric, and so can the delay measured in gNB-CU-UP.

The case on throughput is not that complex:

Both downlink throughput and uplink throughput are measured by the gNB-DU as defined in Section 6.3.1.4.1 and Section 6.3.1.4.2 of TS 28.558 respectively. There is indeed an “Average DL UE buffered Throughput per DRB” metric defined in Section 5.1.1.3.7 of TS 28.552, but it is claimed that “For very bursty traffic with burst duration within the interval of one DDDS the measured values can be unprecise and cannot be compared across vendors.”

Here is an example why the PDCP-measured one is “unprecise” for “very bursty traffic”

- Assume the DDDS is reported once per second.

- There comes a “very bursty” serial of packets from the DNN in a short period of time, e.g. 50ms. Assume its total volume as 1Mb.

- Due to the limited bandwidth of the radio interface, the maximum available bit rate is only 10Mb/s, lowest among all UP sections. As the result, these data are buffered in the RLC buffer and delivered to the UE in 100ms. The time length when the RLC buffer is not empty is thus 100ms.

- The downlink throughput, as calculated in the DU, is thus 1Mb/100ms = 10Mb/s. It accurately indicates the maximum available bit rate.

- 500ms after the burst arrives, the gNB-DU sends a DDDS frame toward the gNB-CU-UP, causing the PDCP entity to empty its buffer. As the result, the time length when the PDCP buffer is not empty is 500ms. The “PDCP-measured downlink throughput” is thus 1Mb/500ms = 2Mb/s—the number literally makes no sense since the available bit rate of any UP section is far above it.

Observation 7: In the CU/DU-split scenario, the DL throughput measured at the PDCP layer significantly underestimates the actual maximum available data rate for “very bursty” traffics.

The case on packet loss is simple:

- Downlink packet loss is measured by the gNB-DU as defined in Section 4.2.1.5.1 of TS 38.314.

As the summary, there are 5 metrics collected in the gNB-DU whereas 2 metrics collected in the gNB-CU-UP. The remaining one is collected by the UE, which is reported toward the gNB-CU-CP directly and thus has no impact on either E1AP or F1AP.

Proposal 2: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 3: The following 2 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

  - Introducing new metrics

We observe one type of information which is not introduced in the Rel-18 WI but essential in split scenario: the UE traffic volume. It was ever mentioned in the discussion for the Energy Saving use case, as the “Energy Efficiency” is literally the data volume divided by the energy consumption.

Companies found it hard to describe traffic between neighbouring gNBs, but this is not the case within one gNB: the source gNB-CU-CP knows everything from the QoS requirement toward the UE radio status (i.e. RSRP etc), with the only exception is the data rate for each flow.

Therefore we propose enhancing E1AP so that the gNB-CU-UP can provide the data rate for each flow toward the gNB-CU-CP. (The model for energy saving should surely reside in the gNB-CU-CP.)

Proposal 4: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

  - Message to use

For F1AP, we agreed at last meeting that the data collection procedures should be introduced. For E1AP, there are also many metrics needs to be delivered, many of which the same as the metrics included in XnAP Data Collection Update messages. Therefore we propose introducing similar procedures for alignment.

For measurement objects related to one single UE, a measurement ID pair for reference is added into UE-associated messages, such as F1AP UE Context Setup Request messages, E1AP Bearer Context Setup Request messages.

Proposal 5: To introduce new data collection procedures into E1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

  - New node is configured sometime during the feedback collection window

After a handover take place, the target node may need to collect the feedback upon the prior request from the source node. It is possible that a new node is added to serve the UE while it is still right within the feedback collection window:

- The UE further experiences an intra-CU inter-DU handover (or PSCell change), thus a new gNB-DU (or even gNB-CU-UP) starts to serve the UE, and some performance needs to be collected by the new gNB-DU (or gNB-CU-UP).

- The target node further configures an SCG for the UE, and some performance needs to be collected by the SN or secondary gNB-DU.

- The UE further experience an SN change, and some performance needs to be collected by the new SN.

Generally speaking, the existing data collection mechanism can be reused for such case, except the length of collection time duration: the duration is now a reduced value (the original duration minus the time length between the two RRC reconfigurations), rather than the original one.

The source node expects the feedback one “collection time duration” after the handover toward the target node take places (after which the source node may delete the UE context for data collection), so the target node must collect all information and send it back to the source node in time. Therefore, the new serving node must collect information within the reduced duration and send it back instantly.

Considering that the new serving node has no idea of the time length between the two RRC reconfigurations, the new serving node needs to be provided with a “remaining” collection time duration. Considering that the time length between the two RRC reconfigurations varies from one UE to another, we prefer indicating the “remaining” collection time duration within a UE-associated message, i.e. the message containing the Data Collection ID.

Proposal 6: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

- Conclusion

Observation 1: The gNB-CP-centric solution is not significantly more complex on F1AP than the gNB-UP centric solution, because both solutions requires enhancement to support data collection initiation.

Observation 2: It is strange to use the TS 38.425 DDDS frame for delivering delay results.

Observation 3: Using the TS 38.425 Assistance Information frame to deliver the averaged downlink delay result (as in the legacy specification) making it impossible to align the averaging windows (to precisely the same section of continuous packets) for the DL side, totally same as the CP-centric solution

Observation 4: It is anyway impossible to align the averaging windows for UL delay since some part of the UL delay is measured by the UE.

Observation 5: It is meaningless for AI/ML purpose to align strictly the averaging windows for delay measurement.

Observation 6: The CP-centric solution follows the principle that the measurement result is reported on the same interface where the initiating message is delivered, and the UP-centric solution violates.

Proposal 1: Revise the agreements captured last meeting and adopt the CP-centric solution for average packet delay.

Observation 7: In the CU/DU-split scenario, the DL throughput measured at the PDCP layer significantly underestimates the actual maximum available data rate for “very bursty” traffics.

Proposal 2: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 3: The following 2 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

Proposal 4: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

Proposal 5: To introduce new data collection procedures into E1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

Proposal 6: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

We also draft two TPs for introducing AI/ML function and data-collection-related messages into E1 and F1 as in the annexes.

- Reference

[1] R3-241829; Enhancement of UE performance metrics; CATT.

- Text Proposal for TS 37.480

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

# 5	Functions of the E1 interface

## 5.1	General

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 5.1.x	AI/ML support function

This function is used to support AI/ML for NG-RAN, including initiation of data collection and reporting of collected data.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

# 6	Procedures of the E1 interface

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 6.y	AI/ML support procedures

The following procedures are used to initiate data collection and report collected data to support, e.g., AI/ML for NG-RAN:

-	Data Collection Reporting Initiation

-	Data Collection Reporting

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////

- Text Proposal for TS 38.470

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 5.2	F1-C functions

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 5.2.x	AI/ML support function

This function is used to support AI/ML for NG-RAN, including initiation of data collection and reporting of collected data.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 6.1	Control plane procedures

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 6.1.y	AI/ML support procedures

The following procedures are used to initiate data collection and report collected data to support, e.g., AI/ML for NG-RAN:

-	Data Collection Reporting Initiation

-	Data Collection Reporting

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////