3GPP TSG-RAN WG3 Meeting #127bis		R3-251834

Wuhan, China, 7th – 11th Aril 2025

Source:	vivo

Title:	Discussion on XR rate control

Agenda Item:	21.3

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meetings, UL XR rate control has been discussed and an LS [1] was sent to ask RAN3 impact on per QoS or per DRB rate control.

RAN2 has made the following agreement regarding a solution on UL bit rate control - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

A reply LS [2] was sent out in RAN3#127, RAN3 agreed that both solutions have F1 impact.

RAN3 thanks RAN2 for the LS on XR UL bit rate control. RAN3 discussed it and agreed both options have an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available.

In this contribution, we will discuss the detailed impacts on Xn and F1 interface for UL XR rate control. And issues for UL XR rate control in DC scenario.

- Discussion

  - Discussion on F1 impact on XR rate control

In last RAN2 meeting, it was agreed that for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control. In case of CU-DU split architecture, gNB-CU will indicate such information via “QoS flow level QoS information” in F1AP signaling directly in case of per QoS flow level rate control. While for per DRB level rate control, either QoS flow information or mapped DRB information need to be indicated to gNB-DU via F1AP signaling.

In case of one QoS flow to one DRB mapping, both DRB based and QoS flow based rate control could be easily performed. In case of more than one QoS flows to one DRB mapping, per DRB based rate control can not provide proper recommended bit rate to UE for the adjustable QoS flow so that such mechanism may not achieve congestion relief result. gNB-DU can hardly choose proper target bit rate by its implementation, since specific UE behavior can not be defined when a DRB based rate control MAC CE be received. As a result, the rate control result will still be unpredictable.

Observation 1: Per DRB based rate control can hardly achieve expected rate adjustment from gNB perspective.

In case of more than one QoS flows to one DRB mapping, for QoS flow based rate control, gNB-DU could calculate a percentage of the adjustable bit rate for given QoS flow by its implementation. For example, using GBR ratio or another ratio based on implementation. Then send the calculated bit rate to UE via MAC CE and achieve predictable rate adjustment.

Since both solutions have F1AP signaling impact and both are feasible to implement, it is proposed that:

Proposal 1: It is up to RAN2 to decide the XR rate control whether it is per DRB or per QoS flow basis.

Beside the related QoS flow information, other rate control configuration may also need to be included in the F1AP signaling, such as bit rate multiplier or UL query activation. The detailed specification impacts can be finalized after RAN2 conclusion.

  - Discussion on Xn impact on XR rate control

In case of DC configured or handover procedure, the QoS flow information which provided from CN need to be forwarded to S-NG RAN node or target gNB via “QoS flow level QoS parameters” in XnAP messages.

Proposal 2: Include “rate control QoS flow information” in HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE, S-NODE ADDITION REQUEST, S-NODE MODIFICATION REQUEST messages.

  - Discussion on XR rate control in DC configuration

In Rel-18, we have discussed the PSI-based SDU discard activation in case DC configured, and agreed that NG RAN nodes will not perform co-ordination and UE will always follow the latest activation or deactivation command via MAC CE. However, in case of XR rate control, the indication from gNB-DU is used to suggest the maximum bit rate according to related radio from one NG RAN node. UE can not easily perform rate adaptation according to rate control MAC CEs send from two NG RAN node separately. For example, if UE receive XR rate control bit rate from one NG-RAN node, whether it should wait for the bit rate from the other NG RAN node or consider the value as the suggest bit rate in total. As a result, it is suggested M-gNB-DU and S-gNB-DU perform bit rate co-ordination and send the overall value to UE for simplicity. One detailed example would be S-gNB-DU send its suggested bit rate per QoS flow to M-gNB-DU via F1AP and XnAP message. And only M-gNB DU is allowed to send the XR rate control MAC CE to affect UE. Furthermore, user plane GTP-U header could be also used to send such suggested bit rate information.

Proposal 3: In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE. An LS is sent to RAN2 with the conclusion.

- Conclusion

In this contribution, we discuss the RAN3 impacts and open issues of XR rate control. Based on the discussion, we have the following observations and proposals:

Observation 1: Per DRB based rate control can hardly achieve expected rate adjustment result from gNB perspective.

Proposal 1: It is up to RAN2 to decide the XR rate control whether it is per DRB or per QoS flow basis.

Proposal 2: Include “rate control QoS flow information” in HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE, S-NODE ADDITION REQUEST, S-NODE MODIFICATION REQUEST.

Proposal 3: In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE. An LS is sent to RAN2 with the conclusion.

- References

- R3-250012	LS on XR UL bit rate control, RAN2

- R3-250805	Reply LS on XR UL Rate Control, RAN3

- Annex - draft reply LS

Title:	[Draft] LS on XR rate control

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	TBD [to be RAN3]

To:	RAN2

Cc: 	SA2

# Contact Person:

# Name:	              TBD

E-mail Address:	 TBD

# 1. Overall Description:

RAN3 would like to thank RAN2 LS for XR rate control.

RAN3 has discussed the related issues on XR rate control, and concluded the following responses [To be updated based on the discussion conclusion]:

1. Both per QoS flow and per DRB XR rate control are feasible and have F1AP impacts, thus it is up to RAN2 to decide the granularity. 2. In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE.

# 2. Actions:

# To RAN WG 2:

# RAN3 kindly request RAN2 to take the above information into account during the future work, and provide feedback, if any.

# 3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #128		19th May. – 23rd May. 2025		Malta

TSG RAN WG3 Meeting #129		25th Aug. – 29th Aug. 2025		India