3GPP TSG-RAN WG3 Meeting #127bis	R3-251849

Wuhan, China, 7th - 11th April, 2025

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

Title:	F1 impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

The revised Work Item Description (WID) for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on the F1 interface.

In particular, we discuss the main principles of the intra NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering split architecture.

A companion contribution discussing inter NG-RAN node signaling and Xn impact is provided in R3-25xxxx.

# Discussion

## General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

## Previous agreements

In relation to the impact on the F1 interface, at the RAN3#125 meeting, the following was agreed [2]:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU.

Moreover, at the RAN3#126 meeting, the following agreements related to the new procedures were captured [3]:

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

Hence, in split architecture, RAN3 has agreed that the UL WUS configuration is decided by the NES-Cell gNB-DU and sent to the NES-Cell gNB-CU. Furthermore, the existing Served Cell Information IE is enhanced to include the UL WUS configuration; this is a natural choice since this IE already comprises all the configuration information of a cell.

## Further discussion on the F1AP signaling

### Signaling in the NES-Cell gNB

A simple diagram showing the fundamental parts of the previous agreements is seen in Figure 2 below.

Figure 2: Signaling of the UL WUS configuration from NES-Cell gNB-DU to NES-Cell gNB-CU.

The NES-Cell gNB-CU can interpret the reception of the UL WUS configuration as interest of the NES-Cell gNB-DU to operate the NES Cell in OD-SIB1 transmission mode. Alternatively, an explicit indication could be introduced. The gNB-CU receives the UL WUS configuration from gNB-DU and starts coordinating the UL WUS configuration provisioning, e.g., by sending a request to start UL WUS configuration provisioning to a neighboring Cell-A gNB (here gNB2) as discussed in the companion paper.

It has been discussed and reflected in agreements in RAN3 that feedback on UL WUS configuration provisioning is needed over Xn. The same applies to F1 in case of split architecture, i.e., the gNB-DU cannot start OD-SIB1 transmission in the NES Cell before its gNB-CU indicates that it may do so, because the gNB-DU does not know if and when the UL WUS configuration provisioning is ensured. Thus, the gNB-DU cannot autonomously start OD-SIB1 transmission in a NES Cell without risking making the cell inaccessible for UEs. Hence, the gNB-DU requires indication from the gNB-CU.

In split architecture, the NES-Cell gNB-DU does not know if and when the UL WUS configuration provisioning for a NES Cell is ensured, and thus it cannot autonomously start OD-SIB1 transmission in the NES Cell. Instead, it requires assistance information from the NES-Cell gNB-CU.

In split architecture, the NES-Cell gNB-CU should indicate to the NES-Cell gNB-DU whether a NES Cell operation can be changed to OD-SIB1 transmission mode.

This is shown in Figure 3 below. Once the UL WUS configuration provisioning for the NES Cell is ensured, the gNB-CU indicates to the gNB-DU that OD-SIB1 transmission is allowed in the NES Cell.

Figure 3: Indication from NES-Cell CU to NES-Cell DU that OD-SIB1 transmission is possible/permitted in a NES Cell.

We believe that the F1AP gNB-CU Configuration Update procedure can be reused for this purpose, namely the GNB-CU CONFIGURATION UPDATE message may carry an indication, e.g., a list, of cells that can be operated in OD-SIB1 transmission mode. Conceptually, such list closely resembles the Cells Allowed to be Deactivated List IE, introduced in the GNB-CU CONFIGURATION UPDATE message in Rel-18, indicating cells within which the gNB-DU is allowed to deactivate SSB beams for network energy saving purpose. Thus, it seems natural to also reuse the gNB-CU Configuration Update procedure for the purpose described above.

RAN3 to agree to reuse the gNB-CU Configuration Update procedure over F1 for sending from NES-Cell gNB-CU to NES-Cell gNB-DU an indication that certain cell(s) can be operated in OD-SIB1 transmission mode.

While OD-SIB1 transmission is ongoing, the gNB-CU may determine that the UL WUS configuration provisioning for a NES Cell is interrupted, e.g., upon receiving an according indication from a neighboring Cell-A gNB as reflected in existing RAN3 agreements. In that case, the gNB-CU can revoke the permission to operate the NES Cell in OD-SIB1 transmission mode, e.g., by sending to the gNB-DU a modified (in this case reduced) list of cells for which OD-SIB1 transmission is allowed. In other words, the cells for which OD-SIB1 transmission is no longer allowed will not be on the list anymore. This is shown in Figure 4 below. Consequently, the gNB-DU immediately stops OD-SIB1 transmission, if it is ongoing.

Figure 4: Indication from NES-Cell gNB-CU to NES-Cell gNB-DU that OD-SIB1 transmission is not possible (not permitted) in a NES Cell.

Similarly, if the gNB-CU identifies that the current conditions are not suitable for OD-SIB1 transmission, e.g., due to high load of legacy UEs in overlaid cells, it can revoke the permission to operate the NES Cell in OD-SIB1 transmission mode.

Inversely, if the gNB-DU identifies that the current conditions are not suitable for OD-SIB1 transmission, e.g., due to large number of SIB1 transmission requests in a NES Cell, resulting in diminishing or even negative NES gains, it can revoke UL WUS configuration of the NES Cell. Alternatively, it may explicit indicate that it has no (longer) interest in operating the NES Cell in OD-SIB1 transmission mode for the time being. This is shown in Figure 5.

Figure 5: Indication from NES-Cell gNB-DU to NES-Cell gNB-CU that the UL WUS configuration of a NES Cell is invalidated (gNB-DU identifies unsuitable conditions for OD-SIB1 transmission).

It is important for the gNB-CU to know when the gNB-DU has interest to operate a NES Cell in OD-SIB1 transmission mode and when not, because the gNB-CU is coordinating the UL WUS configuration provisioning, regardless of whether the NES Cell and the Cell A(s) are in different gNBs or in the same gNB or even in the same DU, and because unnecessary provisioning of an UL WUS configuration means a waste of radio resources as well as energy.

### Signaling in the Cell-A gNB

It was already agreed in RAN2#127bis that the UL WUS configuration of a NES Cell is provided to UEs via a new SIB in Cell A, as well as in the NES Cell. This new SIB is herein referred to as “SIBx”. The SIBx will often contain UL WUS configurations of NES Cells of neighboring gNB-DUs (of the same or neighboring gNB-CUs). We thus believe it makes most sense that SIBx is a CU-owned SIB, like other SIBs containing neighbor cell information.

Note that, in the case where SIBx of a NES Cell must be updated because the UL WUS configuration of that NES Cell has been changed, the NES-Cell gNB-DU needs to send the updated UL WUS configuration over F1 to the NES-Cell gNB-CU for potential coordination with other nodes. Hence, also in that case, the NES-Cell gNB-CU can prepare the updated SIBx and send it over F1.

We therefore propose that SIBx is a CU-owned SIB.

RAN3 to agree that SIBx (the new SIB containing one or more UL WUS configurations) is a CU-owned SIB.

Moreover, the existing gNB-CU System Information IE, which can be sent from gNB-CU to gNB-DU over F1 as part of the F1 Setup, the gNB-CU Configuration Update, or the gNB-DU Configuration Update procedures, is specified in a way that it can directly be reused upon introduction of a new SIB. If SIBx is a CU-owned SIB and is therefore encoded by the gNB-CU, the introduction of SIBx has no obvious F1 impact.

If SIBx is encoded by the gNB-CU, the F1 impact for the introduction of SIBx is only to include the new SIB.

During the previous RAN3 meeting, several companies argued in favor of the Cell-A gNB-DU having control on the UL WUS configuration provisioning in Cell A. For example, they proposed that the Cell-A gNB-DU could reject the request to provide the UL WUS configuration, or that it could signal a request to stop an ongoing provision. The introduction of these capabilities would require the creation or enhancement of multiple F1 procedures while, simultaneously, no clear benefits were identified for providing such capabilities.

If concerns arise regarding the impact on the resources of the Cell-A gNB-DU, it should be noted that the Cell-A gNB-CU can configure the SIBx to be provided on-demand or with a long periodicity. This approach would minimize any significant impact on the resources of the Cell-A gNB-DU.

In our view, there is no strong motivation to introduce changes in the F1 specification to support a similar functionality for OD-SIB1 as over the Xn interface. RAN3 should strive to implement this feature with as little impact as possible, to ensure that the possible sources of errors are minimal. For this reason, RAN3 should not consider functionality for the Cell-A gNB-DU to have control on the UL WUS configuration provisioning in Cell A.

RAN3 to agree that the UL WUS configuration provision is controlled by Cell A gNB-CU.

As shown in Figure 6 below, when a Cell-A gNB-CU identifies a change in the UL WUS configuration provision demand concerning Cell A, i.e., that a new UL WUS configuration needs to be provided, or that an existing UL WUS configuration does not need to be provided anymore, the Cell-A gNB-CU sends an updated version of SIBx of Cell A to the Cell-A gNB-DU, so that the Cell-A gNB-DU provides all (and only) the required UL WUS configuration(s) to UEs in Cell A.

Figure 6: Signaling of the SIBx for Cell A (comprising one or several UL WUS configurations of overlaid NES Cells) from Cell-A gNB-CU to Cell-A gNB-DU.

The Cell-A gNB-CU may identify a change in the UL WUS configuration provision demand based on external indication, e.g., a UL WUS configuration provision start (or stop) request from a neighboring NES-Cell gNB (here gNB1) as discussed in the companion paper.

If the NES Cell and Cell A are in the same gNB, the identification of a change in the UL WUS configuration provision demand may be based on an indication over F1, e.g., a new UL WUS configuration, from a gNB-DU of that gNB to the gNB-CU.

A TP reflecting the above proposals is provided in Appendix A.

RAN3 to agree on the TP for BL CR for TS 38.473 provided in Appendix A.

# Conclusion

Based on the discussion in the previous sections, we make the following observations:

Observation 1	In split architecture, the NES-Cell gNB-DU does not know if and when the UL WUS configuration provisioning for a NES Cell is ensured, and thus it cannot autonomously start OD-SIB1 transmission in the NES Cell. Instead, it requires assistance information from the NES-Cell gNB-CU.

Observation 2	If SIBx is encoded by the gNB-CU, the F1 impact for the introduction of SIBx is only to include the new SIB.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	In split architecture, the NES-Cell gNB-CU should indicate to the NES-Cell gNB-DU whether a NES Cell operation can be changed to OD-SIB1 transmission mode.

Proposal 2	RAN3 to agree to reuse the gNB-CU Configuration Update procedure over F1 for sending from NES-Cell gNB-CU to NES-Cell gNB-DU an indication that certain cell(s) can be operated in OD-SIB1 transmission mode.

Proposal 3	RAN3 to agree that SIBx (the new SIB containing one or more UL WUS configurations) is a CU-owned SIB.

Proposal 4	RAN3 to agree that the UL WUS configuration provision is controlled by Cell A gNB-CU.

Proposal 5	RAN3 to agree on the TP for BL CR for TS 38.473 provided in Appendix A.

# References

RP-242354	Revised WID: Enhancements of network energy savings for NR

R3-244708	Summary of offline disc at RAN3#125, Ericsson (Moderator)

R3-247806	Summary of offline disc at RAN3#126, Ericsson (Moderator)

Text Proposal for BL CR for TS 38.473

*************** Start of TP ***************

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6]. (FFS)

Interaction with the gNB-CU Configuration Update procedure:

As specified in TS 38.401 [4], once the UL WUS configuration provisioning is ensured, the gNB-CU shall initiate the gNB-CU Configuration Update procedure with the proper indication.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6]. (FFS)

Interaction with the gNB-CU Configuration Update procedure:

As specified in TS 38.401 [4], once the UL WUS configuration provisioning is ensured, the gNB-CU shall initiate the gNB-CU Configuration Update procedure with the proper indication.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Cells Allowed to Use OD-SIB1 Transmission List IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU can activate OD-SIB1 transmission in the cells indicated in the Cells Allowed to Use OD-SIB1 Transmission List IE. If OD-SIB1 transmission is active in a cell not indicated in the Cells Allowed to Use OD-SIB1 Transmission List IE, the gNB-DU shall deactivate OD-SIB1 transmission in that cell.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to Use OD-SIB1 Transmission List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to Use OD-SIB1 Transmission List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| UL WUS Configuration Information [FFS] | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

*************** End of TP ***************