3GPP TSG-RAN WG3 Meeting #127bis	R3-251853

Wuhan, China, 07 - 11 April 2025

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for Femto BL CR for TS 38.300) Discussion on SA2's LS on NR Femto node shared by PLMN and PNI-NPN

Document for:	Discussion

1	Introduction

In this meeting, RAN3 received an LS from SA2 about the femto node access control, with the content as follows [1].

SA2 discussed access control description agreed in RAN3, as captured in RAN3 BL CR to TS 38.300 (R3-247936).

- The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Among above three options, SA2 seeks some clarifications regarding the second option.

As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list.

As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration.

Therefore, SA2 would like to understand whether the second option in the RAN3 BL CR for NR Femto contradicts with the existing SA2 specifications.

To RAN3:

SA2 kindly asks RAN3 to clarify whether the second option in the RAN3 BL CR has any contradictions with the existing definitions in SA2 specifications regarding PNI-NPN and Network Sharing.

.

In this contribution, we provide our views on the issue of the SA2 LS.

2	Discussion

According to the SA2 LS, the PLMN cell and PNI-NPN cell should be distinguished and treated differently by the AMF. While in the RAN3 BL CR to TS 38.300, it is captured “The NR Femto may activate a cell shared by both PLMN and PNI-NPN”. That causes SA2’s concern on the potential contradiction between SA2 spec and RAN3 BL CR.

Clause 4.6 of TS 38.300 introduced the radio access network sharing, as follows:

------Exerpt from TS 38.300------ ## 4.6 Radio Access Network Sharing NG-RAN supports radio access network sharing as defined in TS 23.501 [3]. If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI-NPNs and SNPNs. NR access provides only one TAC and one Cell Identity per cell per PLMN, SNPN or PNI-NPN. In this version of the specification, a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]. Each Cell Identity associated with a subset of PLMNs, SNPNs or PNI-NPNs identifies its serving NG-RAN node.

According to TS 38.300, in case of RAN sharing, the “shared cell” is a physical cell which can be shared by both PLMN and PNI-NPN, but the PLMN and PNI-NPN will be assigned with different Cell Identities, because “a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]”.

------Exerpt from TS 38.331------

#### –	CellAccessRelatedInfo

The IE CellAccessRelatedInfo indicates cell access related information for this cell.

CellAccessRelatedInfo information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFO-START

CellAccessRelatedInfo   ::=         SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}             OPTIONAL,   -- Need R

...,

[[

cellReservedForFutureUse-r16        ENUMERATED {true}             OPTIONAL,   -- Need R

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16      OPTIONAL    -- Need R

]],

------skip unrelatted parts----------

----------unrelated parts are skipped-----------

#### –	PLMN-IdentityInfoList

The IE PLMN-IdentityInfoList includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

------skip unrelatted parts----------

----------unrelated parts are skipped-----------

Moreover, according to the excerption below from TS 38.331, it is also clear that both the PLMN-IdentityInfoList and NPN-IdentityInfoList can be included in CellAccessRelatedInfo IE of SIB 1, but a subset of PLMN can be associated with one Cell Identity, while a subset of PNI-NPN can be associated with another Cell Identity.

------exerpt from TS 38.331------

#### –	NPN-IdentityInfoList

The IE NPN-IdentityInfoList includes a list of NPN identity information.

NPN-IdentityInfoList information element

-- ASN1START

-- TAG-NPN-IDENTITYINFOLIST-START

NPN-IdentityInfoList-r16 ::=     SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16

NPN-IdentityInfo-r16 ::=         SEQUENCE {

npn-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16,

trackingAreaCode-r16             TrackingAreaCode,

ranac-r16                        RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity-r16                 CellIdentity,

cellReservedForOperatorUse-r16   ENUMERATED {reserved, notReserved},

------skip unrelatted parts----------

#### –	NPN-Identity

The IE NPN-Identity includes either a list of CAG-IDs or a list of NIDs per PLMN Identity. Further information regarding how to set the IE is specified in TS 23.003 [21].

NPN-Identity information element

-- ASN1START

-- TAG-NPN-IDENTITY-START

NPN-Identity-r16 ::=             CHOICE {

pni-npn-r16                      SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

cag-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16

},

snpn-r16                         SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

nid-List-r16                     SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

}

CAG-IdentityInfo-r16 ::=         SEQUENCE {

cag-Identity-r16                 BIT STRING (SIZE (32)),

manualCAGselectionAllowed-r16    ENUMERATED {true}                         OPTIONAL   -- Need R

}

NID-r16 ::=                      BIT STRING (SIZE (44))

-- TAG-NPN-IDENTITY-STOP

-- ASN1STOP

In the NG interface, the NR CGI composed by the PLMN Identity and the Cell Identity is used to indicate the logical cell, while such logical cell will not be shared by the PLMN and PNI-NPN according to the clarification in RAN’s specification TS 38.300. And, the information about UE’s serving cell which is visible by the AMF is the NR CGI contained in the user location information. So, from the CN’s point of view, a cell is the “logical cell” not the “physical cell”.

When UE access the NR femto cell, it will indicate the information about the selected PLMN/PNI-NPN/SNPN (i.e., selected network index IE) in Message 5 (i.e., the RRCSetupComplete) to the NG-RAN node, and the index for PLMN will be different from the index for PNI-NPN. In the following part, we will use an example to explain how the RAN sharing between PLMN and PNI-NPN works from RAN side and CN side.

If there are two UEs try to access a same NR femto cell, which is a physical cell be shared by PLMN and PNI-NPN, UE 1 selects the PNI-NPN, and UE 2 selects the PLMN. Then, based on the information in Message 5, the NG-RAN node will include the NPN Access Information IE in the INITIAL UE MESSAGE message for UE1, the user location information for UE1 contains NG CGI 1, and AMF will perform access control for UE1 based on the its allowed CAG list. For UE2, there is no NPN Access Information IE in the INITIAL UE MESSAGE message, the user location information for UE1 contains NG CGI 2, and AMF will not do any CAG based access control for the UE2. The cell identified by NR CGI 1 and the cell identified by NG CGI 2 are independent cells from the AMF’s point of view, so the AMF will never think a cell identified by one NG CGI can be accessed by both PLMN UE and PNI-NPN UE.

Observation 1: Based on RAN specification, a physical cell can be shared by PLMN and PNI-NPN, but the PLMN and the PNI-NPN will be associated with different Cell Identities.

Observtion 2: From the CN point of view, the cell information is the NR CGI (PLMN+ Cell Identity) contained in the user location information, while the NG CGI identifies a logical cell which will not be shared by PLMN and PNI-NPN.

Observtion 3: If the NR Femto node activates a shared physical cell, it will include different NG CGIs as the user location information for the UE selects PLMN and the UE selects PNI-NPN.

Based on the analysis above, current RAN3 BL CR does not technically contradict with the SA2 specifications. We suggest RAN3 slightly modify the wording for the BL CR to be aligned with clause 4.6, for eliminating the potential misunderstandings.

Proposal: RAN3 reply SA2 the second option in the RAN3 BL CR does not technically contradict with the existing definitions in SA2 specifications, and informs SA2 the updated BL CR to TS 38.300.

We provide the TP for BL CR to TS 38.300 and the reply LS in the Annex.

3	Conclusion

This paper provides analysis the access control of NR femto based on the LS from SA2, the following observation and proposal are obtained:

Observation 1: Based on RAN specification, a physical cell can be shared by PLMN and PNI-NPN, but the PLMN and the PNI-NPN will be associated with different Cell Identities.

Observtion 2: From the CN point of view, the cell information is the NR CGI (PLMN+ Cell Identity) contained in the user location information, while the NG CGI identifies a logical cell which will not be shared by PLMN and PNI-NPN.

Observtion 3: If the NR Femto node activates a shared physical cell, it will include different NG CGIs as the user location information for the UE selects PLMN and the UE selects PNI-NPN.

Proposal: RAN3 reply SA2 the second option in the RAN3 BL CR does not technically contradict with the existing definitions in SA2 specifications, and informs SA2 the updated BL CR to TS 38.300.

4. Reference

- R3-251524/S2-2502787. LS on NR Femto node shared by PLMN and PNI-NPN, SA2.

Annex 1 - TP for BL CR on TS 38.300

START OF CHANGE

4.x.4	Access Control

- Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE: The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto may activate a shared physical cell (see clause 4.6) which is shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

END OF CHANGE

Annex 2 - Reply LS on NR Femto node shared by PLMN and PNI-NPN

3GPP TSG-RAN3 Meeting #127b	R3-xxxxxx

Wuhan, China, 07th – 11th April 2025

Title:	[DRAFT] Reply LS on NR Femto node shared by PLMN and PNI-NPN

Response to:	R3-251524/S2-2502787

Release:	Rel-19

Work Item:	FS_VMR_Ph2, NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  updated Femto BL CR for 38.300

1. Overall Description:

RAN3 thanks SA2 for the LS on NR Femto node shared by PLMN and PNI-NPN and would like to provide the feedback as following:

There is no contradiction between RAN3’s BL CR and the SA2’s specification. The second bullet for the access control of NR Femto in the RAN3’s BL CR for TS 38.300 follows the legacy RAN sharing operation, which allows a physical cell be shared by PLMN and PNI-NPN. And same as legacy, the PLMN and the PNI-NPN supported by the same “physical cell” will be associated with different Cell Identities. Therefore, if the NR Femto node activates a shared physical cell, it will include different NG CGIs as the user location information for the UE selects PLMN and the UE selects PNI-NPN.

Clause 4.6 of TS 38.300 is exerpted as follows:

## 4.6 Radio Access Network Sharing NG-RAN supports radio access network sharing as defined in TS 23.501 [3]. If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI-NPNs and SNPNs. NR access provides only one TAC and one Cell Identity per cell per PLMN, SNPN or PNI-NPN. In this version of the specification, a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]. Each Cell Identity associated with a subset of PLMNs, SNPNs or PNI-NPNs identifies its serving NG-RAN node.

By the way, RAN3 updates the BL CR for TS 38.300 slightly, as attached.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

RAN3#128			19th May – 23rd May, 2025			Malta, MT

RAN3#129			25th August – 29th August, 2025		India, IN