3GPP TSG-RAN WG3 Meeting #127bis	R3-251883

Wuhan, China, 17th - 21st Apr 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Further discussion on emergency services and eDRX

Document for:	Discussion

- Introduction

In RAN2#128，RAN2 has discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. And it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle. Therefore, RAN2 sent an LS to SA2 and CT1 to ask them to consider the above scenario and provide feedback[1]. In last RAN3 meeting, this issue was discussed and the conclusion is that RAN3 waits progress in SA2.

In last SA2 meeting, SA2 has replied the LS and provided more clarification on this issue[2], which is given as below.

SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

In this contribution, we provide our views on the potential RAN3 impact based on the reply LS from SA2.

2	Discussion

As mentioned in the reply LS from SA2, the description of ARP value usage for Emergency Services has been given in TS 23.501.

-----------------------------------------------------------TS 23.501-------------------------------------------------------------------------

5.16.4.3	Mobility Restrictions and Access Restrictions for Emergency Services

When Emergency Services are supported and local regulation requires IMS Emergency Sessions to be provided regardless of Mobility Restrictions or Access Restrictions, the Mobility Restrictions or Access Restrictions (see clause 5.3.4.1) should not be applied to UEs receiving Emergency Services. Additionally, due to Mobility Restrictions or Access Restrictions (e.g. CAG restrictions) for normally registered UEs, that have established both non-emergency PDU Sessions and emergency PDU Session, the AMF indicates to the SMF to perform a local release of all non-emergency PDU Sessions via PDU Session Release procedure as specified in clause 4.3.4 of TS 23.502 [3]. The UE locally releases non-emergency PDU Sessions. The AMF and the UE behave as if the UE is emergency registered as described in TS 24.501 [47].

When the (R)AN resources for Emergency Services are established, the ARP value for Emergency Services indicates the usage for Emergency Services to the 5G-AN.

During handover, the source NG-RAN and source AMF ignore any UE related restrictions during handover evaluation when there is an active PDU Session associated with emergency service.

-----------------------------------------------------------TS 23.501-------------------------------------------------------------------------

Currently, each PLMN/operator reserves a specific ARP value for emergency PDU session. When the emergency PDU session is established, the ARP value shall be provided to RAN node. If the received ARP value can match with the reserved ARP value for emergency services, the RAN node is able to be aware of an emergency PDU session.

Observation: The NG-RAN is able to be aware of the existence of emergency PDU session based on the received ARP value from CN.

As given in the reply LS from SA2, some companies still have concern on whether to introduce new signalling to NG-RAN on this issue. According to current NGAP specification 38.413, the Priority Level IE has already introduced within the Allocation and Retention Priority IE in the PDU Session Resource Setup procedure.

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

9.3.1.19	Allocation and Retention Priority

This IE specifies the relative importance of a QoS flow compared to other QoS flows for allocation and retention of NG-RAN resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (1..15) | Desc.: This IE defines the relative importance of a resource request (see TS 23.501 [9]). Usage: Values are ordered in decreasing order of priority, i.e., with 1 as the highest priority and 15 as the lowest priority. |

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevelARP				PriorityLevelARP,

pre-emptionCapability			Pre-emptionCapability,

pre-emptionVulnerability		Pre-emptionVulnerability,

iE-Extensions		ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

Therefore, there is no need to introduce a new signalling to indicate the existence of the emergency PDU session from CN to RAN.

Proposal 1: There is no need to introduce a new signalling to indicate the existence of the emergency PDU session from CN to RAN.

Based on the received LS from SA2, some companies in SA2 thought that UE configured with emergency PDU session should be kept in RRC_CONNECTED. However, from the RAN perspective, ensuring this behavior can be challenging. For example:

- A radio link failure (RLF) occurs, forcing the UE to transition to RRC_IDLE state;

- The network congestion or prolonged UE PDU session inactivity triggers the network to release the UE to RRC_INACTIVE or RRC_IDLE state.

Therefore, we propose that the NG-RAN should be allowed to release a UE with an emergency PDU session to either RRC_IDLE or RRC_INACTIVE state.

Proposal 2: NG-RAN should be allowed to release a UE with an emergency PDU session to either RRC_IDLE or RRC_INACTIVE state.

According to current TS23.501, to page a UE in RRC_IDLE, when the MT callback that follows an emergency call, the UE and AMF shall not use the extended idle mode DRX. In other words, the eDRX configuration will not used for CN paging in this case.

If the NG-RAN releases a UE with emergency services to RRC_INACTIVE, the NG-RAN must not configure eDRX parameters during the release procedure. In last RAN3 meeting, in order to achieve this, there are two main options on the table, i.e., CN-based solution and RAN-based solution.

1) When the UE has a PDU session for emergency services, the AMF shall not configure the eDRX cycle to the NG-RAN node (CN-based solution)

2)  When the UE has a PDU session for emergency services, the NG-RAN node is able to be aware of the emergency PDU session and shall not use the eDRX configuration provided by the AMF (RAN-based solution)

Technically, both of the two options can solve the issue. While, based on the above Observation, the RAN-based solution has no impact on CN, and it could be done by adding some clarification in stage 2 and stage 3 respectively.

Proposal 3:  When a UE with emergency services is released to RRC_INACTIVE by the NG-RAN, the NG-RAN node shall not configure UE with the eDRX configuration provided by AMF. Agree the corresponding stage 2 and stage 3 CRs accordingly.

Although RAN3 is in the cc list of the incoming LS from RAN2 and SA2, the question needs to clarified has the impact on RAN3. And it is beneficial that RAN3 replies the LS to RAN2 and SA2 to provide RAN3’s understanding on this issue.

Proposal 4:  Reply the LS to RAN2 and SA2 to provide RAN3’s understanding on this issue.

3	Summary

Observation: The NG-RAN is able to be aware of the existence of emergency PDU session based on the received ARP value from CN.

Proposal 1: There is no need to introduce a new signalling to indicate the existence of the emergency PDU session from CN to RAN.

Proposal 2: NG-RAN should be allowed to release a UE with an emergency PDU session to either RRC_IDLE or RRC_INACTIVE state.

Proposal 3:  When a UE with emergency services is released to RRC_INACTIVE by the NG-RAN, the NG-RAN node shall not configure UE with the eDRX configuration provided by AMF. Agree the corresponding stage 2 and stage 3 CRs accordingly.

Proposal 4:  Reply the LS to RAN2 and SA2 to indicate RAN3’s understanding on this issue.

The corresponding CRs and draft LS are given in [3][4][5][6][7].

4	Reference

[1] R3-250006 (R2-2411041), LS on emergency services and eDRX, RAN2

[2] R3-251521 (S2-2501367), Reply LS on emergency call back and paging, SA2

[3] R3-251884,  [Draft] Reply LS on emergency call back and paging, ZTE

[4] R3-251885, Clarification on emergency services and eDRX, ZTE

[5] R3-251886, Clarification on emergency services and eDRX, ZTE

[6] R3-252238, Clarification on emergency services and eDRX over NG, ZTE

[7] R3-252239, Clarification on emergency services and eDRX over NG, ZTE