3GPP TSG-RAN WG3 Meeting #127bis			                           R3-251893

Wuhan, China, 7th – 11th Apr 2025

Agenda Item:	17.3

Source:	ZTE Corporation

Title:		(TP to BL CR for 38.423) Further discussion on on-demand SIB1

Document for:	Discussions & Approval

1	Introduction

- This paper provides further discussion on on-demand SIB1.

2	Discussion

During the RAN3#126 meeting, the following agreements were reached, and a corresponding XnAP BL CR was endorsed in R3-250912.

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

When Cell A first receives UL WUS, It is agreed that "it broadcasts in SIBx at the first opportunity", however, as the NES cell may require feedback from other base stations to determine whether to enter the NES state. If the NES cell does not enter the NES state, the immediate broadcasting by Cell A would result in unnecessary resource and energy consumption, especially if multiple Cell A and base stations initiate the broadcasting.

Observation 1: If the NES cell does not enter the NES state, the immediate broadcasting by Cell A would result in unnecessary resource and energy consumption.

As a compromise strategy, we propose introducing a delay time period for broadcasting. When Cell A first receives the UL WUS, it will delay broadcasting until the period expires. This approach provides the NES cell with an opportunity to notify the assisting cells to cancel the broadcast if it fails to enter the NES state .

Proposal 1: Introduce a delay time period for broadcasting. When Cell A first receives the UL WUS, it will delay broadcasting until the period expires. This approach provides the NES cell with an opportunity to notify the assisting cells to cancel the broadcast if it fails to enter the NES state .

Based on the current RAN3 progress, there is only one Class 1 message for requesting UL WUS provision, and it has been agreed that "one option is “start with UL WUS Configuration." We believe that this procedure should also support the pause and cancellation of the UL WUS configuration. When the request type is set to "cancel", the previously stored UL WUS configuration shall be removed.

Proposal 2: The class 1 procedure supports the functionality of start/pause/cancel of the UL WUS configuration. When the request type is set to "cancel", the previously stored UL WUS configuration shall be removed.

Considering that there may be more than one NES cell in Node1, it is proposed that the UL WUS Configuration Provision Request message include a list of UL WUS configurations for the NES cells, with each configuration corresponding to a specific NES cell.

Proposal 3: The UL WUS Configuration Provision Request message includes a list of UL WUS configurations for the NES cells, with each configuration corresponding to a specific NES cell.

Based on the above discussion, we provide corresponding TP for TS38.423 in annex.

Proposal 4: RAN3 discuss the corresponding TP for TS38.423 in annex.

Additionally, we suggest waiting for the Xn interface solution to be finalized before discussing the impact on the F1 interface.

Proposal 5: RAN3 should wait for the Xn interface solution to be finalized before discussing impact on the F1 interface.

3 Conclusion

Based on the above discussion, we provide the following proposals.

Observation 1: If the NES cell does not enter the NES state, the immediate broadcasting by Cell A would result in unnecessary resource and energy consumption.

Proposal 1: Introduce a delay time period for broadcasting. When Cell A first receives the UL WUS, it will delay broadcasting until the period expires. This approach provides the NES cell with an opportunity to notify the assisting cells to cancel the broadcast if it fails to enter the NES state .

Proposal 2: The class 1 procedure supports the functionality of start/pause/cancel of the UL WUS configuration. When the request type is set to "cancel", the previously stored UL WUS configuration shall be removed.

Proposal 3: The UL WUS Configuration Provision Request message includes a list of UL WUS configurations for the NES cells, with each configuration corresponding to a specific NES cell.

Proposal 4: RAN3 discuss the corresponding TP for TS38.423 in annex.

Proposal 5: RAN3 should wait for the Xn interface solution to be finalized before discussing impact on the F1 interface.

Annex new TP for TS38.423

*************** Start of Changes ***************

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

8.x.x	UL WUS Configuration Provision (FFS)

8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide a list of  UL WUS configuration information of one or more cells supporting on-demand SIB1 to NG-RAN node2 and request NG-RAN node2 to transmit UL WUS configuration information.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to discontinue transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message to the peer NG-RAN node2 to start, pause, or cancel the provision of UL WUS configuration information of in one or more assisting cells of the NG-RAN node2 .Upon receipt of the message, the NG-RAN node2 :

-	shall initiate the requested provision according to the parameters given in the request in case the Request Type IE is set to "start". if the Delay time IE is present, delays provision accordingly; or

-	shall pause the indicated (previously admitted) provision in case the Request Type IE is set to "stop"; or

-  shall cancel the indicated (previously admitted) provision in case the Request Type IE is set to "cancel", and remove the corresponding UL WUS configuration.

NG-RAN node2 shall then respond with the UL WUS CONFIGURATION PROVISION RESPONSE message to indicate the fulfillment of each assisting cell.

8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot broadcast UL WUS Configuration in SIBx (Naming FFS), or a failure occurs during UL WUS Configuration Provision (Naming FFS), the NG-RAN node2 shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value.

8.x.x.4	Abnormal Conditions

Void.

8.x.y	UL WUS Configuration Transmission Status Update (FFS)

8.x.y.1	General

The purpose of the UL WUS Configuration transmission Status Update procedure is to enable an NG-RAN node2 to inform NG-RAN node1, which initiated the UL WUS configuration transmission request, that the ongoing broadcasting of the UL WUS configuration has stopped or resumed in one or more assisting cells.

The procedure uses non UE-associated signalling.

8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

The NG-RAN node2 initiates the procedure by sending a UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message to an NG-RAN node1 to inform that the ongoing broadcasting of the UL WUS configuration has stopped or resumed in one or more assisting cells.

8.x.y.3	Unsuccessful Operation

Not applicable.

8.x.y.4	Abnormal Conditions

Void.

*************** Next Change ***************

9.x.x.a	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the NG-RAN node2 to start, stop,or cancel the transmitting of UL WUS configuration information.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| NES Cell UL WUS List |  | 1 |  | The list of the UL WUS configuration of NES cells at the NG-RAN node1 | YES | ignore |
| >NES cell UL WUS item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>UL WUS Configuration | O |  | OCTET STRING | FFS. | – |  |
| >> NES Cell ID | M |  | NR CGI 9.2.2.7 | This IE indicates the UL WUS configuration's cell. | – |  |
| >>Request Type | M |  | ENUMERATED (start, stop, cancel, …) |  | – |  |
| >> Delay time | O |  | INTEGER (0.. 600, ...) unit : 1s | This IE indicates the time period for the NG-RAN node delays transmitting the UL WUS when the request type is set to "start.". | – |  |
| >>Assisting Cells List |  | 1 |  | List of assisting cells at the NG-RAN node2 for transmitting the indicated UL WUS configuration. | – |  |
| >>>Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>Assisting Cell ID | M |  | NR CGI 9.2.2.7 | Indicates the NR_CGI of assisting cell at the NG-RAN node2 | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

9.x.x.a	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by a NG-RAN node2 to a NG-RAN node1 to acknowledge the request of UL WUS configuration provision.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| NES Cell UL WUS List |  | 1 |  | The list of the UL WUS configuration of NES cells at the NG-RAN node1 | YES | ignore |
| >NES cell UL WUS item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  |  |  |
| >> NES Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Assisting Cells List |  | 1 |  | List of assisting cells at the NG-RAN node2 for transmitting the indicated UL WUS configuration. | – |  |
| >>>Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 | Indicates the NR_CGI of assisting cell at the NG-RAN node2 | – |  |
| >>>>Request result | M |  | ENUMERATED (accept, reject, …) |  | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

9.x.x.c	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to an NG-RAN node1 to indicate that the UL WUS configuration provision cannot be supported by NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

9.x.x.a	UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE

This message is sent by an NG-RAN node2to an NG-RAN node1 to indicate that the updated status of UL WUS configuration transmission.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| NES Cell UL WUS List |  | 1 |  | The list of the UL WUS configuration of NES cells at the NG-RAN node1 | YES | ignore |
| >NES cell UL WUS item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >> NES Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Assisting Cells List for Status Change |  | 1 |  | List of Assisting Cells at NG-RAN node2 for status change in transmitting the indicated UL WUS Configuration. | – |  |
| >>>Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 | Indicates the NR_CGI of assisting cell at the NG-RAN node2 | – |  |
| >>>>status | M |  | ENUMERATED (broadcasting, not broadcasting, …) |  | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

*************** End of Changes ***************