3GPP TSG-RAN WG3 Meeting #127bis	R3-251909

Wuhan, China, 07 - 11 April, 2025

Agenda Item:	19.2

Source:	Ericsson

Title:	Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD

Document for:	Discussion, Approval

# 1	Introduction

RAN1 has started work on “Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, as per WID in RP-241614.

As part of this WID, RAN3 has been identified as one of the WGs that needs to contribute to solutions on enhancements for CLI handling, as per the excerpt below (from RP-241614):

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - … - Note: Without dedicated optimization for dynamic/flexible TDD. - … - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

Additionally, RAN1 sent an LS to RAN3 in R3-245006, where the objectives quoted above were clarified as follows:

While the WID objective lists only RAN3 as responsible working group, RAN1 has discussed the sub-bullets of the objective and would like to clarify several PHY/L1-related aspects that are not typically discussed in RAN3. RAN1 would like to inform RAN3 (RAN2 is cc’d) of the following RAN1 agreement. Agreement Send an LS to RAN3 with the following information (cc: RAN2) From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB. Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3. # 2 Actions To TSG RAN3 ACTION: RAN1 respectfully asks RAN3 to take the above information into account in their further work.

During the last RAN3 meetings discussinos on CLI mitigation were carried out. For clarity, we report below the agreements and open points taken so far in RAN3:

RAN3-125bis:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

RAN3-126:

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Exchange reference signalling configuration(SSB and NZP CSI-RS) in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure?

RAN3-127:

RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2.

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

To define CLI-mitigation request as an optional IE. FFS on the details.

How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

Additionally, RAN3 agreed an LS to RAN2 in R3-250888, where the following was stated:

RAN3 has started to work on the SBFD information exchange among gNBs for CLI mitigation, according to the received LS in R1-2407533 from RAN1. RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, and also between gNB-DU and gNB-CU in case of split architecture. RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration information, which should be based on parameters provided by RAN1. With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN2 to work on the IE design of SBFD time and frequency configuration information similar to the TDD-UL-DL-ConfigurationCommon IE which is signalled over Xn and F1. ACTION: RAN3 respectfully asks RAN2 to work on the IE design of SBFD time and frequency configuration informaiton.

In this contribution an analysis of the work RAN3 needs to carry out to support enhancements for CLI handling is made and conclusions on a way forward are derived. A TP for TS 38.423 is provided in R3-251910.

# 2	Discussion

## 2.1	Semi-static cell-specific SBFD time and frequency location configuration

The first objective to analyse concerns the exchange of following information between gNBs:

–	Semi-static cell-specific SBFD time and frequency location configuration

With respect to this objective RAN3 made some good progress at RAN3-127 where the following was agreed:

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2.

RAN3 has also sent an LS to RAN2 in R3-250888, where RAN3 requests RAN2 to define an RRC IE to be used transparently over the RAN interfaces to signal between RAN nodes the SBFD time and frequency configuration.

In light of this, it is proposed that RAN3 agrees to add an IE carrying the SBFD time and frequency configuration in the in the Served Cell Information IE over the Xn and F1 interfaces, encoded as an OCTET STRING. A provisional name proposed for this IE is SBFD UL-DL Configuration Common NR IE.

Proposal 1: RAN3 to add in the Served Cell Information NR IE an IE defined as OCTET STRING, to be defined by RAN2, specifying “Semi-static cell-specific SBFD time and frequency location configuration”, and to signal it transparently over the F1 and Xn interfaces for inter node communication. For the Xn, the IE is included in the XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message

## 2.2	Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

The second objective to analyse concerns the exchange of following information between gNBs:

–	Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

With respect to this objective, RAN3 has captured the following agreement and open point in its most recent discussion:

RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

The above agreements indicate that RAN3 does not need to add any information to their protocols to describe SSB signals used for CLI mitigationpurposes.

The above open point was captured because of proposals concerning the signalling of specific NZP CSI-RS reference signals for CLI mitigation purposes. Some of these proposals also contemplate specific signalling procedures between NG-RAN nodes to indicate when such CLI-mitigation-specific RS would be activated.

With respect to this open point, the first clarification to be made is that RAN1 has not agreed to the definition of CLI-mitigation-specific SSB and NZP CSI-RS reference signals, hence this assumption cannot be made. 
The standard shall allow the reuse of SSB and NZP CSI-RS signals used by the NG-RAN for whatever other purpose, also for CLI-mitigation purposes. 
This is logical because transmission of reference signals over the air is expensive in many aspects, e.g. radio resource consumption, cross cell interference, power consumption etc. Hence the standard shall not force the adoption of new specific RSs for CLI mitigation when that has not been identified as a requirement.
An implementation that chooses to use specific SSB or NZP CSI-RS signals for CL mitigation pruposes can in any case still reuse existing procedures to signal reference signals configurations between NG-RAN nodes at the time such RS are activated.

The second clarification to be made is that it is unclear how CLI aggressors/victims can be identified if the reference signals used for CLI mitigation is not always transmitted. Hence, it is unclear how a gNB would e.g. identify which other gNB is an aggressor and after that trigger an Xn signalling message towards that node to indicate the activation of specific reference signals for CLI mitigation.

Observation 1: The standard shall not assume that SSBs and NZP CSI-RS reference signals are used solely for CLI mitigation purposes

Observation 2: Existing Xn interface management procedures, i.e. Xn Setup and NG-RAN node Configuration Update, can be used to exchange per cell reference signals configurations used for CLI mitigation purposes

In light of the observations above, we now discuss how to addres the measurement object configuration objective. To do that, it should be reminded that RAN1 specifies in their LS that:

“From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.”

With the above, RAN1 states that configuration for the NZP-CSI-RS signals is already specified in TS 38.331 as part of the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE. One point to note is that the NZP-CSI-RS-ResourceSet IE points to multiple instances of the NZP-CSI-RS-Resource IE, each identified by means of a NZP-CSI-RS-ResourceId IE, as it can be seen below:

#### NZP-CSI-RS-ResourceSet The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters. NZP-CSI-RS-ResourceSet information element -- ASN1START -- TAG-NZP-CSI-RS-RESOURCESET-START NZP-CSI-RS-ResourceSet ::= SEQUENCE { nzp-CSI-ResourceSetId NZP-CSI-RS-ResourceSetId, nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId, repetition ENUMERATED { on, off } OPTIONAL, -- Need S aperiodicTriggeringOffset INTEGER(0..6) OPTIONAL, -- Need S trs-Info ENUMERATED {true} OPTIONAL, -- Need R ..., [[ aperiodicTriggeringOffset-r16 INTEGER(0..31) OPTIONAL -- Need S ]], [[ pdc-Info-r17 ENUMERATED {true} OPTIONAL, -- Need R cmrGroupingAndPairing-r17 CMRGroupingAndPairing-r17 OPTIONAL, -- Need R aperiodicTriggeringOffset-r17 INTEGER (0..124) OPTIONAL, -- Need S aperiodicTriggeringOffsetL2-r17 INTEGER(0..31) OPTIONAL -- Need R ]], [[ resourceType-r18 ENUMERATED {periodic} OPTIONAL -- Cond LTM ]] } CMRGroupingAndPairing-r17 ::= SEQUENCE { nrofResourcesGroup1-r17 INTEGER (1..7), pair1OfNZP-CSI-RS-r17 NZP-CSI-RS-Pairing-r17 OPTIONAL, -- Need R pair2OfNZP-CSI-RS-r17 NZP-CSI-RS-Pairing-r17 OPTIONAL -- Need R } NZP-CSI-RS-Pairing-r17 ::= SEQUENCE { nzp-CSI-RS-ResourceId1-r17 INTEGER (1..7), nzp-CSI-RS-ResourceId2-r17 INTEGER (1..7) } -- TAG-NZP-CSI-RS-RESOURCESET-STOP -- ASN1STOP

It would therefore suffice to signal the NZP-CSI-RS-ResourceSet IE and a list of NZP-CSI-RS-Resource IE as specified in TS38.331 over the F1 and Xn information as part of the Served Cell Information NR IE to achieve inter node exchange of such configurations.

Proposal 2: RAN3 to agree that, for inter gNB signalling of “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS”, the NZP-CSI-RS-ResourceSet IE defined in TS38.331 and a list of NZP-CSI-RS-Resource IE defined in TS38.331 will be included in the Served Cell Information NR IE and signalled transparently over the F1AP and XnAP. For the Xn, the IE is included in the XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message

The objective also mentions inter node signalling of SSB resource configuration. As confirmed in the latest agreements in RAN3, this information is already contained in the Measurement Timing Configuration IE signalled as part of the Served Cell Information NR IE over F1 and Xn, where the IE contains the MeasurementTimingConfiguration IE defined in TS 38.331.

Therefore, no enhancements need to be applied fo the signalling of SSB configuration details.

## 2.3	Strongest DL beam information and CLI mitigation request

The third objective to analyse concerns the exchange of following information between gNBs:

–	Strongest DL beam information

With respect to this objective, RAN1 specifies in their LS that:

“Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.”

The strongest DL beam information is an indication sent from a victim gNB to an aggressor gNB of the strongest DL beam detected by the victim gNB. Such indication from victim to aggressor gNB helps taking actions towards CLI mitigation. However, it should be pointed out that the WID is very clear in stating that:

“No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3”.

The above agreement is valid for the “CLI-mitigation request”, but the concept is the same for reception of Strongest DL Beam information, namely a node that receives this information may use it for CLI mitigation although the specific CLI actions (and node behaviour) shall not be specified. Therefore, even though a Strongest DL Beam information may be signalled from one gNB to another, there should be no description in the standard of how the receiving gNB will use this information to mitigate CLI.

Conclusion 1: While the strongest DL beam information is signalled from a gNB to another gNB to indicate the strongest beam detected and to possibly help in taking CLI mitigation actions, RAN3 shall not specify any receiver behaviour concerning CLI mitigation actions.

The strongest DL beam information can be provided with respect to a detected CSI-RS resource or with respect to an SSB. As expressed in the LS from RAN1, a CRI is used in case the strongest DL beam information is provided for a CSI-RS resource. In this case the CRI is an Index (INTEGER) in the range from 1 to N, where “N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB”.

The NZP-CSI-RS resources that can be exchanged between gNBs follow the definitions in the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE in TS 38.331. When looking at these IEs in TS 38.331 it can be easily deduced that the maximum number of resources per set is defined as follows:

maxNrofNZP-CSI-RS-ResourcesPerSet = 64

Therefore, in case the Strongest DL Beam identifies NZP-CSI-RS resources, the CRI can be an INTEGER (1..64).

On the other hand, in case the Strongest DL Beam identifies SSB resources, the information to exchange should be an SSB index, which is already defined over Xn as SSB Index: INTEGER (0..63). It is of course the case that if the NZP-CSI-RS-Resources in the set is less than 64, the CRI shall not be selected to a number higher than the number of resources in the set.

Proposal 3: It is proposed to represent the CRI as an INTEGER (1..64), where the CRI shall never exceed the number of NZP-CSI-RS-Resources in the set, and to represent an SSB Index by means of the legacy Xn SSB-Index IE

At the last RAN3 meeting, the following open point was captured:

How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

Given that both the CRI and the SSB Index that represent the strongest DL beam information are very small pieces of information, their signalling between gNBs could be achieved by adding this information into existing procedures. Indeed, defining a completely new procedure would incur very high impact when considering the very small amount of information to transfer.

We note that at the last RAN3 meeting a number of companies proposed to support the exchange of the CLI Mitigation Request and Strongest DL Beam Information by means of the F1/Xn: Resource Status Request/Response/Update messages.

Such approach is, in our view, not suitable for the CLI mitigation solution defined by RAN1, here are some reasons to justify out position.

The CLI mitigation solution defined by RAN1 is based on a victim gNB signalling to an aggressor gNB an indication that CLI is detected. This happens by signalling the CLI Mitigation Request and/or the Strongest DL Beam Information. Such design implies that the victim gNB shall be free to trigger signalling towards an aggressor gNB whenever CLI is detected and it is in need of mitigation.

However, if the F1/Xn: Resource Status Request/Response/Update messages are used, a victim gNB will only be able to signal the CLI Mitigation Request and Strongest DL Beam Information to the aggressor gNB if the aggressor has initiated the Resource Status Request/Response procedure. Namely, if the aggressor does not trigger such procedure, no CLI indication can be reported by the victim gNB and no CLI mitigation actions can be triggered at all, deniying the whole purpose of CLI mitigation support.

Observation 3: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information prevents a victim gNB to autonomously flag the presence of strong CLI and trigger CLI mitigation actions

Moreover, using the F1/Xn: Resource Status Request/Response/Update messages implies that the CLI Mitigation Request and Strongest DL Beam Informaiton have to be reported periodically. This mechanism does not fit the nature of CLI occurrence, which is volatile, i.e. it might occur for e.g. 1 second, then disappear, then occur again after one minute and so on. Hence the periodic reporting of such information does not fit this process and it may generate the following drawbacks:

- Reporting of short lived CLI events is missed due to coarse periodicity for the Resource Status Update messages

- Resource Status Reporting messages will be signalled even when there is no detected CLI. This could happen often, causing an unnecessary waste of signalling resources and increase of signalling load

Observation 4: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information incurs in missing the occurrence of short lived CLI events and it generates unnecessary signalling when there is no need for CLI mitigation actions

Finally, the WID in [1] is very clear in stating that there is “No need to further send a stop message for CLI mitigation.”. Reusing the F1/Xn: Resource Status Request/Response/Update implies a “start” and a “stop” to the CLI mitigation process, which is not needed.

Observation 5: the WID in RP-241614 clearly states that there is “No need to further send a stop message for CLI mitigation.”, while using the F1/Xn: Resource Status Request/Response/Update implies a “start” and a “stop” to the CLI mitigation process. This causes unnecessary complexity without a need for it.

In concludsion, we believe that a better signalling solution for CLI mitigation consists in reusing the Xn: NG-RAN Node Configuraiton Update procedures and the F1: gNB-DU/gNB-CU Configuration Update procedures for the exchange of CLI mitigation repated information.

Proposal 4: RAN3 to agree that CLI mitigation information exchange, namely the the strongest DL Beam information per cell, consisting of CRI or SSB Index, and the CLI MitigationRequest, is supported by means of addition of the information in the Xn: NG-RAN Node Configuraiton Update procedures and the F1: gNB-DU/gNB-CU Configuration Update procedures

The fourth objective to analyse concerns the exchange of following information between gNBs:

–	CLI-mitigation request

We have already discussed how a CLI mitigation request does not require any normative behaviour at the gNB receiving it. Therefore, a CLI mitigation request is simply an indication from a victim gNB to an aggressor gNB that CLI mitigation is needed for a given cell.

Another important piece of guidance from RAN1 is that there is “No need to further send a stop message for CLI mitigation”.

It is therefore proposed that the CLI mitigation request is encoded as an ENUMERATED per cell in the NG-RAN Configuration Update message.

Proposal 5: RAN3 to agree to signal the CLI Mitigation Request as an ENUMERATED per cell included in the XnAP: NG-RAN Node Configuration Update message

At RAN3-125bis the following was captured:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

Our understanding for adding an FFS for these two items is that some companies wanted to discuss whether a Strongest DL beam Information and CLI mitigation request could be also “implicit” and for that not explicitly signalled over the F1 and Xn.

Such approach is presented in R3-245562, there the two following options are presented:

- Option 1 (explicit CLI Mitigation Request): Victim gNB first exchanges the CLI-RS configuration (either for SSB or NZP-CSI-RS or both) with the aggressor gNB. The aggressor gNB measures the channel on the corresponding resources. Once inter-cell CLI is detected, the victim gNB sends a CLI Mitigation Request to the aggressor gNB containing cell id. Upon the reception, the aggressor gNB starts applying the CLI mitigation scheme (i.e., beam nulling). - Option 2 (implicit CLI Mitigation Request): There is no initial CLI measurement resource exchange. When the victim gNB detects increased CLI level based on UL performance degradation, it triggers CLI measurement resource exchange and activation of CLI-RS transmission. Upon the reception of the CLI-RS configuration, the aggressor gNB is expected to perform measurements and also apply the beam nulling scheme. In this way the signalling of CLI measurement resource information is an implicit trigger of CLI mitigation.

We understand that, while the options above refer to the CLI Mitigation Request, they were discussed to also cover the case of Strongest DL Beam Information.

We note that Option 1 is in line with the guidance sent by RAN1 in the LS in R3-245006, as such LS describes the CLI Mitigation Requests and Strongest DL Beam Informaiton as explicit pieces of information signalled between gNBs.

Observation 6: Signalling of the CLI Mitigation Request and Strongest DL Beam Information as explicit IEs over the F1 and Xn interfaces is in line with the guidance from RAN1 in their LS in R3-245006

We also note that Option 2 above implies that the reference signals used to evaluate whether CLI interference is present or not seem to be considered as dedicated to CLI mitigation. Namely, they are only activated when CLI detection is needed. 
As already mentioned in the previous section, we believe that the standard shall not mandate such approach. It should be possible to reuse reference signals already in use for other purposes also for the detection of CLI, hence such reference signals would not need to be activated “ad hoc”. As a consequence, an explicit signalling of the CLI Mitigation Requests and Strongest DL Beam Informaiton would be needed.
Further, Option 2 seems to be based on an initial detection of CLI based on UL performance degradation, but how would it be possible to know from which gNB/cell/beam such CLI is generated? Without knowing this information it is not possible to trigger signalling of the CLI measurement resource exchange and activation of CLI-RS transmission toward the aggressor node, as Option 2 describes.

In conclusion, it is proposed that the FFSs are removed from the following agreements and that an explicit signalling of CLI Mitigation Requests and Strongest DL Beam Informaiton is supported over F1 and Xn as part of the node configuration update procedures.

Proposal 6: RAN3 agrees to explicitly exchange the following information over Xn interface and F1 interface as separate IEs over the node configuration update proceures:

-	Strongest DL beam information

-	CLI-mitigation request

## 2.5	Overall Solution for CLI mitigation handling

In light of the above analysis, the overall solution proposed to be agreed in RAN3 for CLI mitigation handling consists of the following:

- Include in the Served Cell Information NR over F1AP and XnAP the “Semi-static cell-specific SBFD time and frequency location configuration” to be defined by RAN2 in TS 38.331. This information is to be passed transparently.

- Include in the Served Cell Information NR over F1AP and XnAP a list of NZP-CSI-RS-Resource IEs and NZP-CSI-RS-ResourceSet IE defined in TS 38.331. This information is to be passed transparently.

- Include in the XnAP NG-RAN NODE CONFIGURATION UPDATE message, on a per-cell basis, the Strongest DL Information IE, defined as a CHOICE between the following IEs:

  - CRI: INTEGER (1..64).

  - SSB Index: INTEGER (0..63).

- Include in the XnAP NG-RAN NODE CONFIGURATION UPDATE message, on a per-cell basis, the CLI Mitigation Request defined as an ENUMERATED with value “True”.

# 3	Conclusion

In this paper an analysis of the RAN3´s objectives on Evolution of enhancements for CLI handling, as part of the WI “NR duplex operation: Sub-band full duplex (SBFD)”, has been carried out.

The paper derived the following observations, proposals and conclusions:

Proposal 1: RAN3 to add in the Served Cell Information NR IE an IE defined as OCTET STRING, to be defined by RAN2, specifying “Semi-static cell-specific SBFD time and frequency location configuration”, and to signal it transparently over the F1 and Xn interfaces for inter node communication. For the Xn, the IE is included in the XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message

Observation 1: The standard shall not assume that SSBs and NZP CSI-RS reference signals are used solely for CLI mitigation purposes

Observation 2: Existing Xn interface management procedures, i.e. Xn Setup and NG-RAN node Configuration Update, can be used to exchange per cell reference signals configurations used for CLI mitigation purposes

Proposal 2: RAN3 to agree that, for inter gNB signalling of “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS”, the NZP-CSI-RS-ResourceSet IE defined in TS38.331 and a list of NZP-CSI-RS-Resource IE defined in TS38.331 will be included in the Served Cell Information NR IE and signalled transparently over the F1AP and XnAP. For the Xn, the IE is included in the XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message

Conclusion 1: While the strongest DL beam information is signalled from a gNB to another gNB to indicate the strongest beam detected and to possibly help in taking CLI mitigation actions, RAN3 shall not specify any receiver behaviour concerning CLI mitigation actions.

Proposal 3: It is proposed to represent the CRI as an INTEGER (1..64), where the CRI shall never exceed the number of NZP-CSI-RS-Resources in the set, and to represent an SSB Index by means of the legacy Xn SSB-Index IE

Observation 3: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information prevents a victim gNB to autonomously flag the presence of strong CLI and trigger CLI mitigation actions

Observation 4: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information incurs in missing the occurrence of short lived CLI events and it generates unnecessary signalling when there is no need for CLI mitigation actions

Observation 5: the WID in RP-241614 clearly states that there is “No need to further send a stop message for CLI mitigation.”, while using the F1/Xn: Resource Status Request/Response/Update implies a “start” and a “stop” to the CLI mitigation process. This causes unnecessary complexity without a need for it.

Proposal 4: RAN3 to agree that CLI mitigation information exchange, namely the the strongest DL Beam information per cell, consisting of CRI or SSB Index, and the CLI MitigationRequest, is supported by means of addition of the information in the Xn: NG-RAN Node Configuraiton Update procedures and the F1: gNB-DU/gNB-CU Configuration Update procedures

Proposal 5: RAN3 to agree to signal the CLI Mitigation Request as an ENUMERATED per cell included in the XnAP: NG-RAN Node Configuration Update message

Observation 6: Signalling of the CLI Mitigation Request and Strongest DL Beam Information as explicit IEs over the F1 and Xn interfaces is in line with the guidance from RAN1 in their LS in R3-245006

Proposal 6: RAN3 agrees to explicitly exchange the following information over Xn interface and F1 interface as separate IEs over the node configuration update proceures:

-	Strongest DL beam information

-	CLI-mitigation request

A TP mirroring the above proposals is available for agreement in R3-251910.

# 4	References

- RP-241614, WID on Evolution of NR duplex operation: Sub-band full duplex (SBFD)