3GPP TSG-RAN WG3 Meeting #127-bis	R3-251925

Wuhan, CN, 7-11 Apr, 2025

Title:	(TP for SON BLCR for 37.340, 38.423) MRO for others

Agenda Item:	10.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms.

# 2	Discussion

## 2.1	MRO for CHO with candidate SCGs

The last RAN3 meeting has achieved the following agreements:

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

The last RAN2 meeting agreed on the following enhancements to the RLF report and SCGFailureInformation:

Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC.

Enhance RLF report for CHO with candidate SCGs to include at least the following information:

- -	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

- -	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

- -	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

We should avoid specifying the procedure in a way that the UE sends redundant information

### 2.1.1	Failure scenarios

In case of too late CHO, the RLF report including the fulfilled PCell ID and C-RNTI of the source PCell is forwarded from the source MN to the candidate MN via the Handover Report message. The candidate MN could use the C-RNTI information to identify the UE context. The combination of UE context and RLF report is sufficient to assist the candidate MN to perform analysis and corresponding adjustment, e.g. whether the execution condition setting for candidate PSCell associated with the fulfilled PCell needs to be optimized. Therefore, we propose:

No additional information is needed to reuse the HANDOVER REPORT message to forward the RLF report to candidate MN.

Except for that NTN NR supports time-based CHO over NG, TN NR does not support NG-based CHO. That means, the source MN and the candidate MN always have Xn connectivity for support of CHO with candidate SCGs. Therefore, we do not see any impact on the Handover Report message over NG.

The last RAN3 meeting agreed on a new message, e.g. SCG Failure Indication message, to convey SCGFailureInformtion to the respective MN(s). Since the UE sends the SCGFailureInformation to the MN immediately after the SCG failure occurs, the new message containing the specific SCG failure information should also be forwarded by the source MN to the respective MN without delaying, which guarantees the UE context kept at the respective MN. In addition to the immediate message, the UE ID should be also included. It could be the C-RNTI assigned by the source PCell or the candidate PCell. If the new message uses the UE-associated signalling, the UE ID could also be the UE XnAP IDs allocated by the source MN and candidate MN.

At last, from our point of view, the respective MN does not need to involve each candidate MN. If RAN2 agrees to indicate the fulfilled PCell ID in SCGFailureInformation, the respective MN is the candidate MN which the fulfilled PCell belongs to.

SCG failure indication should only contain one SCGFailureInformation container (see also the TP in the annex)

Include information that enables context retrieval in SCG failure Indication (see also the TP in the annex)

### 2.1.2	Near failure scenarios

The near failure monitoring for CHO with candidate SCG reuses the existing SHR and SPR mechanism to the maximum extent. To distinguish from the legacy mobility cases and support the analysis of the execution condition delaying or triggering the mobility event, the SHR and SPR report could include the type of first fulfilled execution condition (i.e., PSCell or PCell fulfilled first) and time gapbetween the two types fullfiled (i.e., between PCell and associated PSCell fulfilled). Another benefit of time gap of the two types fullfiled could be an implicit indicator of this new mobility event type, i.e., CHO with candidate SCGs.

The SHR and SPR report could include the type of first fulfilled execution condition and time duration between two fulfilled execution conditions

The SPR may be generated when the UE access to the target MN and target SN successfully. The target MN receives the SPR from the node fetching the SPR and forward to the associated target SN. Besides, the target MN may need to forward the SPR to the source MN for optimizing the mobility parameters associated to the target PCell.

The target MN may need to forward the SPR to the source MN for optimizing the mobility parameters associated to the target PCell.

If both CHO with candidate SCGs and CHO only for the target PCell are configured, CHO only is triggered and executed. The CHO only is naturally more easily triggered than CHO with candidate SCGs, since there is only one type of execution condition to be evaluated for triggering the execution. In our understanding, it is not a near failure case.

CHO only executed is not treated as a near failure case.

For near failure case as below illustrated we assume the legacy mechanism can be re-used to the maximum extent. For near failure in PCell mobility and PSCell mobility, the UE generate SHR and SPR.

In order for the source node to correlate the SHR and SPR reports, the UE should also report the time between the execution of event and the reporting. With this information, the current solution would be able to combine the information from SHR and SPR associated to the same mobility event.

Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR.

To conclude, at the moment it seems that the impact on RAN3 is limited but that information is required form the UE. We propose to send an LS to RAN2 to request the information mentioned above.

For the SHR and SPR caused by CHO with candidate SCGs, the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

## 2.2	MRO for subsequent CPAC

### 2.2.1	Failure scenarios

In last meeting, some companies proposed that UE should report some information in SCGFailureInformation to MN for cause analysis and optimization, e.g., the indication on MN-triggered or SN-triggered, the indication on intial execution or following one, the initiating PSCell ID. Recalling the procedures of MN/SN-initiated S-CPAC in §10.20, TS 37.340, MN is aware of every S-CPAC operation since the UE will anyway send RRCReconfigurationComplete message to MN in either initial or following phases. Hence, the NW will always know such information and there seems no need for UE to report it again in SCGFailureInforamtion.

- Observation 1: The MN is aware of the S-CPAC status, e.g., MN or SN triggered, initial or following execution, initiating PSCell ID, etc.

There is no need for UE to report the above information in SCGFailureInforamtion.

### 2.2.2	Near failure scenarios

For near failure cases, we assume the legacy mechanism and the above enhancements can be re-used.

### 2.2.3	Successful scenarios

Similar as the root causes of the failure, we’d like to address two main aspects related to S-CPAC. The first one is that the expired UE information. As standardized in Rel-18, the target SN(s) receive the SCG UHI from the MN via SN Addition Request message for CPC, which is at the preparation phase of the subsequent CPAC. Hence, for further CPC, the subsequent target SNs are not able to access the latest SCG UHI, which may hinder the timely and precise operations of target SNs, e.g., parameter optimization, resource allocation, etc. Notice that updated SCG UHI may be sent to MN from SN via SN MODIFICATION REQUEST ACKNOWLEDGE message when UE leaves this SN in S-CPAC, i.e., MN is already aware of the latest SCG UHI. Thus, MN is responsible to send such information to the candidate SNs if needed. Symmetrically, MN can use SN MODIFICATION REQUEST message to inform the candidate SNs about SCG UHI. On the other hand, we can leave it up to the MN to decide when to update the information, i.e. there is no mandate to always update this information.

Add UHI to SN MODIFICATION REQUEST and add stage2 text to describe this (see also the TP in the annex)

Another similar information exchange that may be beneficial is that the initiating node may be informed of the outcome of the S-CPAC configuration. S-CPAC requires reservation of resources in advance. The initiating node (MN or SN) could evaluate the selection of candidate cells and possibly the time spent in each cell to evaluate if S-CPAC was beneficial or not.

In case of MN-initiated S-CPAC, we believe the MN already can get this information, since MN is already responsible for collecting the UHI. But in the case of SN initiated S-CPAC, the initiating SN may not be aware of what is happening to this UE, since SN is not responsible for collecting this information. Therefore the MN has to provide this to the SN.

The MN could be informed also about intra SN changes since the MN:

- can get the updated UHI after the UE leaves the SN, or

- can subscribe to UHI updates from the SN.

Note that SN MODIFICATION REQUEST is sent from the MN to the SN if this SN is configured as a candidate SN, as stated in §10.20 in TS 37.340. Otherwise, the SN release procedure will be initiated. Thus, in this case, the updated UHI should be carried in the corresponding messages from the MN to SN, i.e., SN RELEASE REQUEST or SN RELEASE CONFIRM.

RAN3 agree that it is beneficial to inform the initiating SN about the outcome of mobility events for SN initiated S-CPAC  using the SN RELEASE REQUEST or SN RELEASE CONFIRM (see also the TP in the annex).

# 3	Conclusion

This document proposes:

Proposal 1:	No additional information is needed to reuse the HANDOVER REPORT message to forward the RLF report to candidate MN.

Proposal 2:	Except for that NTN NR supports time-based CHO over NG, TN NR does not support NG-based CHO. That means, the source MN and the candidate MN always have Xn connectivity for support of CHO with candidate SCGs. Therefore, we do not see any impact on the Handover Report message over NG.

Proposal 3:	SCG failure indication should only contain one SCGFailureInformation container (see also the TP in the annex)

Proposal 4:	Include information that enables context retrieval in SCG failure Indication (see also the TP in the annex)

Proposal 5:	The SHR and SPR report could include the type of first fulfilled execution condition and time duration between two fulfilled execution conditions

Proposal 6:	The target MN may need to forward the SPR to the source MN for optimizing the mobility parameters associated to the target PCell.

Proposal 7:	CHO only executed is not treated as a near failure case.

Proposal 8:	Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR.

Proposal 9:	the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

Observation 1: The MN is aware of the S-CPAC status, e.g., MN or SN triggered, initial or following execution, initiating PSCell ID, etc.

Proposal 10:	There is no need for UE to report the above information in SCGFailureInforamtion.

Proposal 11:	Add UHI to SN MODIFICATION REQUEST and add stage2 text to describe this (see also the TP in the annex)

Proposal 12:	RAN3 agree that it is beneficial to inform the initiating SN about the outcome of mobility events for SN initiated S-CPAC  using the SN RELEASE REQUEST or SN RELEASE CONFIRM (see also the TP in the annex).

# Annex	TP for CHO with candidate SCGs for 37.340

## 10.X	Self-optimisation for CHO with candidate SCGs

The procedure follows the steps described for Mobility Robustness Optimization in 38.300 [3] and Self-optimisation for PSCell addition/change in clause 10.18.3.

Handling multiple reports from a single CHO with candidate SCGs event

In case both the execution towards the target PCell and the execution towards the associated target PSCell succeed, MRO evaluation of sub-optimal conditional PCell execution and condition PSCell execution may be triggered simultaneously for the same mobility event. In this case, the corresponding SHR and SPR should be correlated.

# Annex	TP for CHO with candidate SCGs for 38.423

#### 9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source MN. | YES | ignore |
| Candidate S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate MN. | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |

Editor’s note: FFS is this message will contain only one SCGFailureInformation message or a list of SCGFailureInformation messages.

# Annex	TP for S-CPAC for 37.340

## 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in the cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN.

For S-CPAC the MN may send an updated UE History Information to the initiating SN (if applicable) using the UE S-NG-RAN node initiated S-NG-RAN node Release procedure or the M-NG-RAN node initiated S-NG-RAN node Release procedure. The MN may also send it to candidate SN(s) using the MN initiated SN Modification procedure.

# Annex	TP for S-CPAC for 38.423

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

[snip]

If the PSCell History Information Retrieve IE set to "query" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information as specified in TS 37.340 [8].

If the UE History Information from the UE IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the UE History Information IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the SCG UE History Information IE is included in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

### 8.3.6	M-NG-RAN node initiated S-NG-RAN node Release

#### 8.3.6.1	General

The M-NG-RAN node initiated S-NG-RAN node Release procedure is triggered by the M-NG-RAN node to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: M-NG-RAN node initiated S-NG-RAN node Release, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUEST message. Upon reception of the S-NODE RELEASE REQUEST message the S-NG-RAN node shall stop providing user data to the UE.

[snip]

If the S-NODE RELEASE REQUEST message contains a PDU session resource to be released which is configured with the SCG bearer option within the PDU Session Resources To Be Released List IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the S-NODE RELEASE REQUEST ACKNOWLEDGE message. The RLC Mode IE indicates the RLC mode used in the S-NG-RAN node for the DRB.

If the S-NODE RELEASE REQUEST message contains the UE History Information IE, the S-NG-RAN node shall, if supported, store this information.

[snip]

### 8.3.7	S-NG-RAN node initiated S-NG-RAN node Release

#### 8.3.7.1	General

This procedure is triggered by the S-NG-RAN node to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: S-NG-RAN node initiated S-NG-RAN node Release, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUIRED message to the M-NG-RAN node.

[snip]

If the S-NODE RELEASE REQUIRED message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the S-NODE RELEASE REQUIRED message contains the User Plane Error Indicator IE within the PDU Sessions List To be Released - UPError IE set to "GTP-U Error Indication Received", the M-NG-RAN node shall, if supported, consider that the PDU session is released due to GTP-U Error Indication received over the NG-U tunnel, and inform the SMF as specified in 3GPP TS 23.527 [57].

If the S-NODE RELEASE CONFIRM message contains the UE History Information IE, the S-NG-RAN node shall, if supported, store this information.

### 9.1.2	Messages for Dual Connectivity Procedures

[snip]

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| ... |  |  |  |  |  |  |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

#### 9.1.2.14	S-NODE RELEASE REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the release of resources.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session Resources To Be Released List | O |  | PDU Session List with Cause 9.2.1.26 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | Indicates that the target M-NG-RAN node reconfigured the listed DRBs as MN-terminated bearers. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.18	S-NODE RELEASE CONFIRM

This message is sent by the M-NG-RAN node to confirm the release of all resources for a specific UE at the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session Resources Released |  | 0..1 |  |  | YES | ignore |
| >PDU sessions released List – SN terminated | O |  | PDU Session List with data forwarding info from the target node 9.2.1.25 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

[snip]

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST

--

-- **************************************************************

SNodeModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequest-IEs}},

...

}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause												PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication							PRESENCE optional }|

{ ID id-selectedPLMN							CRITICALITY ignore		TYPE PLMN-Identity										PRESENCE optional }|

{ ID id-MobilityRestrictionList					CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-SCGConfigurationQuery					CRITICALITY ignore		TYPE SCGConfigurationQuery							PRESENCE optional }|

{ ID id-UEContextInfo-SNModRequest				CRITICALITY reject		TYPE UEContextInfo-SNModRequest					PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING										PRESENCE optional }|

{ ID id-requestedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-requestedSplitSRBrelease				CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore		TYPE DesiredActNotificationLevel					PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List											PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL			CRITICALITY reject		TYPE BitRate											PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL			CRITICALITY reject		TYPE BitRate											PRESENCE optional }|

{ ID id-LocationInformationSNReporting			CRITICALITY ignore		TYPE LocationInformationSNReporting				PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo				PRESENCE optional }|

{ ID id-PCellID									CRITICALITY reject		TYPE GlobalNG-RANCell-ID								PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern						CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3				PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3Release	CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3Release	PRESENCE optional }|

{ ID id-SNTriggered								CRITICALITY ignore		TYPE SNTriggered										PRESENCE optional }|

{ ID id-TargetNodeID							CRITICALITY ignore		TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-PSCellHistoryInformationRetrieve		CRITICALITY ignore		TYPE PSCellHistoryInformationRetrieve			PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE			CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-CHOinformation-ModReq					CRITICALITY ignore		TYPE CHOinformation-ModReq							PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationModReq					CRITICALITY ignore		TYPE CPAInformationModReq							PRESENCE optional }|

{ ID id-CPCInformationUpdate					CRITICALITY ignore		TYPE CPCInformationUpdate							PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY ignore		TYPE UESliceMaximumBitRateList						PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList	CRITICALITY	ignore		TYPE MDTPLMNModificationList						PRESENCE optional }|

{ ID id-SelectedNID								CRITICALITY	ignore		TYPE NID												PRESENCE optional }|

{ ID id-QMCCoordinationRequest					CRITICALITY ignore		TYPE QMCCoordinationRequest							PRESENCE optional }|

{ ID id-Src-SN-to-Tgt-SNQMCInfoInquiry			CRITICALITY ignore		TYPE Src-SN-to-Tgt-SNQMCInfoInquiry				PRESENCE optional }|

{ ID id-IABAuthorizationStatus					CRITICALITY ignore	TYPE IABAuthorizationStatus							PRESENCE optional }|

{ ID id-UEHistoryInformation					CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional },

...

}

[snip]

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST

--

-- **************************************************************

SNodeReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequest-IEs}},

...

}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleased-RelReq			CRITICALITY ignore		TYPE PDUSession-List-withCause					PRESENCE optional }|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore		TYPE DRB-List										PRESENCE optional }|

{ ID id-UEHistoryInformation					CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional },

...

}

[snip]

-- **************************************************************

--

#### -- S-NODE RELEASE CONFIRM

--

-- **************************************************************

SNodeReleaseConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseConfirm-IEs}},

...

}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionReleasedList-RelConf			CRITICALITY ignore		TYPE PDUSessionReleasedList-RelConf			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-UEHistoryInformation					CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional },

...

}

PDUSessionReleasedList-RelConf ::= SEQUENCE {

pduSessionsReleasedList-SNterminated		PDUSession-List-withDataForwardingFromTarget					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionReleasedList-RelConf-ExtIEs} }	OPTIONAL,

...

}

PDUSessionReleasedList-RelConf-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

# Annex	TP for LS to RAN2

3GPP TSG-RAN WG3 Meeting #127-bis	TDoc <TDoc#>

Wuhan, China, 7-11 April, 2025

Title:	[DRAFT] LS on SON and MDT enhancements for MRO

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person: 	Henrik Olofsson

henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 would like to inform RAN2 about the latest progress in MRO for SON/MDT.

MRO for LTM

- Beam management: RAN3 agreed to continue working on a solution for the failure and near failure scenario without any RAN2 impact.

- TA acquisition: RAN3 agree to continue working on solutions for failure and near failure scenarios without any RAN2 impact

MRO for CHO with candidate SCGs

- RAN3 agreed to the benefit of including the type of first fulfilled execution condition and time duration between two fulfilled execution conditions in the SHR and SPR report.

- RAN3 agreed to the benefit of including the time between the execution of the event and the reporting in the SHR/SPR in order to enable correlation of SHR and SPR from the same event

MRO for subsequent CPAC

- RAN3 agreed did not yet identify any solution with RAN2 impact.

# 2	Actions

To RAN2,

ACTION: 	RAN3 kindly asks RAN2 to take above progress into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#128	2025-05-19 - 2025-05-23		Malta, MT

RAN3#129	2025-08-25 - 2025-08-29		Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17		Prague, CZ

RAN3#130	2025-11-17 - 2025-11-21		Dallas, US