3GPP TSG-RAN WG3 #127bis								R3-251969

Wuhan, China, 7th – 11st April 2025

Agenda Item:	9.2

Source:	China Telecom

Title: 	Discussion on Cell Information for ATG and UAV

Document for:	Discussion

Introduction

In the RAN3#127 meeting, [1] proposed the introduction of cell-specific information, including the cell type and corresponding configuration details related to ATG and UAV, for SON purposes. During the online discussion, some companies expressed a preference to address this issue through an OAM-based solution, and the RAN3 Chair encouraged companies to conduct further investigation on this matter. Therefore, this paper aims to further examine the necessary cell-specific information to be exchanged over the Xn, F1, and X2 interfaces

Discussion

OAM based Solution or Signaling based Solution

In the previous RAN3 meetings held in Athens, some companies expressed a preference to address the issues raised in [1] through an OAM-based solution. Additionally, we reviewed the SA5-related specifications after the meetings and found that no ATG/UAV-related attributes are specified in the cell relation and frequency relation. From our perspective, the proposed information to be exchanged between two nodes is at the cell level. Therefore, a signalling-based solution appears to be more efficient than an OAM-based solution. In the RAN2#129 meeting, RAN2 discussed a similar scenario involving HSDN cells. In this scenario, some cells support HSDN, while others do not [2]. During the online discussion in RAN2, the applicability of an OAM-based solution to the scenario raised by operators was also considered. Operators and the majority of vendors pointed out that an OAM-based solution imposes a burden on the network. Therefore, we believe that a signalling-based solution is a more efficient approach to address these issues and reduce OPEX.

Proposal 1: to support signaling based solution to solve the issues of cell information exchange for HSDN, ATG and UAV.

ATG cell information

[1] and [3] proposed the introduction of new ATG cell information, including a new cell type and gNB location parameters, over the Xn interface. During the online discussion, some companies expressed the view that the concept of an ATG cell is not specified in RAN2. However, the following excerpts from TS38.300 and TS38.331 indicate that the term "ATG cell" is already used in the RAN2 specifications:

- Clause 16.19.2.2 of TS38.300

For the serving cell, the network broadcasts coarse gNB location information. The UE shall have valid GNSS position as well as gNB location before connecting to an ATG cell. To achieve synchronisation, before and during connection to an ATG cell, the UE shall compute the RTT between UE and the gNB based on the GNSS position and the gNB location parameters, and autonomously pre-compensate the TTA for the RTT between the UE and the gNB.

In connected mode, the UE shall be able to continuously update the Timing Advance.

The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. For an RRC_CONNECTED UE, event-triggered reporting of the Timing Advance is supported.

- TS38.331

clause 5.2.2.4.1	Actions upon reception of the MIB

NOTE 1:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

NOTE 2:	A UE capable of ATG access should acquire SIB1 to determine whether the cell is an ATG cell.

Clause 6.3.2

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency, in TS 38.101-1 [15] and in TS 38.101-5 [75], and is used to calculate the parameter Pcompensation defined in TS 38.304 [20]. In ATG cell, actual value of P-Max = 9 + field value [dBm].

si-BroadcastStatus: Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting.

Based on the above excerpts, we understand that the term "ATG cell" also needs to be introduced in the RAN3 specification. With respect to the ATG cell configuration, the following information is included in SIB22:

ATG-NeighCellConfig-r18 ::=           SEQUENCE {

atg-gNB-Location-r18                  ReferenceLocation-r17                              OPTIONAL,     -- Need R

height-gNB-r18                         INTEGER (-16384..16383)                            OPTIONAL,     -- Need R

carrierFreq-r18                       ARFCN-ValueNR                                      OPTIONAL,     -- Need R

physCellId-r18                        PhysCellId                                         OPTIONAL      -- Need R

}

The highlighted ATG cell configuration information, including atg-gNB-Location-r18 and height-gNB-r18, can be conveyed to neighbouring nodes. Upon receiving the ATG cell configuration information, the target node can identify that the source node supports the ATG feature. However, whether the concept of an ATG cell was agreed upon in RAN2 remains controversial. Therefore, we propose not to introduce a new explicit cell indicator for ATG over the Xn interface, but instead to introduce new ATG configuration information, including atg-gNB-Location-r18 and height-gNB-r18, over the Xn and F1 interfaces.

Proposal 2: to introduce a new ATG configuration information including atg-gNB-Location-r18 and height-gNB-r18 over Xn and F1 interface.

UAV cell

A new MultiFrequencyBandListNR-Aerial-SIB-r18 IE is specified in the FrequencyInfoUL-SIB IE and FrequencyInfoDL-SIB IE, which provide the basic parameters of uplink and downlink carriers and transmissions in an NR cell. Additionally, the additionalPmax and additionalSpectrumEmission parameters in the MultiFrequencyBandListNR-Aerial-SIB-r18 IE are cell-specific.The following information is quoted from TS38.331:

FrequencyInfoUL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB                            OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                 OPTIONAL    -- Need S

]]

}

MultiFrequencyBandListNR-Aerial-SIB-r18 ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfoAerial-r18

NR-MultiBandInfoAerial-r18 ::=              SEQUENCE {

freqBandIndicatorNR-Aerial-r18              FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxListAerial-r18                    NR-NS-PmaxListAerial-r18    OPTIONAL    -- Need S

}

NR-NS-PmaxValueAerial-r18 ::=           SEQUENCE {

additionalPmax-r18                      P-Max                               OPTIONAL,   -- Need N

additionalSpectrumEmission-r18          AdditionalSpectrumEmission-r18

}

Moreover, the MultiFrequencyBandListNR-Aerial-SIB-r18 IE is also included in SIB4. Therefore, it is necessary to exchange the additionalPmax and additionalSpectrumEmission parameters between nodes. With this information, the target node can determine whether the source node supports the UAV feature and can generate the corresponding information for SIB4. The UAV feature is supported in both LTE and NR. According to TS38.331, similar information is also specified on the LTE side.

EUTRA-MultiBandInfoAerial-r18 ::=         SEQUENCE {

eutra-FreqBandIndicator-r18               FreqBandIndicatorEUTRA,

eutra-NS-PmaxListAerial-r18               EUTRA-NS-PmaxList                 OPTIONAL    -- Need R

}

Proposal 3: to introduce new additionalPmax IE and additionalSpectrumEmission IE in uplink frequency configuration over X2/Xn/F1 interace.

Based on the above analysis, we kindly ask RAN3 to agree the corresponding CRs [3][4].

Proposal 4: to agree the corresponding CRs [3-6] for Xn/F1/X2 interface.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: to support signaling based solution to solve the issues of cell information exchange for HSDN, ATG and UAV.

Proposal 2: to introduce a new ATG configuration information including atg-gNB-Location-r18 and height-gNB-r18 over Xn and F1 interface.

Proposal 3: it is need to introduce a new cell type indicator and corresponding nr-NS-PmaxListAerial for one UAV cell.

Proposal 4: to agree the corresponding CRs [3-6] for Xn/F1/X2 interface.

Reference

- R3-250389, Discussion on Cell Type Information for UAV and ATG

- R2-2501113, Discussion on the issue of ANR reporting of HSDN cells [ANR_HSDN], Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo

- R3-251970  Correction on ATG information over Xn interface

- R3-251973  Correction on UAV information over F1 interface

- R3-251974  Correction on UAV information over Xn interface

- R3-25xxxx  Correction on UAV information over X2 interface