3GPP TSG-RAN WG3 Meeting #R3-127bis		R3-251972 Wuhan, China, April 7 - 11, 2025

Title: 	gNB-DU initiated LTM candidate cell reconfiguration

Source: 	Rakuten, Qualcomm Inc

Agenda item:	13.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss the resource management issues associated with configuration and maintenance of multiple candidate cells, the impacts on gNB-DU and the importance of gNB-DU initiated reconfiguration of LTM target cell configuration.

# 2. Discussion

## 2.1 Resource management for multiple LTM target cells

The lead time (duration between LTM candidate cell preparation and execution) during an LTM cell switch could be unpredictably long depending on mobility scenario. Secondly, not all prepared candidate cell configurations end up in LTM cell switches.

Radio resources are scarce and considering that a UE can be configured with up to 8 LTM candidate cells, resource reservation is always going to be an issue. Reservation of radio resources in all the configured LTM candidate cells for a UE leads to sub-optimal performance and network overload. Especially considering that this is applicable on a large scale, for multiple UEs configured with LTM in the network.

Rel18 LTM requires the UE to perform UL sync to acquire timing advance from the candidate/target cell before the serving cell switch with the objective of performing a RACH-less LTM cell switch and therefore reducing HO latency and  service interruption.

Based on the UE’s measurements, the serving gNB-DU may ask the UE to perform early UL sync with the candidate cell to acquire UE’s candidate cell timing advance. Hence, UE performing early UL sync with the candidate cell can be considered as an indication of a likelihood of UE undergoing LTM cell switch to the given candidate cell.

To address the resource reservation issues, the following could be considered - at the time of preparing an LTM candidate cell, the source gNB-CU could indicate to the candidate gNB-CU and the candidate gNB-CU could further indicate to the candidate gNB-DU whether or not the resource reservation can be delayed. Depending on on this, the target gNB-DU can decide when to reserve resources, at the time of LTM candidate cell preparation or at a later point, for example when the UE performs early UL sync.

Based on the UE performing early TA acquisition with a candidate cell, the candidate/target gNB-DU can anticipate an incoming LTM cell switch and initiate resource reservation at the candidate/target cell including a potential update of RACH resource configuration corresponding to the UE’s best beam/beam-group at that point of time.

This introduces multiple benefits

- This ensures that the resource reservation at the target gNB-DU is performed very close to the LTM serving cell switch and hence optimized, as the likelihood of an LTM cell switch is strong. This also prevents unnecessary blocking of resources for a longer duration in case of long lead times.

- This also ensures that the right RACH resources corresponding to the best beam/beam-group are configured at the candidate/target gNB-DU.

- The UE performing early UL sync can be a good indicator of the likelihood of a LTM cell switch. This enables the candidate/target gNB-DU to issue a target cell re-configuration (if required) based on the prevalent resource conditions at that time.

The resource reservation changes may result in a modified candidate/target cell configuration at the candidate/target gNB-DU and could be delivered to the UE via the gNB-CU using a gNB-DU initiated F1: UE Context Modification Required procedure.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

However, in Rel18, gNB-DU initiated LTM candidate cell re-configuration is not permitted by RAN3. The same has been adopted as a baseline in Rel 19 also. This is illustrated using the figure below

Figure 1: Signaling flow illustrating gNB-DU initiated candidate cell reconfiguration

This is a limitation considering the fact that there could be changes in resource status and availability between LTM candidate cell preparation and execution. As an example, a simple RACH preamble modification based on the best beam or beam-group change isn’t possible after the candidate cell configuration has been prepared. Once the candidate cell configuration is prepared, it is either executed or cancelled. This, in our view, is too inflexible and limiting genuine cases requiring re-configuration.

NOTE: The proposed changes are completely limited to gNB and doesn’t have any UE impacts.

Proposal 1: During the candidate cell preparation, the candidate/target gNB-DU is informed if a delayed resource reservation is permitted for the given UE.

Proposal 2: RAN3 discuss gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the candidate/target gNB-DU.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

Proposal 1: During the candidate cell preparation, the candidate/target gNB-DU is informed if a delayed resource reservation is permitted for the given UE

Proposal 2: RAN3 discuss gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the candidate/target gNB-DU.

# 4. Reference

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”