3GPP TSG-RAN WG3 Meeting #127-bis	R3-251975

# Wuhan, China, 7th – 11th April, 2025

Title: 	On collection of throughput measurements for UE performance

Source: 	Ericsson, Nokia, JIO Platforms

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

During RAN3#127, the following discussion point was captured:

For E1 interface:

CU-UP provides the UE performance feedback (UL/DL throughput, delay, packet loss) over E1 to CU-CP?

This paper focuses on the above, primarily discussing the collection and signalling of Throughput as part of UE performance reporting in split architecture.

# 2. Discussion

### 2.1 Average UE Throughput in UE performance

During Release 18, RAN3 formalised the Data Collection Reporting Initiation and the Data Collection Reporting procedures for non-split architectures. One of the intended use-cases for the procedures was UE performance collection, where information/feedback could be collected by the source node from a neighbour node to which UEs were handed over to. The defined UE performance metrics are Average UE Throughput UL, Average UE Throughput DL, Average Packet Delay, and Average Packet Loss DL.

In the context of throughput measurements, it is important to note that the throughput metrics defined during Release 18 had the following definition (TS 38.423, clause 9.2.3.4)- ‘This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL or by the UE in sidelink within a period of time, divided by the duration of the period.’.

Observation 1: The Release 18 metrics for Average UE Throughput were not defined at any particular level, e.g., PDCP, RLC.

However, 3GPP TS 28.558, which defines UE level measurements for 5GS, defines Average UL/DL UE throughput in gNB as a ratio of successfully delivered RLC SDU over time, and that it is computed at the gNB-DU, i.e., RLC level throughput. In the context of split architecture discussions in Release 19, it has emerged that the level at which the metrics are measured has an impact on the specifications, i.e., measuring at the gNB-DU (RLC level) or measuring at the PDCP-level (gNB-CU-UP) has different standards impact, as noted in the SoD in R3-250790.

In the general context of throughput measurements, any performance measure computed at a higher layer is generally considered closer to the application-level or service-level perceived performance. Measuring throughput at a lower layer has the drawback that it may not consider higher-layer interactions in the measure. For example, a high throughput at a lower layer may not necessarily translate to a similarly higher layer throughput when reordering, timing, and higher-layer losses occur. The above phenomenon is fairly common in protocol stacks and a higher layer metric follows the service-level performance better than a lower layer metric.

Observation 2: When possible, it is better to choose a higher-layer measurement as it follows the service- or application-level performance better than a lower-layer measurement.

In the context of UE performance, the source gNB is interested in obtaining data (to be used either as training, or feedback data) that clearly describes the service level performance experienced by a UE after a handover event. Based on the reasoning described above, obtaining measurements at a lower layer does not always unequivocally express the service- or application-level performance experienced by the UE.

Proposal 1: Throughput measurements at PDCP level are better than throughput measurements at RLC level to better represent the actual performance at the UE.

As pointed out earlier, 3GPP TS 28.558 and other specifications defining UE measurements define no PDCP level UE throughput measurements. However, 3GPP TS 37.483 (E1 AP) defines the following IEs and corresponding functionality as part of the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION request messages:

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501.

If the UE Slice Maximum Bit Rate List IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store and use the information for the downlink traffic policing for each concerned slice as specified in TS 23.501.

If the UE Slice Maximum Bit Rate List IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store and replace the previously provided UE Slice Maximum Bit Rate List by the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate List for the downlink traffic policing for each concerned slice as specified in TS 23.501.

It is evident from the above that a gNB-CU-UP must measure the UE throughput at the PDCP level and for the DRBs corresponding to the slice for which the Maximum Bitrate was specified. However, it needs to be noted that no specific PDCP measurements have been defined to make such functionality work.

Observation 3: There is already a requirement on the gNB-CU-UP to measure PDCP throughput without explicit definition of such measurements in the specifications.

Based on the above findings, for the problem at hand, i.e., throughput measurements for UE performance, it is not necessary that PDCP-level throughput measurements shall be specified for this purpose, as in practice the gNB-CU-UP can collect these measurements using existing functionality.

Proposal 2: RAN3 to agree that the gNB-CU-UP can compute the UL and DL PDCP throughput for a UE. This functionality is considered already available.

For the purpose of defining UE throughput for UE performance, throughput at PDCP level may be computed per DRB  for UL or DL. The  Average per DRB UE throughput UL and Average per DRB UE throughput DL may then be signalled (per DRB) by the gNB-CU-UP to the gNB-CU-CP.

Proposal 3: The average PDCP throughput per DRB in UL and DL is considered as the measured throughput for UE Performance at the gNB-CU-UP.

As for the choice of procedure to use to exchange throughput and potentially other UE performance metrics from the gNB-CU-UP to the gNB-CU-CP, there exists two options:

- Extend the Resource Status Reporting procedures with UE performance metrics

- Introduce Data Collection Reporting procedures over E1 interface solely for the purpose of exchanging UE performance metrics.

Introducing a completely new procedure to exchange measured metrics, when existing procedures can already do that, is wasteful. For example, the existing Data Collection Reporting procedures on XN are used for mobility optimization, load balancing, and energy saving use cases, with the possibility to request and report measured and inferred metrics to be applied in different AI/ML-based applications in the network. When comparing this to the use case of exchanging UE performance metrics over a new procedure, the disparity is considerable. Signalling UE Performance metrics does not justify the introduction of a new procedure.

Currently over the E1 interface, there already exists Resource Status Reporting procedure which is used by a gNB-CU to request from the gNB-CU-UP reporting of measurements which may be used for various purposes, e.g. load balancing, resource planning, and possible use of the exchanged metrics as AI/ML inference inputs or feedback information. From this perspective, it seems that extending the Resource Status Reporting procedure with UE performance metrics seems a viable option that does not violate the principles of the procedure.

Proposal 4: Extend E1 AP Resource Status Reporting procedure with UE performance metrics.

A TP reflecting the above proposals is available in R3-251980.

2.2 Signalling of Energy Cost from gNB-DU to gNB-CU-CP

In the context of Energy Cost, RAN3 made the following agreement at RAN3#125-bis, in the context of transmission of Energy Cost from the DU to the CU:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

During RAN3#127, the following agreements were reached:

- Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

- Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

From the above, it is clear that the only metric to be exchanged directly from the gNB-DU to the gNB-CU-CP is the Energy cost. Similarly to what argued above, introducing a completely new procedure to exchange a single measured metric, when existing procedures can already do that, is wasteful and only adds complexity to the interface without there being an actual need. The needed functionality should be implemented with the least disruption to the specifications. Based on the above, we propose the following:

Proposal 5: RAN3 to revert the earlier agreement to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 for the transfer of measured EC from DU to the CU.

Proposal 6: RAN3 to extend the Resource Status Reporting procedure on F1 to exchange the Energy cost from the DU to the CU.

A TP reflecting the above proposals is available in R3-251980.

# 3. Conclusion

# This paper makes the following observations and proposals:

Observation 1: The Release 18 metrics for Average UE Throughput were not defined at any particular level, e.g., PDCP, RLC.

Observation 2: When possible, it is better to choose a higher-layer measurement as it follows the service- or application-level performance better than a lower-layer measurement.

Proposal 1: Throughput measurements at PDCP level are better than throughput measurements at RLC level to better represent the actual performance at the UE.

Observation 3: There is already a requirement on the gNB-CU-UP to measure PDCP throughput without explicit definition of such measurements in the specifications.

Proposal 2: RAN3 to agree that the gNB-CU-UP can compute the UL and DL PDCP throughput for a UE. This functionality is considered already available.

Proposal 3: The average PDCP throughput per DRB in UL and DL is considered as the measured throughput for UE Performance at the gNB-CU-UP.Proposal 4: Extend E1 AP Resource Status Reporting procedure with UE performance metrics.

Proposal 5: RAN3 to revert the earlier agreement to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 for the transfer of measured EC from DU to the CU.

Proposal 6: RAN3 to extend the Resource Status Reporting procedure on F1 to exchange the Energy cost from the DU to the CU.