3GPP TSG-RAN WG3 Meeting #127-bis	R3-251984

Wuhan, China, 7-11 April 2025

Title: 	On packet loss metric in UE performance

Source: 	Ericsson, JIO Platforms, InterDigital

Agenda item:	9.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss aspects related to the Packet Loss metric, which is part of UE performance measurements. The presented discussion is applicable to both split and non-split architecture deployments. Although RAN3 in Release-19 is discussing split architecture as a Release-18 leftover, i.e., specify functionality to bring parity between split and non-split architecture, we show in this paper that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric even based on the Release-18 specification. In the discussion below we bring forward the different shortcomings and propose on a way forward for Release-18, which can be extended also toRelease-19.

# 2. Discussion

Obtaining feedback to actions derived via AI/ML is an important step in bridging the gap between an AI/ML model and the real-world system. In the context of AI/ML based use-cases such as mobility optimization, RAN3 specified a mechanism to collect UE performance after an AI/ML derived mobility action. The UE performance metrics include Average UE throughput DL, Average UE throughput UL, Average Packet Delay UL, Average packet delay DL, and Average Packet loss DL, which is collected by a target gNB after handover and is signalled back to the source gNB upon fulfilment of specified exit conditions.

The source gNB, after receiving the metrics in the UE performance, may take into account the performance at the target cell and use it to tune AI/ML derived mobility actions, e.g. prioritizing actions that maximize UE performance. Such process may lead to model re-training/updating. UE performance may also be used to collect input data for model training where feedback for handed over UEs is requested without the action being derived using AI/ML models.

## Issues with the Packet Loss metric

## Issues with the existing definitions

Packet Loss for UE performance is defined as follows in TS 38.423.

#### 9.2.3.11 Packet Loss Rate This IE indicates the Packet Loss Rate for a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Packet Loss Rate; M; INTEGER (0..1000, ...); Ratio of lost packets per number of packets sent, expressed in tenth of percent..

Based on the above, the specified range for the packet loss rate is an integer between 0 to 1000 with steps of 0.1%.

It needs to be highlighted that the IE above was first introduced as part of the  GBR QoS Flow Information IE (TS 38.423, clause 9.2.3.6). This IE indicates the QoS Parameters for a GBR QoS Flow for downlink and uplink, namely the QoS targets. Packet Loss Rate in this IE provides an indication of the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. It is at this point important to understand that this parameter was not introduce to measure Packet Loss Rate, but to indicate a maximum upper limit for such metric.

Observation 1: The Packet Loss Rate IE has been introduced to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric. The Packet Loss Rate IE in Rel18 has been reused to measure UL/DL Packet Loss Rate for UE Performance exposure purposes.

The essence of the above observation is that reusing the Packet Loss Rate IE to indicated measurements of an UL/DL Packet Loss Rate is semantically incorrect. Any potential change to the Packet Loss Rate IE will affect the meaning and intended use of both UE performance and GBR QoS Flow Information IEs.

Next, we note the definition of the Packet Loss Rate IE, which states the following: This IE indicates the Packet Loss Rate for a QoS flow. Such description made sense when the Packet Loss Rate IE was introduced to indicate a maximum upper limit per QoS Flow for the UL/DL Packet Loss Rate. However, this definition does not make sense when the IE is used to represent a measured UE Performance parameter. UE performance, as the name suggests, is at a per-UE granularity. The mismatch between the two granularities is obvious, i.e., per QoS flow vs per UE. Therefore, it is obvious that the definition of the Packet Loss Rate IE is inappropriate when using this IE for UE Performance Feedback measurements.

Observation 2: There exists a mismatch in the definition of the Average Packet Loss DL IE to its underlying IE’s definition (Packet Loss Rate IE) in relation to its intended use-case as a per-UE performance metric.

Finally, the Packet Loss Rate IE is defined as an integer with a range from 0 to 1000, and each step indicates a 0.1% change in the ratio of lost packets to the number of packets sent. In NR, there exist several different 5QIs, which translate to a certain QoS offered to the data stream (available in TS 23.501). The range of packet loss rates guaranteed by the different 5QIs ranges from 10-2 to 10-8, i.e., 0.01% to 0.00000001%. The Packet Loss Rate IE in its existing form cannot express packet loss that is less than 10-1. Therefore, the Average Packet Loss DL IE, in its current standardised form, would not even be able to represent the measured packet loss rate in a real network (as the value range of the IE is too coarse). Even if the Average Packet Loss DL IE is used in its current form, any collected data will only show 0 or 1 (10-1 or 10-2). Therefore, the Average Packet Loss DL IE introduced in TS38.423 is of no use and ignoring this IE would bring no impact to the overall functionality.

Observation 3: the Average Packet Loss DL IE introduced in TS38.423 for UE performance feedback monitoring has an accuracy of 0.1%, which is far above a typical packet loss rate in a real network.

## Other issues

In the general context of UE performance reporting for packet losses, a value derived at a higher protocol layer, i.e., the measurements on the PDCP level packet loss rate, is considered more relevant as it better reflects the performance at service-level. This is also better because it takes into account losses over the interfaces such as F1. Indeed, the packet loss at e.g. MAC level does not necessarily carry information about the quality of the service because a dropped MAC PDU may end up being successfully retransmitted with no negative effects on the service.

Although PDCP level packet loss measurements are a better choice, there are drawbacks with those too. For a DRB delivered over RLC Unacknowledged mode, a PDCP packet loss metric as defined above, cannot be computed due to the fact that the PDCP entity cannot keep track of packets that were acknowledged in RLC UM but can only keep track of packets submitted to lower layers for transmission. Therefore, the PDCP entity does not know whether a RLC UM packet has been successfully delivered or not. On the contrary, it is possible to compute packet ross rate under RLC unacknowledged mode at lower layers, i.e., at the RLC entity in the gNB-DU. However, as explained above, the service level performance is not necessarily reflected by packet loss at RLC layer.

Observation 4: There seem to be drawbacks associated with measuring packet loss as a UE performance metric at both PDCP and RLC level

Ro understand what is defined in 3GPP specifications, TS 28.552 and TS28.558 define the following measurements for packet loss rate:

- DL Packet Loss rate on Uu: This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface.

- For split gNBs only - UL PDCP SDU Loss Rate: This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface.

Observation 5: RLC Level DL Packet Loss rate on Uu is valid for all types of gNB deployments (split or non-split) and it is only available in DL, while the UL PDCP SDU Loss Rate is valid only for split gNBs and it is not available in DL.

Proposal 1: RAN3 to acknowledge the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

- Drawbacks exist with measuring packet loss as a UE performance metrics at either PDCP or RLC level

- The defined packet loss metrics are not suitably available for all deployment options (split/non-split RAN) and traffic directions (UL/DL)

## Solutions

There are two broad categories of solution available for the aforementioned issues with packet loss as a UE performance metric.

The first category of solutions postulates that fixing the shortfalls to enable measurements of the packet loss rate in all use cases implies big impacts to the standard and in the end, it does not deliver information that is crucial to understand whether a mobility target is good or bad. As a preferred direction, we propose:

Proposal 2: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

The second category of solutions postulates that a suitable trade-off in terms of cost/standard impact can be reached by introducing a new measurement for packet loss in UE performance and decoupling it from the existing Packet Loss Rate measurement in TS 38.423. In this case, such a metric should be granular enough to represent real-world expected packet loss rates and be representative of the application-level or service-level performance. Based on the discussion in this paper, such a metric should be located at the PDCP level. As ways forward for this solution, RAN3 needs to agree to and execute on the following:

Proposal 4:  Send an LS to SA5 and RAN2 to define a DL PDCP SDU Loss Rate metric. This metric shall also be available for non-split gNBs.

Proposal 5: Redefine the Release-18 definition of Average Packet Loss DL IE, by decoupling it from Packet Loss Rate IE.

Note that to implement the proposals above, the first step would be to “null” the Average Packet Loss DL IE in Rel18 and, to ensure a backwards compatible solution, to work on the introduction of an appropriate new parameter in Rel19.

A CR reflecting the main proposals in this paper is presented in R3-251985.

# 3. Conclusions

This paper makes the following observations and proposals:

Observation 1: The Packet Loss Rate IE has been introduced to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric. The Packet Loss Rate IE in Rel18 has been reused to measure UL/DL Packet Loss Rate for UE Performance exposure purposes.

Observation 2: There exists a mismatch in the definition of the Average Packet Loss DL IE to its underlying IE’s definition (Packet Loss Rate IE) in relation to its intended use-case as a per-UE performance metric.

Observation 3: the Average Packet Loss DL IE introduced in TS38.423 for UE performance feedback monitoring has an accuracy of 0.1%, which is far above a typical packet loss rate in a real network.

Observation 4: There seem to be drawbacks associated with measuring packet loss as a UE performance metric at both PDCP and RLC level

Observation 5: RLC Level DL Packet Loss rate on Uu is valid for all types of gNB deployments (split or non-split) and it is only available in DL, while the UL PDCP SDU Loss Rate is valid only for split gNBs and it is not available in DL.

Proposal 1: RAN3 to acknowledge the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

- Drawbacks exist with measuring packet loss as a UE performance metrics at either PDCP or RLC level

- The defined packet loss metrics are not suitably available for all deployment options (split/non-split RAN) and traffic directions (UL/DL)

Proposal 2: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

Proposal 4:  Send an LS to SA5 and RAN2 to define a DL PDCP SDU Loss Rate metric. This metric shall also be available for non-split gNBs.

Proposal 5: Redefine the Release-18 definition of Average Packet Loss DL IE, by decoupling it from Packet Loss Rate IE.

A CR reflecting the main proposals in this paper is presented in R3-xxxxxx.