3GPP TSG-RAN WG3 Meeting #127bis	R3-251992

Wuhan, China, 7th – 11th April, 2025

Title: 	Discussion on AI/ML-based Network Slicing

Source: 	Huawei

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing.

This contribution analyzes further the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the following agreements were achieved [4]:

Stage3 signalling design: Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated. Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting… Slice UE performance: UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ... Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

We will first discuss further the Stage 3 signalling details for the Xn interface and then we provide our views on the granularity of the UE performance feedback for the AI/ML-based Network Slicing use case.

## 2.1 On missing information for the predicted slice available capacity

RAN3 agreed that the Data Collection Reporting Initiation and Data Collection Reporting procedures over Xn need to be enhanced to request and report the predicted radio resource status per slice and the predicted slice available capacity. Moreover, the Slice To Report List for Data Collection IE was agreed to be introduced in the DATA COLLECTION REQUEST message pertaining the Data Collection Reporting Initiation procedure to indicate the slice(s) for which the predicted radio resource status per slice and predicted slice available capacity need to be reported.

In RAN3#126 meeting, instead, further agreements were reached concerning the encoding of the predicted radio resource status per slice (which reuses the Rel-18 Predicted Radio Resource Status IE) and the predicted slice available capacity (which is a new per cell IE in the DATA COLLECTION UPDATE message). Moreover, in the last meeting, RAN3 agreed to enhance the Rel-18 partial reporting mechanism to cover the case where the reporting node is requested to provide slice-specific predictions but it is unable to comply to such request. Therefore, RAN3 agreed to introduce in the DATA COLLECTION RESPOSE message the Slice Measurement Initiation Result IE (within the Cell Measurement Initiation Result List IE) to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated – all the above agreements have already been reflected in the XnAP BLCR for enhancements on AI/ML for NG-RAN in [6].

In the last meeting there was a discussion on whether to introduce the predicted Composite Available Capacity (CAC) to be reported per cell together with the predicted slice available capacity. In the legacy XnAP’s Resource Status Reporting procedure the CAC information (indicating the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources per cell) is reported jointly with slice available capacity (which indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell). This because CAC allows to correctly scale the slice available capacity with respect to the total capacity of the cell for cases where some slices share resources from the same resource pool, and this is done via the Cell Capacity Class Value IE carried within the Composite Available Capacity Group IE. In the current XnAP BLCR [6] the new Predicted Slice Available Capacity IE is introduced in the Data Collection Reporting procedure without considering how to scale the reported values in a similar and comparable way as in the legacy Resource Status Reporting. It should be noted that in TR 38.743 [3], clause 4.1.2.2, as part of the input data from the neighbour nodes, both the measured and predicted slice available capacity may be reported to the local node (hosting the AI/ML model) over Xn and used as input data for the AI/ML model, where the measured slice available capacity is reported via the Resource Status Reporting (along with the CAC information, and hence including the Cell Capacity Class Value IE), while the predicted slice available capacity is reported via the Data Collection Reporting. Therefore, we think it is important to ensure that both the measured and predicted slice available capacity metrics can be correctly interpreted at the local node, therefore we should find ways to make the Cell Capacity Class Value information available for the predicted slice available capacity as already possible for the measured slice available capacity.

Possible solutions to the problem can be summarized as follows:

- rely on the Cell Capacity Class Value IE carried in the CAC information within the legacy RESOURCE STATUS UPDATE message; this has two main implications: the first one is that, between local and neighbour node(s), there needs to be both Data Collection Reporting and Resource Status Reporting procedures in place; the second one is that it should be discussed whether it is reasonable to reuse the Cell Capacity Class Value IE used to scale the slice available capacity also for the predicted slice available capacity.

- Include the predicted composite available capacity in the reporting of predicted slice available capacity, in order to imitate legacy behaviour; the main drawback of this solution is that the new IE contains also unnecessary information in addition to the only information needed to interpret the predicted slice available capacity, that is, the Cell Capacity Class Value.

- Introduce a new Cell Capacity Class Value Group IE in the DATA COLLECTION UPDATE message that provides the DL and UL cell capacity class values for the cell at the time the predicted slice available capacity is generated. These parameters act as weight factors for the predicted slice available capacity both in the downlink and in the uplink.

- Introduce a new IE for the predicted slice available capacity (instead of relying on the existing Slice Available Capacity IE as in the current XnAP BLCR) and, within this IE, introduce the missing Cell Capacity Class Value IEs.

We think that, among the options above, and aiming at reducing the specification impact as much as possible, RAN3 should discuss and decide between solution c. or solution d..

Proposal 1: RAN3 to discuss the issue of missing information to correctly scale the predicted slice available capacity and down-select between the following options:

- Option 1: Introduce a new Cell Capacity Class Value Group IE in the DATA COLLECTION UPDATE message that provides the DL and UL cell capacity class values for the cell for which the predicted slice available capacity is requested to be reported.

- Option 2: Introduce a new IE for the predicted slice available capacity (instead of relying on the existing Slice Available Capacity IE as in the current XnAP BLCR) and, within this IE, introduce the missing Cell Capacity Class Value IEs.

In our view a clearer and more future proof approach is the one as described in Solution d. above. Therefore, we propose the following:

Proposal 2: RAN3 to agree on introducing a new IE for the predicted slice available capacity and, within this IE, introduce the missing Cell Capacity Class Value IEs.

## 2.2 Finer granularity of UE performance feedback

With regard to the feedback information, RAN3 agreed during the SI phase to collect from (neighbour) gNBs both the legacy per-UE UE performance feedback for those UEs handed over from the source gNB as well as the finer granularity UE performance feedback (including Average UE Throughput DL/UL, Average Packet Delay and Average Packet Loss DL) for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE. However, the exact level of finer granularity was left for further discussion during the normative phase.

So far, three possible options for the UE performance finer granularity have been proposed and discussed:

- Option 1: per PDU session per slice (S-NSSAI)

- Option 2: per slice (S-NSSAI)

- Option 3: per-QoS-flow-group

Among the three options above, Option 3 is the finest granularity, while Option 2 is the coarsest one. Therefore, it seems that Option 1 – per PDU session per slice granularity – is a compromise which should provide enough granular information for the requesting node to evaluate its AI/ML model. In fact, in RAN3#125bis meeting [7], the majority of companies preferred to have UE performance to be reported on a per PDU session per slice granularity, meaning that UE performance should be collected and reported for each PDU session belonging to a certain slice in use by the UE. This led to the following Working Assumption, with the possibility to further discuss about the feasibility of other granularities [7]:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. FFS if other finer granularity of UE performance is feasible.

However, concerns were raised in the last meeting on the actual feasibility of such granularity. Our understanding is that UE performance metrics are typically measured on a per DRB granularity as per TS 28.558 [8], and multiple DRBs can be mapped into the same PDU session, hence it seems there is currently no standardized way to determine per PDU session measurements so to ensure that source and target gNBs apply the same methodology for deriving per PDU session per slice UE performance feedback before and after an AI/ML-based mobility action while the UE is using a certain slice, hence making the comparison between the two per PDU session measurements meaningful.

For Option 3 (per-QoS-flow-group UE performance), it is still unclear whether this could actually be implemented. In our understanding the “QoS-flow-group” represents the set of QoS flows that are mapped to a certain DRB. Therefore, in the source gNB side, the gNB can measure per DRB UE performance metrics (as per TS 28.558 [8]) and then implicitly derive the per-QoS-flow-group measurement (which corresponds exactly to the per DRB measurement). However, after the AI/ML-based mobility action for the UE using a certain slice, the target gNB may decide to map the set QoS flows (mapped into a single DRB in the source gNB) into multiple different DRBs. In other words, we think that only in case the target node decides to map the same set of QoS flows into a single DRB (as done by the source gNB) the comparison of UE performance before and after the AI/ML-based mobility action could be possible. However, it is kind of a corner case that this could occur, and for sure the target node cannot be forced to apply the same QoS-flows-to-DRB mapping as used by the source gNB.

When it comes to Option 2 (per slice UE performance), instead, it is worth to consider that, for the AI/ML-based Network Slicing, we focused during the SI on slice-level resource allocation and slice-based load balancing/mobility actions, therefore it would make sense to provide UE performance feedback on a per slice level as well.

In RAN3#126 meeting it agreed that the source node can request per UE per slice UE performance considering that some companies (including us) think that per slice granularity of UE performance metrics is already supported by the standard as per SA5’s TS 28.558 [8], even if some companies do not think the same and hence they claim that UE performance cannot be measured by the target node on a per UE per slice granularity and reported back to the source node – all these considerations were also captured in the RAN3#126 meeting’s minutes [5]:

Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification. Source node can request per UE per S-NSSAI granularity for UE performance feedback. There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

However, in the last RAN3 meeting, the following was agreed for the slice UE performance [4]:

Slice UE performance: UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

As consequence, the UE performance may be configured to be reported per slice per UE, in addition to the legacy per UE reporting specified in Rel-18. We think that the discussion on whether further finer granularities need to be introduced can be deferred to future 3GPP releases. .

Proposal 3: RAN3 to agree that the discussion on whether further finer granularities need to be introduced can be deferred to future 3GPP releases.

Still concerning the slice UE performance, there is the following FFS from the last RAN3 meeting that need to be discussed [4]:

Slice UE performance: Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

The above FFS refers to the encoding aspects to enable the source node to configure the reporting of per slice per UE performance. In the last meeting, two main approaches have been proposed as summarized in [9]:

- Option 1: to introduce a new Slice Level UE Performance Reporting IE in the DATA COLLECTION REQUEST message to configure the reporting of all per slice per UE performance metrics. This was proposed in [10].

- Option 2: to extending the existing Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message to explicitly indicate the UE performance metrics that need to be reported per slice per UE. This was proposed in [11].

In our view Option 1 above allows to explicitly indicate the granularity for which UE performance metrics need to be collected and reported, but it should be enhanced with respect to the current encoding in [11] – where the new Slice Level UE Performance Reporting IE is an enumerated type of IE with only one codepoint to be set to ‘true’ – in order to allow for collecting and reporting both legacy and finer granularity UE performance (as agreed in TR 38.743 [3]). One main drawback of Option 1 is that it does not allow to configure the collection of certain metric(s) among all the possible UE performance metrics: for instance, the source node may be interested in receiving only the average throughput DL/UL collected per slice per UE, while the other metrics (i.e., average packet loss and average packet delay) are configured to be reported as per Rel-18 per UE granularity. This drawback could be solved if the existing Report Characteristics for Data Collection IE is enhanced to explicitly indicate the UE performance metrics that need to be reported per slice per UE, as per Option 2. This anyway comes at a cost of easily running out of bits in case new bits need to be introduced in the future in the Report Characteristics for Data Collection IE, in that sense Option 2 is not so much future-proof.

Another option that in our view allows the requesting node to configure the reporting node to report UE performance not only for a specific granularity but also for a subset of UE performance metrics is based on the introduction of a new IE Finer Granularity UE Performance Collection Configuration List in the DATA COLLECTION REQUEST message. This list includes a certain number of items – dependent on the number of finer granularities introduced in Rel-19 (only one if Proposal 3 above is agreed, i.e., per UE per slice) – each of which allows to configure the granularity for which the UE performance needs to be reported, the set of UE performance metrics to be reported, as well as the time window (starting from the successful handover) for the collection of measurements. Moreover, the new IE allows to indicate the list of slices for which UE performance metrics need to be reported. It is worth noting that the legacy (Rel-18) per UE UE performance metrics can still be reported according to the current signalling.

Proposal 4: RAN3 to agree to introduce a new IE Finer Granularity UE Performance Collection Configuration List in the DATA COLLECTION REQUEST message for configuring the reporting of UE Performance metrics according to the finer granularity introduced in Rel-19.

Based on the above, a new IE Finer Granularity UE Performance List is introduced in the DATA COLLECTION UPDATE message for the actual reporting of per UE per slice UE performance metrics, under the UE Associated Info Result List IE. This new IE also contains the UE performance metrics associated to the list of slices as per the configuration in the DATA COLLECTION REQUEST message.

Proposal 5: RAN3 to agree to introduce a new IE Finer Granularity UE Performance List in the DATA COLLECTION UPDATE message for the actual reporting of UE Performance metrics according to the finer granularity introduced in Rel-19.

When it comes to the partial reporting aspects, also the DATA COLLECTION RESPONSE message needs to enhanced to allow the reporting node to indicate whether it can perform the requested measurements for the UE performance. Similar to the agreement from last meeting on the introduction of the Slice Measurement Initiation Result IE within the Cell Measurement Initiation Result List IE for the partial reporting of per cell per slice prediction information (i.e., predicted radio resource status per slice and predicted slice available capacity), the same Slice Measurement Initiation Result IE is proposed to be introduced within the Node Measurement Initiation Result List IE for the partial reporting of per node per slice information (i.e., slice-level UE performance metrics). As a consequence,

Proposal 6: RAN3 to agree to introduce the Slice Measurement Initiation Result IE within the Node Measurement Initiation Result List IE for the partial reporting of per node per slice information (i.e., UE performance metrics).

Proposal 6bis: RAN3 to include new bits within the S-NSSAI Measurement Failed Report Characteristics IE (under the S-NSSAI Measurement Initiation Result List IE) for the UE performance metrics.

The TP for the XnAP BLCR in [12] addresses the above proposals.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Predicted Slice Available Capacity

Proposal 1: RAN3 to discuss the issue of missing information to correctly scale the predicted slice available capacity and down-select between the following options:

- Option 1: Introduce a new Cell Capacity Class Value Group IE in the DATA COLLECTION UPDATE message that provides the DL and UL cell capacity class values for the cell for which the predicted slice available capacity is requested to be reported.

- Option 2: Introduce a new IE for the predicted slice available capacity (instead of relying on the existing Slice Available Capacity IE as in the current XnAP BLCR) and, within this IE, introduce the missing Cell Capacity Class Value IEs.

Proposal 2: RAN3 to agree on introducing a new IE for the predicted slice available capacity and, within this IE, introduce the missing Cell Capacity Class Value IEs.

Finer granularity of UE performance feedback

Proposal 3: RAN3 to agree that the discussion on whether further finer granularities need to be introduced can be deferred to future 3GPP releases.

Proposal 4: RAN3 to agree to introduce a new IE Finer Granularity UE Performance Collection Configuration List in the DATA COLLECTION REQUEST message for configuring the reporting of UE Performance metrics according to the finer granularity introduced in Rel-19.

Proposal 5: RAN3 to agree to introduce a new IE Finer Granularity UE Performance List in the DATA COLLECTION UPDATE message for the actual reporting of UE Performance metrics according to the finer granularity introduced in Rel-19.

Proposal 6: RAN3 to agree to introduce the Slice Measurement Initiation Result IE within the Node Measurement Initiation Result List IE for the partial reporting of per node per slice information (i.e., UE performance metrics).

Proposal 6bis: RAN3 to include new bits within the S-NSSAI Measurement Failed Report Characteristics IE (under the S-NSSAI Measurement Initiation Result List IE) for the UE performance metrics.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-251544, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18

- R3-250786, SoD for CB: # AIRAN1_Slicing, ZTE

- R3-250569, AI/ML Slicing Open Issues, Nokia, Jio Platforms (JPL)

- R3-250471, AI/ML enabled Network Slicing, Ericsson

- R3-251993, (TP for AI/ML BLCR to TS 38.423) XnAP enhancements for AI/ML-based Network Slicing, Huawei