3GPP TSG-RAN WG3 Meeting #127bis	R3-251998

Wuhan, China, 7th – 11th April, 2025

Title: 	(Draft LS to SA5 on UE performance) Discussion on split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to split architecture support for Rel-18 use cases.

This contribution analyzes the technical enablers for specifying support of split architecture for Rel-18 use cases, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

In the last meeting RAN3 discussed the split architecture support for Rel-18 RAN AI/ML use cases and agreed the following [4]:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture. Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture. Stage 3 details of the solution are FFS.

In the next Section 2.1 we discuss further about the reporting of UE performance metrics in split architecture taking the above agreements into account.

## 2.1 On reporting measured UE performance

The scenario to consider for UE performance metrics collection and reporting is illustrated in Figure 1: the measurement collection is triggered e.g. after an AI/ML-based mobility decision (i.e., handover) taken by the source gNB towards a split target gNB is successfully executed.

Figure 1 - Scenario for reporting UE performance from split target gNB to source gNB.

At least for the packet delay, based on agreements from the last meeting, the corresponding delay components measured at gNB-DU and gNB-CU-UP (as specified in TS 28.558 [5]) are expected to be aggregated at the gNB-CU-UP level and then the aggregated result is sent to the gNB-CU-CP over the E1 interface. Concerning the packet delay measurement at gNB-DU level, it is our understanding from TS 38.425 [6] that the UL Delay DL Result and DL Delay DU Result IEs are measured per DRB and provided from the gNB-DU to the gNB-CU-UP over F1-U within the ASSISTANCE INFORMATION DATA (PDU Type 2) frame, and not within the DL DATA DELIVERY STATUS (PDU Type 1) as per the agreement from the last RAN3 meeting:

### 5.4.3 Transfer of Assistance Information #### 5.4.3.1 Successful operation NOTE 1: In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP. The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures. An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only. *** skip non-relevant text *** The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN. #### *** skip non-relevant text *** #### 5.5.2.3 ASSISTANCE INFORMATION DATA (PDU Type 2) This frame format is defined to allow the node hosting the PDCP entity to receive assistance information. The following shows the respective ASSISTANCE INFORMATION DATA frame. NOTE 1: All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=2); PDCP Dupl. Ind.; Assistance Info. Ind.; UL Delay Ind.; DL Delay Ind.; 1. Spare; UL Congestion Information Ind.; DL Congestion Information Ind.; PDCP Duplication Activation Suggestion; 1. Number of Assistance Information Fields; 0 or 1. Assistance Information Type; 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields). Number of octets for Radio Quality Assistance Information Fields. Radio Quality Assistance Information. UL Delay DU Result; 0 or 4. DL Delay DU Result; 0 or 4. UL Congestion Information; 0 or 2. DL Congestion Information; 0 or 2. Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format **** skip non-relevant text **** #### 5.5.3.48 UL Delay DU Result Description: This field indicates UL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total UL delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8]. This information element is not applicable to E-UTRA PDCP. Value range: {0..232-1}. Field length: 4 octets. #### 5.5.3.49 DL Delay DU Result Description: This field indicates DL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total DL delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8]. This information element is not applicable to E-UTRA PDCP. Value range: {0..232-1}. Field length: 4 octets.

Therefore we propose to revise and correct the agreement from RAN3#127:

Proposal 1: RAN3 to agree to revise and correct the agreement from RAN3#127 as follows “Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA (PDU Type 2) in the case of CU-DU split architecture.”.

Once the UL Delay DL Result and DL Delay DU Result measurements are made available at the gNB-CU-UP level, the latter will aggregate them with the corresponding UL and DL delay components measured at the gNB-CU-UP level – that is, average delay over F1-U, average delay DL in CU-CP and average PDCP re-ordering delay (all defined per DRB as per TS 28.558 [5]) – in order to derive the overall UL Delay Result and DL Delay Result measurements to be sent over E1 to the gNB-CU-CP as per the agreement from the last RAN3 meeting.

In addition to Average Packet Delay DL/UL, RAN3 also introduced in Rel-18 the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss DL as part of per UE metrics representing the UE performance to be reported from target gNB to source gNB over Xn.

The agreements from last meeting concerning the Average Packet Delay DL/UL suggest that a “gNB-CU-UP-centric” approach should be considered also for the remaining UE performance metrics.

Average UE Throughput UL/DL

As per TS 28.558 [5], the Average UE Throughput UL/DL is measured in the gNB-DU and there is no current measurement specified in this TS that allows to obtain the throughput metric at PDCP level, i.e., measured by the gNB-CU-UP. However, TS 28.552 clause 5.1.1.3.7 [7] contains a measurement referred to as Average DL UE buffered Throughput per DRB which is performed at PDCP based on desired buffer size for the DRB communicated within the DL DATA DELIVERY STATUS (PDU Type 1) frame over F1-U from DU to CU-UP (refer to TS 38.425 [6] clause 5.5.2.2) and is intended for services with burst duration spanning over the time interval of a couple of consequent DDDSs. We think it could be checked with SA5 whether such measurement could be referred for the purpose of UE performance monitoring following an AI/ML-derived mobility decision. The main issue we see with this approach is that there is currently no PDCP-level measurement for the Average UL UE Throughput, therefore we should make SA5 aware of this.

Proposal 2: RAN3 to agree to send a LS to SA5 to check whether the Average DL UE buffered Throughput per DRB could be referred for the purpose of UE performance monitoring. In the LS, RAN3 should make SA5 aware of the missing Average UL UE Throughput measurement.

Average Packet Loss UL/DL

For the Packet Loss, TS 28.558 [5] defines two measurements:

- DL Packet Loss Rate on Uu: this is valid for all gNB deployment scenarios (split and non-split), it is only available for DL, and it is measured in the gNB-DU, by considering DL packet loss rate on the Uu interface (i.e., RLC SDU). Therefore, considering the discussion so far, it seems useless for the purpose of UE performance monitoring;

- UL PDCP SDU Loss Rate: this is valid for the split gNB deployment scenario only, it is only available for UL, but it is measured in the gNB-CU-UP, by taking into account the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Therefore, it seems this measurement could be used as a metric for UE performance, even though this would be applicable to split gNBs only. However, the DL PDCP-level packet loss rate is not defined.

Based on the above, and considering the impacts on the Rel-18 UE performance framework, before taking a decision on whether we should remove the already specified Average Packet Loss DL as one of the UE performance metrics, we propose to check this aspect with SA5 as well, to see whether it is feasible to define a Packet Loss Rate metric for DL and UL which is measured in the gNB-CU-UP and is valid for all gNB deployment scenarios.

Proposal 3: RAN3 to agree to include in the LS to SA5 a question on whether it is feasible to define a Packet Loss Rate metric for DL and UL which is measured in the gNB-CU-UP and is valid for all gNB deployment scenarios.

A draft LS to SA5 can be found in the Annex of this paper.

Another aspect that should be discussed concerns the granularity of UE performance. For Rel-18, the granularity has been specified to be per-UE but most of the measurements at the gNB-CU-UP that could be of interest for the UE performance are defined in TS 28.558 with a per DRB granularity and, to derive the corresponding per UE metrics, the RAN3 common understanding from RAN3#124 meeting can be considered, that is, per UE UE performance is derived as the average value of per DRB measurements of the UE performance metrics [8]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

Still referring to Figure 1, the focus of the discussion is on collecting at gNB-CU-CP of the target split gNB all the UE performance metrics reported by target gNB-CU-UP. In this case we think that per DRB UE performance metrics can be directly reported to the gNB-CU-CP on a per DRB granularity as per current specification. It will be then up to the target gNB-CU-CP to derive the corresponding per UE measurements following the RAN3 common understanding from RAN3#124 meeting and report over Xn to the source gNB.

Proposal 4: RAN3 to agree that UE performance metrics that are defined per DRB are reported by the gNB-CU-UP to the gNB-CU-CP with per DRB granularity. It will be up to the gNB-CU-CP to derive the corresponding per UE measurements.

Regarding the signalling details deriving from the agreements on packet delay from the last meeting, we think that RAN3 should discuss whether new procedures over E1 should be introduced (similar to the XnAP’s Data Collection Reporting Initiation and Data Collection Reporting) or existing procedure should be enhanced. Based on the above Proposal 4, and considering that the main purpose of this signalling is to transfer per DRB measurements from the gNB-CU-UP to the gNB-CU-CP, we think that this can be achieved by enhancing the existing Resource Status Reporting Initiation and Resource Status Reporting procedures over E1.

Proposal 5: RAN3 to discuss whether UE performance metrics could be transferred from the gNB-CU-UP to the gNB-CU-CP by enhancing the existing Resource Status Reporting (Initiation) procedures over E1.

The TP for the AI/ML BLCR to TS 37.483 in [9] reflects the above Proposals, and includes both options of enhancing the existing Resource Status Reporting (Initiation) procedures over E1 as well as of introducing new Data Collection Reporting (Initiation) procedures.

## 2.2 On gNB-DU Energy Cost (EC) reporting over F1

In RAN3#125bis meeting, as part of the Rel-18 leftover related to split architecture support for Rel-18 use cases, RAN3 agreed to introduce the Data Collection Reporting Initiation and Data Collection Reporting procedures over F1, as done in Rel-18 for the Xn interface, to enable the reporting of the measured Energy Cost (EC) from the DU to the CU [10]:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In the RAN3#125 meeting, further agreements for enabling the reporting of the gNB-DU’s EC to the gNB-CU, and for the intended usage of the reported gNB-DU’s EC have been reached [10]:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs. Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

Moreover, due to the support needed from the OAM side for configuring the unified “energy consumption – to – energy cost” mapping rule to the gNB-DU, RAN3 agreed to send a LS to SA5 in [12].

In the previous section we discussed the UE performance for the split architecture case and, pending SA5 feedback on the feasibility to introduce new PDCP-level measurements, it could be plausible to assume that RAN3 may eventually end up in exchanging the UE performance metrics over the E1 interface only, i.e., without any impact on the F1 interface. Based on this assumption, RAN3 should discuss whether it would be better to revisit the agreements from previous meetings on the reporting of the gNB-DU’s EC over F1, in order to exchange this information by enhancing an existing procedure such as the Resource Status Reporting rather than introducing a brand new procedure in Rel-19 for purpose of transferring just a single metric (i.e., the gNB-DU’s EC).

Proposal 6: RAN3 to discuss whether previous agreements related to the exchange of the gNB-DU’s EC should be revisited in order to enable the transfer of the gNB-DU’s EC to the gNB-CU by enhancing the existing Resource Status Reporting (Initiation) procedures over F1.

The TP to the BLCR to TS 38.473 in [13] includes both options of enhancing the existing Resource Status Reporting (Initiation) procedures over F1 as well as of introducing new Data Collection Reporting (Initiation) procedures.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

On reporting measured UE performance

Proposal 1: RAN3 to agree to revise and correct the agreement from RAN3#127 as follows “Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA (PDU Type 2) in the case of CU-DU split architecture.”.

Proposal 2: RAN3 to agree to send a LS to SA5 to check whether the Average DL UE buffered Throughput per DRB could be referred for the purpose of UE performance monitoring. In the LS, RAN3 should make SA5 aware of the missing Average UL UE Throughput measurement.

Proposal 3: RAN3 to agree to include in the LS to SA5 a question on whether it is feasible to define a Packet Loss Rate metric for DL and UL which is measured in the gNB-CU-UP and is valid for all gNB deployment scenarios.

A draft LS to SA5 can be found in the Annex of this paper.

Proposal 4: RAN3 to agree that UE performance metrics that are defined per DRB are reported by the gNB-CU-UP to the gNB-CU-CP with per DRB granularity. It will be up to the gNB-CU-CP to derive the corresponding per UE measurements.

Proposal 5: RAN3 to discuss whether UE performance metrics could be transferred from the gNB-CU-UP to the gNB-CU-CP by enhancing the existing Resource Status Reporting (Initiation) procedures over E1.

The TP for the AI/ML BLCR to TS 37.483 in [9] reflects the above Proposals, and includes both options of enhancing the existing Resource Status Reporting (Initiation) procedures over E1 as well as of introducing new Data Collection Reporting (Initiation) procedures.

On gNB-DU EC reporting

Proposal 6: RAN3 to discuss whether previous agreements related to the exchange of the gNB-DU’s EC should be revisited in order to enable the transfer of the gNB-DU’s EC to the gNB-CU by enhancing the existing Resource Status Reporting (Initiation) procedures over F1.

The TP to the BLCR to TS 38.473 in [13] includes both options of enhancing the existing Resource Status Reporting (Initiation) procedures over F1 as well as of introducing new Data Collection Reporting (Initiation) procedures.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP TS 28.558 “Management and orchestration; UE level measurements for 5G system”, Rel-19

- 3GPP TS 38.425, “NG-RAN; NR user plane protocol”, Rel-19

- 3GPP TS 28.552, Management and orchestration; 5G performance measurements, Rel-19

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)u

- R3-252000, (TP for the AI/ML BLCR to TS 37.483) E1AP enhancements for the split architecture support for Rel-18 AI/ML for NG-RAN use cases, Huawei

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-247907, LS on energy saving for split gNB, CATT

- R3-251999, (TP for the AI/ML BLCR to TS 38.473) F1AP enhancements for the split architecture support for Rel-18 AI/ML for NG-RAN use cases, Huawei

# 5. Annex – Draft LS to SA5 on UE performance metrics for split architecture

3GPP TSG-RAN3 Meeting #127bis	R3-25abcd

Wuhan, China, 7th – 11th April, 2025

Title:	[Draft] LS on UE performance metrics for split architecture

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-core

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Work Item on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 has been tasked to specify necessary signalling enhancement to support split architecture for Rel-18 “AI/ML for NG-RAN” use cases.

In Rel-18 RAN3 considered non-split architecture only, and introduced UE performance metrics in TS 38.423 to be exchanged over the Xn interface from target gNB to source gNB after a handover event occurs. The UE performance metrics are collected by the target gNB and reported to the source gNB upon request. The specified UE performance metrics are Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL.

RAN3 is now discussing how the gNB-CU-UP of a target split gNB can collect the above mentioned metrics and report to the gNB-CU-CP over the E1 interface. For the packet delay measurement RAN3 agreed the following:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture. Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA (PDU Type 2) in the case of CU-DU split architecture.

In an effort to define a “gNB-CU-UP” unified framework for UE performance metrics, RAN3 would like to have SA5 feedback on:

- whether the Average DL UE buffered Throughput per DRB, defined in TS 28.552 clause 5.1.1.3.7 and measured in gNB-CU-UP (at PDCP), could be considered as one UE performance metric. If that case, RAN3 suggests introducing it in TS 28.558;

- whether it is feasible to specify the corresponding UL throughput measurement, to be measured in gNB-CU-UP (at PDCP);

- whether it is feasible to specify a packet loss rate metric for DL and UL, measured in gNB-CU-UP (at PDCP) and valid for all gNB deployment scenarios.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to provide feedback on:

- whether the Average DL UE buffered Throughput per DRB, defined in TS 28.552 clause 5.1.1.3.7 and measured in gNB-CU-UP (at PDCP), could be considered as one UE performance metric. If that case, RAN3 suggests introducing it in TS 28.558;

- whether it is feasible to specify the corresponding UL throughput measurement, to be measured in gNB-CU-UP (at PDCP);

- whether it is feasible to specify a packet loss rate metric for DL and UL, measured in gNB-CU-UP (at PDCP) and valid for all gNB deployment scenarios.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#128				2025-05-19 – 2025-05-23				Malta, MT

RAN3#129				2025-08-25 – 2025-08-29				Bangalore, IN