3GPP TSG-RAN WG3 Meeting #127bis								    	R3-252001

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	20.2

Source:	Huawei

Title:	(TP for AI/ML BLCR to TS 38.455) Discussion on RAN3 impacts for Direct AI/ML positioning (Case 3b)

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution focuses on Direct AI/ML positioning Case 3b – being the only case of AI/ML positioning that have RAN3 impacts, together with Direct AI/ML positioning Case 3b, discussed in a separate contribution [2] – and provides proposals based on the latest progress in RAN1 and in RAN3. It is worth noting that the latest status of RAN1 discussions for Case 3b is summarized in the LS sent by RAN1 in [3].

# Discussion

In the following we further discuss aspects related to model training, model inference and model monitoring for AI/ML positioning Case 3b (i.e., NG-RAN node assisted positioning with LMF-side model).

In the last meeting RAN3 discussed technical aspects for Case 3a only. For Case 3b, the following agreements and open issues captured in RAN3#125bis meeting minutes are referenced [4]:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

For AI/ML positioning, the preliminary Rel-18 study phase – whose outcome is summarized in TR 38.843 [5] – identified that, for Case 3b, the output of the AI/ML model hosted in the LMF is an estimate (prediction) of the UE location which is inferred by means of measurements provided by the NG-RAN.

In order to assess the RAN3 impacts for supporting AI/ML positioning we will discuss model training, model inference and model monitoring aspects.

## 2.1 Model training considerations

For Case 3b the AI/ML model training is located at the LMF, which is the termination point of training data, as reflected in TR 38.843 clause 7.2.4 [5]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** o For LMF-side model, the LMF is the termination point for training data.

Proposal 1: RAN3 to confirm that model training for Case 3b is performed in the LMF. Details on how model training is performed is out of RAN3 scope.

As per RAN1 agreements below, a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions). For Part A RAN1 agreed that it is generated by the TRP/gNB (for both Case 3a and Case 3b), that is, NG-RAN; while Part B for Case 3b can be generated by PRU, non-PRU UE with estimated location or LMF. RAN1 also agreed on the components that could be included in Part A and Part B of the training data sample (valid for both Case 3a and Case 3b).

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement ******** RAN1#116bis ******** Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. ******** RAN1#117 ******** Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed ******** RAN1#118 ******** Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications ******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; FFS: details on how to associate quality indicator to timing information ******** RAN1#120 ******** Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

For Case 3b, there is RAN3 impact in providing Part A of the training data sample from NG-RAN to LMF, and this is also reflected in the agreement that RAN3 reached in RAN3#125bis meeting [4]:

For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

In Case 3b, for Part B of the training data sample, if it is generated by PRU/Non-PRU UE with estimated location, there is no RAN3 impact since this information can be provided via LPP protocol to the LMF. If Part B is generated by the LMF, there is as well no RAN3 impact since this information is already available in the entity performing AI/ML model training, i.e., LMF.

In RAN3#125bis meeting the open point on the details of the measurements (Part A) being pending and subject to further discussion in RAN1 was captured in the meeting minutes for both Case 3a and Case 3b [4]. In previous meetings RAN1 achieved some progress on Part A’s content which, for Case 3b, needs to be made available at the LMF to perform AI/ML model training. In our understanding Part A’s content for Case 3b reflects the content of the measurement report provided by the gNB to the LMF for successive AI/ML model inference as per the agreement from RAN1#118bis meeting (discussed in Section 2.2).

Still based on RAN1 agreements, for Case 3b, Part A of the training data sample also includes a timestamp of the channel measurement and quality indicator of the channel measurement (where the channel measurement could be either timing information or paired timing information and power information, as per RAN1#116 meeting). For these pieces of information RAN1 also agreed that the existing IEs “Time Stamp” and “Timing Measurement Quality” in TS 38.455 [6] (NRPPa protocol), respectively, can be reused from RAN1 perspective:

******** RAN1#118 ******** Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. ******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; FFS: details on how to associate quality indicator to timing information

Therefore, for Case 3b, there is no additional RAN3 impact on how to transfer the time stamp and quality indicator both associated to the channel measurement to the LMF.

And from RAN1#120 meeting agreements, since in Case 3b only Part A (channel measurement) is reported from the gNB to the LMF, the time stamp (of Part A) is needed for pairing of Part A entry with a Part B entry, and it refers to the time instance when the channel measurement is performed:

******** RAN1#120 ******** Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

Proposal 2: For Case 3b, there is no RAN3 impact in providing the LMF with the time stamp and quality indicator of the channel measurement included in Part A of the training data sample.

The above Proposal 2 is also in line with the agreement reached in RAN3#125bis meeting concerning the actual reporting of measurements to the LMF for Case 3b, which will be performed by re-using the existing measurement procedure (and hence messages) already specified in NRPPa [4]:

Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

In addition, RAN1 agreed in RAN1#118 meeting a Note that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18 as captured in TS 38.300 clause 16.20.2 [7]. Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3b.

Proposal 3: For the discussion of RAN3 impacts of Case 3b, RAN3 to agree that the purpose, such as “training data collection”, or “monitoring”, or “inference” will not necessarily be specified in RAN3 specification when providing Part A of the training data sample to the LMF.

## 2.2 Model inference considerations

For Case 3b, to enable the AI/ML model in the LMF to directly infer the UE location, some input data needs to be provided to the AI/ML model itself. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. ******** RAN1#116bis ******** Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. ******** RAN1#118 ******** Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. ******** RAN1#118bis ******** Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. ******** RAN1#119 ******** Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion. Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified ******** RAN1#120 ******** Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

From the above agreements, it can be seen that RAN1 introduced two channel measurement types as (A) and (B) – note that the notation for measurement types “(A)” and “(B)” is used in RAN1 for discussion purpose only.

Measurement type (A) corresponds to path-based measurement, that is, measurement in the existing specifications (up to Rel-18), while (B) corresponds to Rel-19 enhanced measurement which has been defined as part of the following RAN1 agreements and was previously known as sample-based measurement:

******** RAN1#119 ******** Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified ******** RAN1#120 ******** Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24}

In order to support measurement type (B), there is the need to signal from LMF to gNB a set of configuration parameters (Nt, Nt' and k as agreed by RAN1), whose value ranges have been discussed and agreed in RAN1 as captured above. Therefore, RAN3 needs to start discussing about the signalling details, e.g., which procedure over NRPPa to use. In our view, and in light of the RAN3 agreements from RAN3#125bis that the Measurement procedure defined in NRPPa needs to be reused to support requesting and reporting the related measurements to LMF for Case 3b, we think that the TRP Measurement Quantity Item IE in the MEASUREMENT REQUEST message (LMF  NG-RAN node) could be enhanced so that the LMF can signal Nt' and Nt to the gNB. In our understanding, in fact, k can already be signalled to the gNB with the values agreed by RAN1, that is, k = 0,..,5.

Proposal 4: RAN3 to agree that the TRP Measurement Quantity Item IE in the MEASUREMENT REQUEST message in NRPPa is enhanced to signal the configuration parameters (Nt and Nt' as agreed by RAN1) from LMF to gNB in order to support the channel measurement type (B) for Case 3b.

Based on the agreement from RAN1#116 that, for Case 3b, at least timing information as well as paired timing information and power information are supported for reporting, it is our understanding that the channel measurements for which the measurement types (A) and (B) apply is UL RTOA. This because the gNB Rx-Tx time difference (i.e., a gNB measurement) is used along with the UE Rx-Tx time difference (i.e., a UE measurement) for multi-RTT positioning to compute a UE location based on a round trip time. Extending this to AI/ML models, then we would need both gNB measurements and UE measurements as model input to determine the UE location, which is like combining Case 3b and Case 2b for which RAN1 has no related agreements. Moreover, Case 2b was removed from the objectives in the latest version of the WID [1], hence it will not be specified in Rel-19. Therefore, we think that for Case 3b the LMF can only request UL RTOA measurements (and, optionally, also UL SRS-RSRPP) from the gNB. Based on this, the presence of the measurement type (B) configuration parameters (Nt, Nt' and k) within the MEASUREMENT REQUEST message needs to be conditional to the fact that the TRP Measurement Type IE in the TRP Measurement Quantities Item IE is UL-RTOA.

Proposal 5: RAN3 to agree that the presence of the measurement type (B) configuration parameters (Nt and Nt') within the TRP Measurement Quantity Item IE in the MEASUREMENT REQUEST message needs to be conditional to the fact that the TRP Measurement Type IE in the TRP Measurement Quantities Item IE is UL-RTOA.

Concerning the measurement report for Case 3b, in the last meeting RAN1 agreed that “descriptive info” of the channel measurement is included together with the channel measurement values – either explicitly or implicitly – and such “descriptive info” includes at least the measurement type (either (A) or (B)) and measurement parameters. For the case of measurement type (A), RAN1 agreed that the measurement parameters include the number of additional path and k, while for measurement type (B) the measurement parameters are at least Nt' and k (Nt is FFS in RAN1):

******** RAN1#120 ******** Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only.

In light of the RAN3 agreements from RAN3#125bis that the Measurement procedure defined in NRPPa needs to be reused to support requesting and reporting the related measurements to LMF for Case 3b, we think that the UL RTOA Measurement IE within the TRP Measurement Result IE – included in both the MEASUREMENT RESPONSE and MEASUREMENT REPORT messages (NG-RAN node  LMF) – could be enhanced so that the gNB can allow the LMF to implicitly deduce the measurement type ((A) or (B)) as well as the measurement parameters included in the definition of “descriptive info” of the channel measurement as agreed by RAN1, that is:

- number of additional paths and k, for measurement type (A);

- Nt' and k, for measurement type (B).

In our view a new IE Further Extended Additional Path List (NAME FFS) in the UL RTOA Measurement IE may serve the purpose above.

Proposal 6: RAN3 to agree that the UL RTOA Measurement IE within the TRP Measurement Result IE (included in both the MEASUREMENT RESPONSE and MEASUREMENT REPORT messages of NRPPa) is enhanced by introducing a new IE Further Extended Additional Path List (NAME FFS) so that the gNB can allow the LMF to implicitly deduce the measurement type ((A) or (B)) as well as the measurement parameters included in the definition of “descriptive info” of the channel measurement.

From the Conclusion in RAN1#120 meeting, RAN1 has the understanding that if the gNB cannot report channel measurements according to either type (A) or type (B) as requested by the LMF, then the gNB needs to indicate that the gNB cannot provide measurements according to the requested measurement type, and the corresponding signalling details are in the scope of RAN3. In addition, the gNB cannot provide measurements corresponding to a measurement type different from the requested one.

By checking TS 38.455 [6] clause 9.2.1, the following Radio Network Layer related failure causes have been specified:

### 9.2.1 Cause The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol. *** skip non-relevant text *** The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

Based on the above, it seems that the current signalling (i.e., failure causes) does not allow the gNB to indicate the LMF that it cannot perform channel measurements according to the requested measurement type. In fact, the “Requested Item not Supported” failure cause may be related to the scenario for which the gNB does not support the requested measurement object (or cannot provide the requested information item) regardless of the measurement type. And it is our understanding that RAN1’s intention is to let the LMF understand that it cannot report channel measurements for that specific requested measurement type – either (A) or (B) – even if the measurement per se is supported by the gNB. Therefore we think that RAN3 should introduce a new failure cause in the NRPPa protocol such as “Requested Measurement not Supported with the requested measurement type”. Moreover, considering the gNB’s current operational situation at the time of receiving the MEASUREMENT REQUEST from the LMF, we think it could be useful to have an additional new failure cause such as “Requested Measurement Temporarily not Available for the requested measurement type” to cover the case in which the gNB cannot report the channel measurements performed according to the requested measurement type (even if supported) due to e.g. temporary RAN overload.

Proposal 7: RAN3 to agree to introduce the following new failure causes in the NRPPa protocol:

- Requested Measurement not Supported with the requested measurement type;

- Requested Measurement Temporarily not Available for the requested measurement type.

Back to RAN1#118bis meeting, RAN1 had some agreements concerning the content of the measurement report from gNB to LMF for Case 3b, which includes the channel measurement, the quality of the channel measurement and the time stamp of the channel measurement. In particular, RAN1 discussed about the presence (mandatory/optional) of components (IEs) that constitute the measurement report. Similar discussion occurred in RAN1#118bis meeting also for Case 3a, where a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3 was captured in RAN1 meeting minutes.

In our understanding such Note also applies for the agreement from RAN1#118bis above mentioned for Case 3b, even if this has not been explicitly captured by RAN1 – the motivation of that is because the agreements about the presence of components that constitute the measurement report for Case 3b and Case 3a have been taken in different times during the meeting week, which led RAN1 in including the Note only for one of the two AI/ML positioning cases having RAN3 impacts.

Still based on the current NRPPa specification [8], in our understanding the presence of IEs (channel measurement, quality of the channel measurement, time stamp) in the measurement report from gNB to LMF for Case 3b is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report, which has no additional standard impact on NRPPa.

Proposal 8: RAN3 to confirm the RAN1 agreement that the presence of IEs in the measurement report from gNB to LMF for Case 3b is as in current NRPPa, that is:

- Channel measurement: mandatory IE

- Quality of the channel measurement: optional IE

- Time stamp of the channel measurement: mandatory IE.

For Case 3b, RAN1 agreed that channel measurements supported for reporting can be either timing information or paired timing information and power information (RAN1#116).

In our understanding the power information associated to channel measurements that are provided from NG-RAN to the LMF are based on the existing UL SRS (Uplink Sounding Reference Signal), that is, UL SRS Reference Signal Received Path Power (UL SRS-RSRPP) measurement. Therefore, we think there is no standard impact in reporting such information to from NG-RAN to LMF for Case 3b.

Proposal 9: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information (UL SRS-RSRPP) from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Moreover, still for Case 3b, for the timing information of channel measurements, it was agreed in RAN1#116bis that the timing information is represented relative to a reference time which is the existing UL RTOA (Uplink Relative Time of Arrival) reference time T0+tSRS as defined in TS 38.215 . Hence in our view there is no RAN3 impact in reporting timing information of channel measurements from NG-RAN to LMF for Case 3b.

Proposal 10: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

## 2.3 Model monitoring considerations

Regarding the AI/ML model monitoring, we think RAN3 should agree on a general principle concerning the location of the model monitoring function with respect to the entity which is in charge of the AI/ML model inference or, in general, of taking a proper “monitoring decision” for the model based on inference results (e.g., (de-)activation, fallback to legacy positioning methods, etc.). In our view, it would be better that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation, which is also the entity which has knowledge of the model being used. This because in our understanding the “monitoring decision” should be based on the model-specific monitoring metric calculated by considering two main components, that is, the inference results (i.e., model output) and the ground-truth labels. And for Case 3b in particular, this is also motivated by the following RAN1 agreement from RAN1#116, where it can be deduced that (model) performance monitoring is performed in LMF (i.e., the entity hosting the AI/ML model):

******** RAN1#116 ******** Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Proposal 11: For AI/ML-based positioning RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

Still based on the above mentioned agreement from RAN1#116 on Case 3b, our understanding is that RAN1 is still studying whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b. Therefore, it is suggested waiting for further RAN1 progress before starting the discussion in RAN3.

Proposal 12: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

A TP for the AI/ML BLCR to TS 38.455 is in the Annex of this paper.

# Conclusions

Model training considerations

Proposal 1: RAN3 to confirm that model training for Case 3b is performed in the LMF. Details on how model training is performed is out of RAN3 scope.

Proposal 2: For Case 3b, there is no RAN3 impact in providing the LMF with the time stamp and quality indicator of the channel measurement included in Part A of the training data sample.

Proposal 3: For the discussion of RAN3 impacts of Case 3b, RAN3 to agree that the purpose, such as “training data collection”, or “monitoring”, or “inference” will not necessarily be specified in RAN3 specification when providing Part A of the training data sample to the LMF.

Model inference considerations

Proposal 4: RAN3 to agree that the TRP Measurement Quantity Item IE in the MEASUREMENT REQUEST message in NRPPa is enhanced to signal the configuration parameters (Nt and Nt' as agreed by RAN1) from LMF to gNB in order to support the channel measurement type (B) for Case 3b.

Proposal 5: RAN3 to agree that the presence of the measurement type (B) configuration parameters (Nt and Nt') within the TRP Measurement Quantity Item IE in the MEASUREMENT REQUEST message needs to be conditional to the fact that the TRP Measurement Type IE in the TRP Measurement Quantities Item IE is UL-RTOA.

Proposal 6: RAN3 to agree that the UL RTOA Measurement IE within the TRP Measurement Result IE (included in both the MEASUREMENT RESPONSE and MEASUREMENT REPORT messages of NRPPa) is enhanced by introducing a new IE Further Extended Additional Path List (NAME FFS) so that the gNB can allow the LMF to implicitly deduce the measurement type ((A) or (B)) as well as the measurement parameters included in the definition of “descriptive info” of the channel measurement.

Proposal 7: RAN3 to agree to introduce the following new failure causes in the NRPPa protocol:

- Requested Measurement not Supported with the requested measurement type;

- Requested Measurement Temporarily not Available for the requested measurement type.

Proposal 8: RAN3 to confirm the RAN1 agreement that the presence of IEs in the measurement report from gNB to LMF for Case 3b is as in current NRPPa, that is:

- Channel measurement: mandatory IE

- Quality of the channel measurement: optional IE

- Time stamp of the channel measurement: mandatory IE.

Proposal 9: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information (UL SRS-RSRPP) from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Proposal 10: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Model monitoring considerations

Proposal 11: For AI/ML-based positioning RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

Proposal 12: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

A TP for the AI/ML BLCR to TS 38.455 is in the Annex of this paper.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107

- R3-252002, (TP for AI/ML BLCR to TS 38.455) Discussion on RAN3 impacts for NG-RAN node-assisted AI/ML positioning (Case 3a), Huawei

- R3-251510 (R1- 2501628), LS on AI/ML Positioning Case 3b, RAN1 (Ericsson)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (v18.0.0), Rel-18

- 3GPP TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa), Rel-18

- 3GPP TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel-18

# Annex – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

- 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

- 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

*** skip non-relevant text ***

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the Number of Consecutive Measurements (NAME FFS) IE and the Number of Selected Measurements (NAME FFS) IE are included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring enhanced channel measurements.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

*** skip non-relevant text ***

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s).

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | Ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. It also indicates the timing granularity of the selected channel response values in time domain (Nt') | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Number of Consecutive Measurements (NAME FFS) | C-ifMeasurement |  | ENUMERATED(32, 64, 128, …) | It indicates the number of consecutive channel response values in time domain (Nt) |  |  |
| >>Number of Selected Measurements (NAME FFS) | C-ifMeasurement |  | ENUMERATED(8, 16, 24, …) | It indicates the number of selected channel response values in time domain (Nt') over the number of consecutive channel response values |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | Ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |
| ifMeasurement | This IE shall be present if the TRP Measurement Type IE is set to the value “UL-RTOA” |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |  |  |
| >k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |  |  |
| >k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |  |  |
| >k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |  |  |
| >k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |  |  |
| >k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |  |  |
| >kminus1 |  |  | INTEGER (0..3940097) | TS 38.133 [16] | YES | ignore |
| >kminus2 |  |  | INTEGER (0..7880193) | TS 38.133 [16] | YES | ignore |
| >kminus3 |  |  | INTEGER (0.. 15760385) | TS 38.133 [16] | YES | ignore |
| >kminus4 |  |  | INTEGER (0.. 31520769) | TS 38.133 [16] | YES | ignore |
| >kminus5 |  |  | INTEGER (0.. 63041537) | TS 38.133 [16] | YES | ignore |
| >kminus6 |  |  | INTEGER (0.. 126083073) | TS 38.133 [16] | YES | ignore |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.2.85 |  | YES | Ignore |
| Further Extended Additional Path List (NAME FFS) | O |  | 9.2.XX |  | YES | Ignore |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.2.1	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ..., Serving NG-RAN node changed, Requested Item not Supported on Time, Requested Measurement not Supported with the requested measurement type, Requested Measurement Temporarily not Available for the requested measurement type ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message), ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |
| Serving NG-RAN node changed | The UE has moved to another serving NG-RAN node. |
| Requested Item not Supported on Time | The NG-RAN node is unable to provide the measurement results on time. |
| Requested Measurement not Supported with the requested measurement type | The NG-RAN node does not support the requested measurement object to be provided according to the requested measurement type. |
| Requested Measurement Temporarily not Available for the requested measurement type | The NG-RAN node can temporarily not provide the requested measurement object according to the requested measurement type. |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerned criticality indicated "reject" (see sub clause 10.3 of TS 38.413) |
| Abstract Syntax Error (Ignore and Notify) | The received message included an abstract syntax error and the concerned criticality indicated "ignore and notify" (see sub clause 10.3 of TS 38.413) |
| Abstract syntax error (falsely constructed message) | The received message contained IEs or IE groups in wrong order or with too many occurrences (see sub clause 10.3 of TS 38.413) |
| Message not Compatible with Receiver State | The received message was not compatible with the receiver state (see sub clause 10.4 of TS 38.413) |
| Semantic Error | The received message included a semantic error (see sub clause 10.4 of TS 38.413) |
| Transfer Syntax Error | The received message included a transfer syntax error (see sub clause 10.2 of TS 38.413) |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related |

| Miscellaneous cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.2.XX	Further Extended Additional Path List (NAME FFS)

This IE contains the extended additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Extended Additional Path Item |  | 1..<maxNoPathFurtehrExtended> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 |  |  | INTEGER(0..16351) | TS 38.133 [16] |  |  |
| >>k1 |  |  | INTEGER(0..8176) | TS 38.133 [16] |  |  |
| >>k2 |  |  | INTEGER(0..4088) | TS 38.133 [16] |  |  |
| >>k3 |  |  | INTEGER(0..2044) | TS 38.133 [16] |  |  |
| >>k4 |  |  | INTEGER(0..1022) | TS 38.133 [16] |  |  |
| >>k5 |  |  | INTEGER(0..511) | TS 38.133 [16] |  |  |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxNoPathFurtherExtended | Maximum no. of further extended additional path measurement. Value is FFS. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified text ***

id-SRSPosPeriodicConfigHyperSFNIndex,

id-NumberOfSelectedMeasurements(NAME-FFS)

id-EnhancedTimingReportingGranularityFactor(NAME-FFS)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- C

CarrierFreq ::= SEQUENCE {

pointA				INTEGER (0..3279165),

offsetToCarrier		INTEGER (0..2199, ...),

iE-Extensions		ProtocolExtensionContainer { {CarrierFreq-ExtIEs} } OPTIONAL,

...

}

CarrierFreq-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

protocol			CauseProtocol,

misc				CauseMisc,

choice-Extension	ProtocolIE-Single-Container {{ Cause-ExtensionIE }}

}

Cause-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

requested-item-not-supported,

requested-item-temporarily-not-available,

...,

serving-NG-RAN-node-changed,

requested-item-not-supported-on-time,

requested-measurement-not-supported-with-the-requested-measurement-type,

requested-measurement-temporarily-not-available-for-the-requested-measurement-type
}

*** skip unmodified text ***

CommonTAParameters ::= SEQUENCE {

epochTime				OCTET STRING,

taInfo					OCTET STRING,

iE-Extensions			ProtocolExtensionContainer {{ CommonTAParameters-ExtIEs}}				OPTIONAL,

...

}

CommonTAParameters-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

*** skip unmodified text ***

Expected-Value-ZoA ::= INTEGER (0..1799)

EnhancedTimingReportingGranularityFactor(NAME-FFS) ::= INTEGER(1,2,3,4,5...)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

*** skip unmodified text ***

NZP-CSI-RS-ResourceID::= INTEGER  (0..191)

NumberOfSelectedMeasurements(NAME-FFS) ::= ENUMERATED(8,16,24,...)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- T

TAC ::= OCTET STRING (SIZE(3))

*** skip unmodified text ***

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional}|

{ID id-NumberOfSelectedMeasurements(NAME-FFS)	CRITICALITY ignore EXTENSION NumberOfSelectedMeasurements(NAME-FFS) PRESENCE	optional}|

{ID id-EnhancedTimingReportingGranularityFactor(NAME-FFS)	CRITICALITY ignore EXTENSION EnhancedTimingReportingGranularityFactor(NAME-FFS) PRESENCE	optional},

...

}

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** skip unmodified text ***

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

*** skip unmodified text ***

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-NumberOfSelectedMeasurements(NAME-FFS)						ProtocolIE-ID ::= bbb

id-EnhancedTimingReportingGranularityFactor(NAME-FFS)			ProtocolIE-ID ::= ccc

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>