3GPP TSG RAN WG3 Meeting #127-bis	R3-252022

Wuhan, China, 07-11 April, 2025

# Title:	[DRAFT] LS on Handling of UE Radio Capability for Paging

# Response to:

# Release:	-

# Work Item:	NR_newRAT-Core

Source:	Ericsson, CMCC, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia [to be RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	R3-25xxxx

1. Overall Description:

RAN3 has discussed the issue of UE radio capability for paging information.

RAN3 is discussing the attached CRs to clarify the following statement from TS 23.501 clause 5.4.4.3:

- As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN.

Especially, RAN3 would like to clarify that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST, and if the UE Radio Capability for Paging Information does not contain information on all NG-RAN RATs that the UE supports, then the NG-RAN node will inform the AMF of the information on the missing NG-RAN RATs in the UE RADIO CAPABILITY INFO INDICATION message if needed. RAN3 also would like to clarify that the report is not limited to the PLMN currently serving the UE, if the “that PLMN” in the above text is interpreted as the serving PLMN.

2. Actions:

To SA2.

ACTION: 	RAN3 asks SA2 to confirm RAN3 assumptions.

3. Date of Next TSG RAN WG3 Meetings:

TSG-RAN3 Meeting #128		May 19-23, 2025			Malta, MT

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India