3GPP TSG-RAN WG3 Meeting #127-bis	R3-252046

Wuhan, China, 7-11 April 2025

Title: 	Enhancements for Data Collection Reporting

Source: 	Ericsson, Jio Platforms (JPL), InterDigital, FiberCop, BT

Agenda item:	31.2

Document Type:	Discussion and agreement

# 1.	Introduction

In Rel-18, the Data Collection Reporting Initiation and Data Collection Reporting procedures were introduced. In this contribution, we discuss enhancement for the reporting of UE performance.

# 2.	Discussion

### 2.2	UE performance reporting

Let’s assume that gNB1 initiates a Data Collection Reporting Initiation procedure to request gNB2 some UE performance. The gNB1 then starts mobility for a number of UEs and the gNB1 reports the UE performance to gNB1. At gNB2, the collection of UE performance can terminate for three reasons:

1) the Collection Time Duration specified in the configuration for the reporting has passed;

2) the UE moved to RRC_INACTIVE or RRC_IDLE;

3) the UE is handed over to another cell.

The gNB2 does not signal to gNB1 the reason for terminating the UE performance collection.

At the last RAN3 meeting, a CR to XnAP was agreed in R3-250883 for Data Collection reporting, according to which:

The result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message.

The same CR introduced a clarification in the procedure text describing the interaction between the Handover preparation procedure and the Data Collection Reporting and Data Collection Reporting Initiation procedures:

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures: If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover via the Data Collection Reporting procedure and according to clause 8.4.13.2, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node,.

According to the agreed CR, during the Collection Time Duration for UE Performance, gNB2 may send to gNB1 multiple DATA COLLECTION UPDATE messages, and when the Collection Time Duration for UE Performance is terminated, gNB2 shall send to gNB1 the remaining collected data in the next available DATA COLLECTION UPDATE message.

In the example below, we discuss one limitation of the reporting mechanism.

In step 1 and 2 the gNB1 subscribes to receive periodic UE performance from gNB2. In the request, gNB1 indicates the UE performance it is interested to receive, the Reporting Periodicity for Data Collection and the Collection Time Duration for UE Performance. These configuration parameters will be used for all UEs. Later (steps 3, 4, 5), gNB1 hands over to gNB2 UE1, UE2 and UE3, and the collection of UE performance starts at gNB2 at successful handover.

The periodic reporting of UE performance proceeds as normal (for example in steps 6 and 7 in the figure), and gNB1 expects to receive the last sample of UE performance for UE1 in step 8 (next available DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ends for UE1). The gNB1 indeed receives this information in step 8. However, gNB1 does not know that the collection of UE performance for UE1 did not terminate when Collection Time Duration for UE Performance ended, but at the time of step 7.A, when UE1 was handed over to another cell.

A similar situation occurs in step 9. The gNB1 expects to receive the last sample of UE performance for UE2 in step 9 (next available DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ends for UE2). The gNB1 indeed receives this information in step 9. However, gNB1 does not know that the collection of UE performance for UE2 did not terminate when Collection Time Duration for UE Performance ended, but at the time of step 8.A, when UE2 was released to RRC_IDLE.

The gNB1 does not know which exit condition was used at gNB2 to terminate the collection of UE performance and the last samples of UE performance for UE1 and UE2 were collected for a smaller (unknown) amount of time compared to the reporting periodicity. One possible consequence could be that the last sample of the UE performance reported for UE1 and UE2 contain performance metrics values that are very similar to the one before last sample. However, such similarity is not because the UE performance remained unchanged, but because the UE performance could not be measured, therefore the same performance as the one before last sample is reported.

Observation 1: In case of periodic reporting, the gNB1 does not know for how long gNB2 collected the UE performance sent in the next available DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ended.

Not knowing the time spent for collecting UE performance becomes an even more severe problem in case of single reporting. The figure below describes the problem with a second example. In step 1 and 2 the gNB1 subscribes to receive periodic UE performance from gNB2. In the request, it indicates the UE performance it is interested to receive, the Collection Time Duration for UE Performance and it does not indicate a reporting periodicity. Later (steps 3, 4, 5), gNB1 hands over to gNB2 UE1, UE2 and UE3, and the collection of UE performance starts at gNB2.

In step 6, gNB1 receives only one DATA COLLECTION UPDATE message (next available DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ended for the last handed over UE, UE3). However, gNB1 does not know whether the collection of UE performance for each UE was conducted during the whole Collection Time Duration for UE Performance, or if the collection time was shorter. In the proposed example, the time spent by gNB2 for collecting the UE performance for the three UEs is very different: for UE1 the collection ended almost immediately (due to handover to another cell), for UE2 the collection ended in the middle of the Collection Time Duration for UE Performance, and for UE3 the collection proceeded until the Collection Time Duration for UE Performance terminated.

Observation 2: In case of single reporting, the gNB1 does not know the time during which gNB2 collected the UE performance, received from gNB2 in the DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ended.

The amount of time during which the UE performance was collected is an important piece of information that makes the UE Performance feedback more or less relevant/accurate for the requesting node. However, the requesting node (gNB1) is totally unaware of the time duration for which the UE Performance was collected and it cannot determine if the reported UE Performance metrics are valid and accurate, or if they should be filtered out and not considered. For example, if we assume that gNB1 is interested in collecting measured UE performance during a 5-second interval, it is quite evident that the usefulness of the measured UE performance is not the same if the exit condition (UE transitions to RRC_INACTIVE or to RRC_IDLE, or UE is handed over to another cell) manifests itself after 200ms or after 4800ms.

Observation 3: The amount of time during which the UE performance is collected is a useful indication of the goodness of the collected UE performance.

For example, the node receiving the measured UE performance could decide to filter out any information for which the time the UE stayed in the target cell was below a certain threshold; or, the receiving node could treat differently the UE feedback depending on the time duration for which the feedback was collected. The present framework, however, does not allow this and forces the receiving node to consider all collected data as equal. This leads to a sub-optimal use of the data, where “noisy” measured UE performance (e.g., data collected during a very short time) can degrade the overall performance of AI/ML processes using this data.

Observation 4: Without knowledge of the amount of time during which the UE performance was collected, the receiving node is forced to treat all the collected data as equal, which can degrade the overall performance of AI/ML processes.

Therefore, we propose to enhance the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message with an IE (e.g., Feedback Collection Duration) that indicates the amount of time during which the UE performance was collected. This will help the receiving node to take better decisions and will lead to a more intelligent use of the received data.

Proposal 1: Enhance the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message with an IE (e.g., Feedback Collection Duration) that indicates the amount of time during which the UE performance was collected.

Proposal 2: Agree to the CR to TS 38.423 in Appendix, which reflects the above proposal.

# 3.	Conclusions and Proposals

In the previous sections we made the following observations and proposals:

Observation 1: In case of periodic reporting, the gNB1 does not know for how long gNB2 collected the UE performance sent in the next available DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ended.

Observation 2: In case of single reporting, the gNB1 does not know the time during which gNB2 collected the UE performance, received from gNB2 in the DATA COLLECTION UPDATE message after the Collection Time Duration for UE Performance ended.

Observation 3: The amount of time during which the UE performance is collected is a useful indication of the goodness of the collected UE performance.

Observation 4: Without knowledge of the amount of time during which the UE performance was collected, the receiving node is forced to treat all the collected data as equal, which can degrade the overall performance of AI/ML processes.

Proposal 1: Enhance the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message with an IE (e.g., Feedback Collection Duration) that indicates the amount of time during which the UE performance was collected.

Proposal 2: Agree to the CR to TS 38.423 in Appendix, which reflects the above proposal.

# Appendix - CR to XnAP – Data Collection Reporting

3GPP TSG-RAN WG3 Meeting #127-bis	R3-25xxxx

Wuhan, China, 7-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement for Data Collection Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms (JPL), InterDigital, FiberCop, BT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core,TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following limitation exists in the Rel-18 framework for reporting of information to support, e.g., AI/ML in NG-RAN: - An NG-RAN node1 receiving measured UE performance from NG-RAN node2 is not aware of the conditions under which the UE performance was collected. If NG-RAN node1 knows the amount of time during which the UE performance was collected, it can filter out “noisy” data and improve the overall performance of AI/ML processes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new Feedback Collection Duration IE in the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message to indicate the amount of time during which the UE performance was collected. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the Data Collection Reporting procedure. This CR is BC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the NG-RAN node1 receives measured UE performance from NG-RAN node2, the NG-RAN node1 does not understand the relevance of the collected UE performance and it is unclear how to use this information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.29, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------ CHANGES STARTS HERE ------------------------------------------

### 8.4.13	Data Collection Reporting Initiation

### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message.

If the Feedback Collection Duration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall, if supported, send to NG-RAN node1 in the DATA COLLECTION UPDATE message the amount of time during which the UE performance was collected.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

------------------------------------------ NEXT CHANGE ------------------------------------------

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>Feedback Collection Duration | O |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements were collected. Unit: millisecond | YES | ignore |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

(Skip Unchanged)

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-FeedbackCollectionDuration,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

------------------------------------------ NEXT CHANGE ------------------------------------------

-- F

(Skip Unchanged)

FeedbackCollectionDuration ::= INTEGER (1..5000, ...)

------------------------------------------ NEXT CHANGE ------------------------------------------

-- U

(Skip Unchanged)

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-FeedbackCollectionDuration					CRITICALITY ignore	EXTENSION FeedbackCollectionDuration						PRESENCE optional },

...

}

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

(Skip Unchanged)

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

(Skip Unchanged)

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-FeedbackCollectionDuration																		ProtocolIE-ID ::= xx1

END

-- ASN1STOP