3GPP TSG-RAN WG3 #127-bis								R3-252060

# Wuhan, China, 7-11 April 2025

Agenda Item:	9.2

Source:	Ericsson, CATT, ZTE, Lenovo, LG Electronics, Google, Verizon Wireless

Title:	F1 support for L3 measurements based LTM

Document for:	Discussion, Approval

# 1	Introduction

In Rel-18, RAN WGs, including RAN2, RAN3, and RAN4, have discussed and agreed on decoupling UE capabilities for L1 measurement and LTM. Also, RAN2 confirmed that early synchronization can also be triggered by L3 measurements, with the CU making the decision based on these measurements and informing the DU.

However, after an extensive discussion in RAN3 about F1 support for L3 measurement-based LTM during the RAN3#127 meeting, no consensus was reached. Subsequently, the RAN plenary reviewed the topic and sent an LS in [1], expressing the view that a single solution should be adopted in Rel-18.

RAN kindly requests RAN3 to agree by RAN#108, a single standard solution as Rel-18 correction, to support the L3 measurement based LTM over the F1 interface (see WI Code above).

Considering that two RAN3 meetings remain before RAN#108, we would like to further analyze the issue and propose an appropriate solution in this contribution.

# 2	Discussion

## 2.1	Background

The RAN2#126 meeting discussed the decoupling of L1 measurement capability from LTM capabilities. Furthermore, RAN2 sent an LS to RAN4, as mentioned in [2], requesting additional input.

RAN2 discussed RAN1 and RAN4 feature lists associated with Rel-18 NR_Mob_enh2. RAN2 discussed whether the following L1 measurement and reporting features are mandatory to support LTM or whether L3 measurement could be used.

| 45-1 | Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure |
| --- | --- |
| 45-1a | Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure |

RAN2 made the following agreement:

RAN2 makes no further assumptions whether L3 measurements can be used or not to trigger LTM.

RAN2 would like to check the following with RAN1 and RAN4:

Question 1 : Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively?

RAN4’s response is provided in [3].

For whether intra-frequency and inter-frequency L1 measurements and reporting are prerequisites to support intra-frequency and inter-frequency LTM, RAN4 analysed RAN4 requirements and achieved following agreements

- For FR1, RAN4 Rel-18 related LTM requirements are applicable to the case without L1 measurement and reporting of LTM candidate cells.

- For FR2, RAN4 Rel-18 related LTM requirements are not applicable if L1 measurements for LTM candidate cells are not supported by the UE, or not configured by the network.

Then, RAN2 and RAN3, along with the RAN plenary, initiated discussions on how to support this in the Rel-18 corrections. However, no specific solution was selected after these discussions.

## 2.2	Basic principle

The scenario involves the CU informing the DU to trigger a MAC CE command to the UE when the CU decides to execute LTM or trigger early sychronization based on the L3 measurements received.

As well known, the CU processes L3 measurements, including beam-level information, to determine the activation of a TCI state. However, L3 measurements are filtered values and cannot be processed in the DU in the same way as L1 measurements The CU should be responsible for making handover and beam selection decisions based on the received L3 measurements.

Throughout the signaling phase, the UE remains unaffected by interruptions as this process occurs entirely within the network. The CU identifies the necessity for triggering a procedure and subsequently instructs the DU. The interruption is only initiated when the UE receives the TCI state activation MAC CE. The DU ensures timely transmission of the MAC CE just before the SSB to optimize synchronization with the LTM candidate cell.

This discussion is independent of RAN4 requirements. Any interruption occurs strictly when the UE receives the LTM cell switch MAC CE or the PDCCH order. The primary consideration is whether the network can send the LTM cell switch MAC CE within 160ms following the PDCCH order or TCI state activation MAC CE. Failure to meet this requirement is unrelated to whether the CU provides the SSB index and Cell ID or L3 measurement to the DU.

The CU should remain responsible for handling L3 information, ensuring compliance with functional split principles in the gNB architecture.

## 2.3	Proposed solutions

Several options have been proposed and discussed as follows.

Option 1: CU-based solution: Based on L3 measurements received from UE, CU makes LTM decision and informs DU to trigger LTM procedures.

Option 2: DU based solution: CU forwards L3 measurements received from UE to DU via F1 signalling and DU makes all LTM decisions similar to L1 based measurements.

Option 3: Hybrid approach. Option 1 plus an optional IE to carry L3 measurements.

Option 4: Do nothing in R18

In the last RAN3 meeting, Option 4 was the final conclusion. However, there remains a strong demand from operators to specify an F1 solution for supporting L3 measurement-based LTM. Considering this, we propose an alternative way forward, referred to as Option 5.

Option 5: CU-based solution with assistance information to be sent to DU.

Another alternative to Option 5 is to allow the CU sends the selected measurement quantities to the DU for the purpose of early UL/DL synchronization with a defined conditional IE.

Option 5b: CU-based solution with assistance information provided to the DU for early synchronization.

When comparing the above 6 options, the first point to clarify is that Option 2 is not feasible. Simply providing the L3 measurement from CU to DU does not allow the DU to fully understand why these measurements were sent by the UE or under which configuration. This is because the interpretation of the L3 measurement result is usually done by the CU in conjunction with the L3 measurement objects configured, plus some other factor such as deployment and current cell load. As a result, if merely the L3 mesurement results are sent to the DU (since it is not acceptable now to send now all other information such as L3 measurement object and other network information), the DU may make suboptimal or incorrect mobility decisions, potentially leading to an increased number of handovers or radio link failures.

Option 2 should be ruled out because the DU cannot interpret L3 measurements in the same way as L1 measurements.

Based on the guidance from RAN plenary, a single standard solution is the way forward.

Option 3 is no longer considered since it is not a single solution.

As is well known, in a split network architecture, the CU manages non-real-time functions, including higher-layer L2 and Layer 3 processing. Therefore, the CU needs to make decisions based on L3 measurements to determine whether LTM execution should be triggered and to which candidate cells. Both Option 1 and Option 5 adhere to this principle.

In Option 1, a single message can be used to indicate the triggering of L3 measurement-based LTM, including information on one candidate cell and the SSB index. While in Option 5, as a compromise, the CU can analyse the received L3 measurements for the candidate cells as usual, process the measurement results, and then initiate early synchronization and/or cell switch procedures. Instead of sending the L3 measurement report as received from the UE in a single container, Option 5 proposes processing the L3 measurements and sending the quantities to the DU in a detailed list format. This would allow the DU to understand the values in a proper way. For example, it could send the assistance information such as: cell ID1 → RSRP1, RSRQ1, SINR1. In the alternative Option 5b, an additional condition is imposed on the CU when sending measurement quantities to the DU, specifically, it is used for early UL/DL synchronization and not for cell switching. In this case, the CU remains the decision-maker.

Therefore, we believe that all Option 1, Option 5 and Option 5b works well and adhere to the functional principles. Option 1 is simple, straightforward, and aligns with the system's functionalities. Option 5, on the other hand, is considered as a way forward if some vendors prefer the DU to receive more assistance information when making decisions for early synchronization and/or cell switching based on the L3 measurements. Option 5b can be viewed as a subset of Option 5, with a specific focus on differentiating the handling of cell switching and early synchronization.

RAN3 to choose one option from Option 1, Option 5, and Option 5b.

Below is an example illustrating Option 1. A point code in BITSTRING format is required to specify the purpose of triggering this procedure, such as for a cell switch, early UL/DL synchronization, or both based on the same L3 measurements.

#### 9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to inform the gNB-DU to trigger a cell switch command and/or early synchronization to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Mobility Triggering Information List |  | 1 |  |  | YES | reject |
| >Mobility Triggering Information Item |  | 1 ..<maxnoofLTMCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |

And here is another example illustrating Option 5. A list of measurement quantities selected by the CU are transferred to the DU as assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Mobility Assistance Information List |  | 1 |  |  | YES | reject |
| >Mobility Assistance Information Item |  | 1 ..<maxnoofLTMCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |
| >>Selected Measurement quantities | O |  |  |  | - |  |
| >>>RSRP | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. | - |  |
| >>>RSRQ | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. | - |  |
| >>>SINR | O |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. | - |  |

In Option 5b, a conditional IE would be defined with the explanation that the measurement quantities are sent to the DU for early synchronization a follows.

| Condition | Explanation |
| --- | --- |
| ifEarlySync | This IE shall be present only if the Triggering Indication IE is configured with early UL synchronization and/or early DL synchronization. |

Three sets of F1AP CRs have been presented in [6], [7] and [8], corresponding to Options 1, 5 and 5b. For the sake of progress and to save time in Rel-18 corrections, and to offer certain vendor flexibility in obtaining information from the CU to the DU, we will adopt the compromised solution, i.e., Option 5.

Introduce a new Class 2 procedure over F1 to allow the CU to instruct the DU to trigger LTM and/or early synchronization based on the L3 measurements received from the UE.

The new message is named as CU-DU Mobility Initiation Request, and optionally includes assistance information.

Agree on the F1AP CR to TS 38.473 in [7], the CR to TS 38.401 in [9], and the CR to TS 38.470 in [10].

# 3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1	The CU should remain responsible for handling L3 information, ensuring compliance with functional split principles in the gNB architecture.

Proposal 1	Option 2 should be ruled out because the DU cannot interpret L3 measurements in the same way as L1 measurements.

Proposal 2	Option 3 is no longer considered since it is not a single solution.

Proposal 3	RAN3 to choose one option from Option 1, Option 5, and Option 5b.

Proposal 4	Introduce a new Class 2 procedure over F1 to allow the CU to instruct the DU to trigger LTM and/or early synchronization based on the L3 measurements received from the UE.

Proposal 5	The new message is named as CU-DU Mobility Initiation Request, and optionally includes assistance information.

Proposal 6	Agree on the F1AP CR to TS 38.473 in [7], the CR to TS 38.401 in [9], and the CR to TS 38.470 in [10].

# 4	References

R3-251532, Reply LS on L3 measurement based LTM support over F1, RAN Plenary

R2-2404014, LS on LTM L1 intra and inter-frequency measurements, RAN2

R4-2410303, Reply LS on LTM L1 intra and inter-frequency measurements, RAN4

R3-245710, Reply LS on L3 measurement based LTM support over F1, RAN3

R3-247013, L3 measurement based LTM support over F1, RAN2

R3-252061, Option 1 for F1 support of L3 measurements-based LTM, Ericsson, Vodafone, CATT, ZTE, NTT DoCoMo, Verizon Wireless, Nokia, Samsung, LG Electronics, Lenovo, Google

R3-252062, Option 5 for F1 support of L3 measurements-based LTM, Ericsson, Vodafone, CATT, ZTE, NTT DoCoMo, Verizon Wireless, Nokia, Samsung, LG Electronics, Lenovo, Google

R3-252166, Option 5b for F1 support for L3 measurements-based LTM, Samsung, Nokia, Ericsson, ZTE, LG Electronics, Verizon Wireless

R3-251859, Stage 2 updates for support of L3 measurements based LTM(option1&5), CATT, Ericsson, Vodafone, ZTE, Google, Samsung, LG Electronics, NTT DoCoMo, Nokia

R3-251874, F1 support for L3 measurements-based LTM, ZTE Corporation, Ericsson, CATT, Vodafone, Nokia, LG Electronics, Samsung