3GPP TSG-RAN WG3 Meeting #127bis	R3-252069

Wuhan, China, 07 – 11 April 2025

Agenda item:	9.2

Source:	Nokia

Title:	Update of Broadcast MCCH Information for MBS packets discard and multiple Cell-ID case

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#126 the unresolved issue of PDCP discard of broadcast packets related to MBS broadcast resource sharing release 18 feature was discussed again without concluding.

The issue arises for both “MOCN” case and “Multiple cell ID” case when there is a switch of MBS shared tunnel.

At RAN3#125, the CR in [1] suggested some solutions which were not agreed.

The proposed solutions were:

- For MOCN case: when the gNB-CU switches the NG-U tunnel from PLMN1 to PLMN2, the gNB-CU ensures PDCP SN continuity i.e. it sets the PDCP SN of the first packet received over the new NG-U 2 and to be delivered over the F1-U tunnel with the PDCP SN of the next packet to be delivered to the UE (PDCP SN continuity), so that the UE will not discard the packets but instead re-initialize its PDCP SN with the received value.

NOTE 1: It is expected that the gNB CU-UP supports PDCP SN continuity after establishment of shared NG-U resources with 5GC2.

- For multiple-cell ID case, it was proposed that the DU may use different MRB configuration in order to trigger a release/add MRB action for the UEs which would re-establish the PDCP.

NOTE 2: The gNB-DU1/2 may use different MRB configuration after the shared NG-U and F1-U path change, to enable the UEs to release and add related MRB to avoid unexpected packets discard.

It was commented that the NOTE 2 does not work because it relies on UE implementation choice and therefore the CR in [1] was noted altogether without discussion on NOTE 1.

This above statement concerning “relies on UE implementation” was challenged by some companies and therefore an official LS was sent to RAN2 for confirmation in R3-247892 [2].

- RAN3 is discussing a recovery scenario related to MBS broadcast. In this scenario MCCH Information of the broadcast session is broadcast in the cell. Some companies propose a solution where the gNB modifies one or more parameters in the broadcast MCCH information in order to force all UEs in the cell involved in this broadcast session to release/add MRB and re-establish PDCP with PDCP SN of the first delivered packet.

- This solution however assumes that all UEs will behave as above. Therefore, RAN3 would like a confirmation from RAN2 if the solution can work.

- RAN3 kindly asks RAN2 to confirm whether it is possible for the network to force the re-establishment of PDCP of all UEs in a cell by an update of parameter broadcast in MCCH Information.

# 2	Discussion

At this meeting RAN3 received the reply from RAN2 in R2-2501341 [3]:

RAN2 has discussed the question raised by RAN3 and would like to answer as follows:

- According to RAN2 specifications it is up to UE implementation how to perform a modification of a broadcast MRB which is already configured.

- There are some parameters in PDCP configuration which require PDCP re-establishment (e.g. RoHC, PDCP SN length), but RAN2 specifications do not capture exact UE behaviour upon change of these parameters for MRB.

For the “Multiple cell ID” case, this LS opens the door for two possible solutions:

- Solution 1: Ensure PDCP SN continuity by transferring the PDCP SN

NOTE 1 can apply for “MOCN” case.

A similar solution could be applied to “Multiple cell ID” case i.e. the gNB-CU of PLMN2 is made aware of the SN of next packet to be delivered by the gNB-DU, then the gNB-CU of PLMN2 initializes or re-initializes the PDCP SN to this value over the new F1-U tunnel of PLMN2.

A set of CRs had been provided at last meeting if RAN3 would decide to go for this solution.

- Solution 2: Broadcast a different PDCP configuration in the MCCH after the tunnel switch

RAN2 basically replies that only some higher layer parameters can reliably lead to PDCP re-establishment of the UEs and these are related to PDCP configuration i.e. RoHC, PDCP SN length.

This means that the issue can be avoided if the gNB-DU broadcast in MCCH Information after the switch a PDCP configuration of CU-UP2 that is different than the PDCP configuration of CU UP1 that the gNB-DU broadcasted before the switch.

This solution could be enabled if the CU UP2 gets informed of the PDCP configuration of CU UP1 in the Broadcast Transport Resource Request message.

A set of CR(s) have been provided at this meeting if RAN3 decides to go for this solution in [4], [5].

Given that solution 1 was not favored at last RAN3 meeting, we propose to agree solution 2.

Proposal 1: RAN3 to discuss and agree solution 2.

Proposal 2: RAN3 to agree the response LS in [6].

# 3	Conclusion and Proposals

This paper has continued the discussion on PDCP discard issue initiated by tdoc [1] based on the reply LS from RAN2 received at this meeting in [3] and which indicates that a change of PDCP configuration broadcast in MCCH information can lead to all UEs doing PDCP re-establishment and thereby avoid the issue.

Two solutions have been proposed in relation with this new information.

Proposal 1: RAN3 to discuss and agree solution 2.

Proposal 2: RAN3 to agree the response LS in [6].

# 4	References

- R3-244097, Correction on PDCP packets discard in case of MBS broadcast path update

- R3-247892, LS on Update of Broadcast MCCH Information

- R3-251512, Reply LS on Update of Broadcast MCCH Information (R2-2501341)

- R3-252071, Update of Broadcast MCCH Information for MBS packets discard and multiple Cell-ID case, TS 38.401 (solution 2)

- R3-252072, Update of Broadcast MCCH Information for MBS packets discard and multiple Cell-ID case, TS 38.473 (solution 2)

- R3-252070, Response LS on Update of Broadcast MCCH Information