3GPP TSG-RAN WG3 #127bis	R3-252073

Wuhan, China, 7-11 April 2025

Source:	Ericsson

Title:	Tracking Area Handling for Regenerative Satellite Access

Document for:	Information

Agenda Item:	14.3

NOTE: A revision of this contribution has been submitted to SA2 #168. Interested companies are welcome to engage in the relevant discussion in SA2.

Abstract of the contribution: This contribution analyses the RAN3 agreement to configure AMF with a timetable per TAI and gNB.

# Background

When gNB is onboard a LEO satellite that travels across an area, it will need to change the TAC(s) it broadcast in order to ensure earth-fixed TAs. The gNB will thus change the set of TAC(s) it supports over time. Since the LEO satellite travels with considerable speed (7-8 km/s) the TAC reconfiguration may happen frequently.

To avoid that the gNB has to send frequent RAN CONFIGURATION UPDATE messages over the feeder link, RAN3 has agreed on a solution where the AMF can be configured via OAM with a “timetable” of TAIs (per gNB). This is possible since the movement of LEO satellites is predictable based on the ephemeris. Such solution allows a gNB to send the full list of TAIs in the NG SETUP (i.e. include even those TAIs that will be broadcast later) and avoid later RAN CONFIGURATION UPDATE procedures. When there is a need to page a UE, the AMF uses the configured “timetable” to determine what gNB(s) serves a specific TA at that specific moment in time.

The RAN3 solution has been described in the LS in R3-250895.

# Discussion

This paper analyses how much signaling such RAN3 proposal can save.  In the analysis below, we will assume that the gNB sends RAN CONFIGURATION UPDATE (RCU) messages to inform AMF about the change in TA configuration, and then compare to a situation without the RCU messages.

NOTE: The intention is not to do an exact calculation but to estimate an order of magnitude!

##### General scenario

As a LEO satellite moves and the gNB-onboard leaves an area (e.g. a TA) the CONNECTED mode UEs in that area have to be handed over to another satellite (with gNB-onboard). This is illustrated in Figure 1, describing this sequence of events:

- 	Time T1: gNB1 serves TA1 and gNB2 serves TA2

- 	Time T2: gNB3 starts to serve TA2 and overlaps coverage with gNB2. HO of UEs from gNB2 to gNB3.

- 	Time T3: gNB2 starts to serve TA1 and overlaps coverage with gNB1. HO of UEs from gNB1 to gNB2.

- 	Time T4: gNB2 serves TA1 and gNB3 serves TA2

Figure 1. Satellites serving a number of UEs relocating from one area to another

At time T2, gNB3 will send a RCU to inform AMF that it serves TAI2 (and maybe remove some other TAI). There will also be Handover signaling (e.g. PATH SWITCH) to handover the UEs.

At time T3, gNB2 will send a RCU to inform AMF that it serves TAI1 (but not TAI2). There will also be Handover signaling (e.g. PATH SWITCH) to handover the UEs.

##### A deployment model

Assume an example (large) country covered by LEO satellites that travel east-west. Each satellite projects multiple beams on the ground where each beam represents a cell.

The country is also divided into AMF areas, where a number of AMFs serve that area. (Figure 2).

To estimate the number of RCUs, it is sufficient to consider it as a one-dimensional problem (east-west), i.e. there is no need to consider the north-south direction as it will not impact the result.

Figure 2. Hypothetical (large) country with four AMF areas

Assume the following deployment parameters:

- The country is covered by satellite cells, where each cell extends x km in east-west direction (e.g. x = 40km)

- The cells are grouped into TAs where each TA has N cells in east-west direction (e.g. N=2). An example with N=2 is shown in Figure 3.

- The distance (east-west) for one AMF area is y km (in the example in Figure 2, y = 2200km)

The total number of TAs per AMF area in the east-west direction is thus: Ta = y/x/N. With the example above (y=2200km, x=40km, N=2) there are Ta=28 TAs per AMF area (east-west direction only).

Figure 3. Example of cells and TA

##### Estimation of the number of NG procedures

The gNB-onboard sends RCU messages to inform AMF about the change in TA configuration, and it needs to send one RCU for each NG interface it has. However, a gNB may not send a RCU for every TAI it changes. For example, if the gNB relocates multiple cells at a time, constituting multiple TAs, only one RCU is needed for those TAs. We make the following assumptions:

- When the gNB-onboard starts serving a new area it “moves” c TAs (east-west) at a time, i.e. it sends a RCU every c:th TA. For example, in Figure 1, c=1. However, other deployments may move two or more TAs in east-west direction at a time, i.e. c=2, 3 etc.

- Each gNB on-board has an established NG interface to a AMFs (e.g. a=2)

Based on the above, the number of RCU procedures needed when a gNB passes one AMF area is thus: a*Ta /c

Now also assume the following about UEs on the ground:

- In each TA on ground there are M UEs in CONNECTED state (e.g. M=50). For example, if the TA is 80km wide (x=40km, N=2) and some similar distance high, then there are 50 CONNECTED mode UEs in that area.

- During the time a gNB travels across an AMF area (east-west) there are W other NGAP procedures, e.g. transitions between IDLE and CONNECTED, REGISTRATION, PDU SESSION ESTABLISHMENT etc (e.g. W=40). In the example in Figure 2, a LEO satellite travels the 2200km distance in roughly 6 minutes, which means that there would be W other procedures in those 6 minutes per gNB.

The total number of NG procedures for one gNB, from the time the gNB enters the AMF area until the gNB leaves the AMF area are then roughly:

- 	NG SETUP: a procedures (i.e. one per AMF)

- 	RAN CONFIGURATION UPDATE to inform about TA reconfiguration: a*Ta /c

- 	Handover (PATH SWITCH) of CONNECTED mode UEs when a gNB leaves each TA: M*Ta

- 	OTHER procedures, e.g. CONNECTED-IDLE transitions: W

- 	NG REMOVAL: a procedures (i.e. one per AMF)

In total we have:

- Total # of procedures without RCU: Po = 2*a + M*Ta + W

- Total # of procedures including RCU: Pi = Po + a* Ta /c

The relative reduction in NGAP procedures is: (Pi - Po) / Pi

##### Results

Some example results are shown in Table 1, where we have assumed y=2200km, x=40km, N=2, W=40 for all cases as changes to these parameters do not affect the result very much. The parameters that impact the ratio the most are: a (# of AMF/gNB), c (# of TA/update) and M (# of CONNECTED UEs). The below calculations however take all procedures into account.

Table 1: NG procedures per gNB and AMF area

| Parameters (assumptions) | # of procedures per gNB and per AMF area passage | Relative reduction in NGAP procedures |
| --- | --- | --- |
| y=2200km x=40km (cell width) N=2 (# of cells/TA) c=2 (# of TA/update) a=2 (# of AMF/gNB) M=50 (# CONNECTED UEs) W=40 (# Other NGAP procedures) | NG SETUP + REMOVAL: 4 RAN CONFIG UPDATE: 28 PATH SWITCH: 1400 Other: 40 Total with RCU: 1452 Total without RCU: 1424 | 1.9% |
| y=2200km x=40km (cell width) N=2 (# of cells/TA) c=1 (# of TA/update) a=6 (# of AMF/gNB) M=50 (# CONNECTED UEs) W=40 (# Other NGAP procedures) | NG SETUP + REMOVAL: 12 RAN CONFIG UPDATE: 168 PATH SWITCH: 1400 Other: 40 Total with RCU: 1620 Total without RCU: 1424 | 10% |
| y=2200km x=40km (cell width) N=4 (# of cells/TA) c=2 (# of TA/update) a=2 (# of AMF/gNB) M=400 (#CONNECTED UEs) W=40 (# Other NGAP procedures) | NG SETUP + REMOVAL: 4 RAN CONFIG UPDATE: 28 PATH SWITCH: 11200 Other: 40 Total with RCU: 11272 Total without RCU: 11244 | 0.25% |
| y=2200km x=40km (cell width) N=2 (# of cells/TA) c=2 (# of TA/update) a=2 (# of AMF/gNB) M=20 (# CONNECTED UEs) W=40 (# Other NGAP procedures) | NG SETUP + REMOVAL: 4 RAN CONFIG UPDATE: 28 PATH SWITCH: 560 Other: 40 Total with RCU: 632 Total without RCU: 604 | 4.4% |
| y=2200km x=40km (cell width) N=2 (# of cells/TA) c=1 (# of TA/update) a=2 (# of AMF/gNB) M=20 (# CONNECTED UEs) W=40 (# Other NGAP procedures) | NG SETUP + REMOVAL: 4 RAN CONFIG UPDATE: 28 PATH SWITCH: 560 Other: 40 Total with RCU: 660 Total without RCU: 604 | 8.5% |

Observation: Up to 10% of the NGAP signalling over the feeder link can be saved by avoiding the RCU, depending on deployment aspects and the number of UEs connected in the different satellite cells.

# Conclusion and Proposal

The signalling saving to avoid RCU has been estimated in this paper, showing some relative signalling load reduction over the feeder link (up to ~10%) depending on deployment parameters and number of connected UEs.

It is proposed that SA2 discusses whether a timetable configured in AMF by OAM is useful to achieve gains for the feeder link load.

The authors of this papers see some potential benefit with an optional solution with timetable configured in AMF by OAM, as optimization (according to the RAN3 LS), but welcome further discussion and feedback, especially from interested operators.